ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

For

REVITALISATION OF MASHOLOMOSHE AND MASIYEPAMBILI SMALLHOLDER IRRIGATION SCHEMES

IN MATABELELAND SOUTH, ZIMBABWE

Prepared for:

United Nations Development Programme

On behalf of the Government of Zimbabwe

Proponent Details

Eng. Shepard Kadaira

Ministry of Lands, Agriculture, Fisheries, Water and Rural Development

10th Floor, Kaguvi Building

Cnr 4th Street and Central Avenue

+263 714 900 693/+263 773 375 353

skadaira@gmail.com

Consultant Details

Eng. Michael Kubare

Sesani (Pvt) Ltd In association with Stewart Scott Zimbabwe (Pvt) Ltd

Contacts:

Tel: +263 (0) 242 30 70 58-61

Mob: +263 775 458 635

E-mail:	michael.kubare@stewartscott.co.zw marshall.maphosa@sesani.com

Harare, Zimbabwe	September 6th, 2022

To:	Attention: UNDP GCF Project Manager

Dear Sir/Madam,

Re: Consultancy to Conduct Detailed Feasibility and Climate Proof Designs and Construction Supervision of GCF Funded Irrigation Schemes: Lot 1

Submission of the clusture   Environmental and Social Management Plan for Masholomoshe and Masiyepambili Irrigation Schemes

Sesani (Pvt) Ltd, in association with Stewart Scott Zimbabwe (Pvt) Ltd, is pleased to submit the cluster Environmental and Social Management Plan for the above-mentioned assignment.

Should you have any questions, please feel free to contact me.

Yours sincerely,

Sesani (Pvt) Ltd

…………………………………………………..

Michael Kubare, Team Leader

BEng MSc MZwIE Pr Eng.

Public Consultation/Disclosure Notice

Date: 10 January 2023

The United Nations Development Programme (UNDP) is requesting feedback on the attached draft Environmental and Social Management Plan for this project. Comments and questions can be sent to the following address:

United Nations Development Programme Physical Address: Block 9, Arundel Office Park, Norfolk Road, Mt Pleasant , Harare , Zimbabwe Tel: +263-242-338836-44 ext. 279 Email: jeremiahmushosho@undp.org Website: www.zw.undp.org

The last date for receiving of comments is 10 February 2023

# EXECUTIVE SUMMARY

ES1: Project Introduction and Background

The Government of Zimbabwe through the Ministry of Lands, Agriculture, Fisheries, Water, and Rural Development (MoLAFWRD) in partnership with United Nations Development Programme (UNDP) is implementing a seven-year project “Building the climate resilience for vulnerable agricultural livelihoods in Southern Zimbabwe”. The project aims to strengthen the adaptive capacities of vulnerable smallholder farmers, especially women, to climate change induced impacts on their agro-ecosystems and livelihoods through revitalization of irrigation schemes, upgrading water and soil moisture management and water use efficiency, climate-resilient agriculture, improved access to climate information and markets, and partnerships with public and private sector actors. The project is targeting the revitalization of 15 existing irrigation schemes and the establishment of 6 new schemes in 15 priority districts of Manicaland, Masvingo and Matabeleland South Provinces.

The irrigation schemes have been classified into three lots, namely lot 1, 2 and 3. Each lot is defined by a specific hydrological catchment with similar biophysical and socio-economic characteristics. Lot 1 consists of three irrigation schemes, all situated in Matabeleland South Province, Umzingwane catchment . Lot 2 consists of 7 irrigation schemes, all situated in Masvingo Province, Runde catchment . Lot 3 consists of 5 irrigation schemes and all are situated in Manicaland Province,Save catchment  . The other 5 alternative schemes (presumably lot 4) are replacement schemes still yet to receive GCF approval. These are alternative schemes that no longer conformed to the GCF selection criteria between the project design phase in 2016 and the project implementation phase in 2020. The single largest 500ha scheme bringing the total to 21 will have a standalone ESMP because of its size and complexity.  While site specific Environmental and Social Management Plans (ESMPs) have been prepared for each of the irrigation schemes under Lot 1 and Lot 2, studies show that the ESMPs for the schemes that fall in within the same catchment or lot largely share the same baseline information, environmental and social impacts, management measures and grievance redress mechanisms. Based on this, it was therefore logical to cluster the ESMPs by lot to ease the review process.

This ESMP is therefore a consolidation of the site-specific ESMPs developed for Masiyepambili and Masholomoshe irrigation schemes which are two of the three irrigation schemes under Lot 1.  The third Midhlo irrigation scheme had already been developed and submitted separately to GCF prior to clustering of these remaining  two. The two irrigation schemes are part of the 21 irrigation schemes targeted for construction, revitalization and climate proofing under the project. The two irrigation schemes are located in Matabeleland South Province of Zimbabwe.

ES2: Project Description

The irrigation schemes will be climate proofed through the installation water efficient irrigation system, adoption of climate-smart crop value chains, improved irrigation scheduling, crop rotation, improved agri-business practices, soil conservation practices, use of solar energy, capacity building of the Irrigation Management Committees, improved access to hydro-meteorological information, robust designs for the civil, structural, and irrigation infrastructures, constituting a Water Point Committee for seasonal water allocation, and improved catchment management. Specifically, Masholomoshe and Masiyepambili irrigation schemes will be climate proofed as follows;

Masholomoshe irrigation scheme: A center pivot irrigation system will be installed at the irrigation scheme to irrigate a total area of 16ha. A 2km 300 PVC pipeline will be installed to supply water to the Pivot and to the laterals. A pump station will be installed to pump water from Nkashe dam to night storage dam. Three (3) axial flow pumps of 22.5 Kw each will be installed. A solar system of 200 Kw capacity will be installed near irrigation scheme office to cater for energy requirements for the irrigation scheme. Mobile construction camps will be erected at the scheme to provide accommodation for skilled contractors.

Masiyepambili irrigation scheme: A solid set sprinkler or drip irrigation system will be installed to irrigate a total area of 10ha. A sand abstraction system will be installed in the river bed adjacent to the scheme for abstraction of water to supply the scheme. Submersible pumps will pump water from the river bed into a sump for onward pumping to the irrigation scheme. Solar panels will be used to power the pumps. Mobile construction camps will be erected at the scheme to provide accommodation for skilled contractors.

ES3: Legal and Technical Basis for the ESMP

The project was screened for environmental and social risks against national requirements as well as the GCF’s and UNDP’s Social and Environmental Standards. In terms of environmental and social risks, the project is categorised as a “moderate risk” Project and the preparation of the ESMP was mandatory. To comply with national requirements as well as GCF and United Nations Development Programme Social and Environmental Requirements, the Ministry of Lands, Agriculture, Fisheries, Water and Rural Development (MoLAFWRD) engaged Sesani Private Limited to assess the environmental and social impacts of the proposed Project and prepare an Environment and Social Management Plan (ESMP). The ESMP was produced as per the requirements of Environmental Management (Environmental Impact Assessment and Ecosystem Protection) Regulations, SI 7 of 2007 and in fulfilment of the requirements of the Environmental Management Act (CAP 20:27) – First Schedule (Section 2 and 97) under the “Prescribed Activities”. Internationally, United Nations Development Programme’s Social and Environmental Standards (2015 Policy) were used. The centrality of human rights, gender equality and women’s empowerment and environmental sustainability were considered throughout the course of project development. The Human rights principle and the five project-level safeguards standards were triggered, and these include:

- Principle 1: Human rights

- Standard 1: Biodiversity conservation and sustainable natural resources management

- Standard 2: Climate change mitigation and adaptation

- Standard 3: Community health, safety and working conditions

- Standard 4: Cultural heritage

- Standard 7: Pollution prevention and resource efficiency

The International Finance Corporation EHS guidelines for Annual Crop Production were also used to compliment the UNDP Social and Environmental Standards.

ES4: National Legal and Policy Framework

The legal and administrative framework governing environmental and social aspects was also identified and considered during identification of impacts and the associated mitigation measures. The legislation include the Environmental Management Act (Chapter 20:27), Water Act (Chapter 20:24), Public Health Act (Chapter 15:17), Labour Act (28:01), Forestry Act (Chapter 15:09), Parks and Wildlife Act (Chapter 20:14), Traditional Leaders Act (Chapter 29:17), Farm Feeds, Fertilizer and Remedies Act (Chapter 18:12), Factories and Works Act (14:08), Rural District Councils Act (Chapter 29:13), The Communal Lands Act (Chapter 20:04) and their applicable regulations. Relevant national policies include the Zimbabwe National Gender Policy (2017-2022), National Occupational Health and Safety Policy (2014), climate change policy (2017), Zimbabwe National Climate Change Response Strategy (2014), Zimbabwe National Environmental Policy (2009), etc.

ES5: Environmental and Social Assessment Methods

The alternative analysis of the proposed Project was conducted with respect to project site (Location), alternative technology (irrigation system), alternative water source and do-nothing alternatives. These alternatives were subjected to evaluation using multiple criteria, ranging from environmental, economic, social, climate change adaptation and resilience and stakeholder views. Environmental and social impacts were identified through a number of methods including those indicated in the UNDP’s Social and Environmental Standards which include stakeholder consultation, checklist, literature review, safety data sheets and professional judgement. Stakeholders were engaged through key informant interviews, questionnaires and focus group discussions. Key informants which include representatives from government agencies such as Responsible Parties (RPs), Rural District Councils, Zimbabwe National Authority, irrigation scheme members and non-members were consulted.

ES6: Anticipated Environmental and Social Impacts

The Environment and Social Assessment process undertaken identified and assessed a range of potential environmental and social impacts including health and safety risks associated with the Project. Positive impacts likely to emanate from the Project include employment creation; increased household incomes; women and youth empowerment; enhanced community livelihoods; improved food security and nutrition among beneficiaries; strengthened social capital; enhanced governance, knowledge and skills in modern irrigation farming and marketing participation in climate smart crop value chains, and operation and maintenance of irrigation infrastructure; and increased resilience and well-being of vulnerable smallholder beneficiaries. Despite these positives, the project is likely to bring negative environmental risks which include loss of biodiversity, water resource depletion, land degradation, water land and air pollution. Social risks include water use conflicts, land use conflicts, gender based violence (GBV), water borne diseases, child labour, poor working conditions, health and safety, sexual exploitation, abuse and harassment (SEAH), etc. In the event that tangible forms of cultural and archaeological importance are encountered during construction the chance find procedure will be triggered. However, these environmental and social risks are likely to be minimal and can be addressed through straightforward application of environmental siting, permitting requirements, pollution prevention, design criteria, construction standards, stakeholder engagement and training and awareness raising. The Project will not cause any economic or physical displacement of persons; neither will it be implemented in areas with indigenous peoples.

ES7: Environmental and Social Mitigation Measures

The following are some of the mitigation measures that will be adopted during construction, operational and decommissioning phases of the project:

- Cultivating crops that are suited or adapted to the local climate and soil conditions

- Adopting regenerative agriculture

- Reducing use of chemical fertilisers by incorporating organic materials (e.g., crop residues, compost, and manures) to replenish soil organic matter

- Minimising use of pesticides, selecting pest-resistant varieties and adopt Integrated Pest Management practices

- Adopt irrigation scheduling and promote water use efficiency

- Promote the use of solar energy

- Avoid open burning for land preparation, weed control, and post-harvest treatments

- Develop and implement the code of conduct on sexual exploitation, abuse and harassment

- Gender based violence and sexual exploitation, abuse and harassment awareness training

- Develop an appropriate irrigation plan and schedule, and monitor consumption and compare regularly with these targets which should be based on available supplies of water.

- Clearing of vegetation shall be restricted to the proposed work site and routes only

- Prohibit burning of pesticide-treated agricultural wastes and by-products

- Develop and implement the code on labour practice aligned to national laws and international best practices to minimise the risk of child labour

- Ensuring that employment opportunities are targeted at locals while taking into consideration gender perspectives;

- Activating chance find procedures during trenching and laying of underground works in the event of encountering tangible forms of cultural heritage;

- Engage landholders, traditional authorities and local governmental structures at district level on discussion sustainable models on land use to prevent disputes over land use

- Implement the health and safety plan including the related trainings

Specific management plans were developed to address the potential environment and social impacts. These plans are targeted at addressing the following:

- Water management

- Pollution Management

- Biodiversity and natural resources management

- Labour and working conditions

- Child safety management

- Land use management

- Cultural and Heritage management

- Occupational Health and Safety Management

- Community Health, Safety  and security

- Socio-economic management including GBV and SEAH related issues

- Air quality management

- Energy management

- Stakeholder Management Emergency preparedness and response

ES8: Roles and Responsibilities in ESMP implementation

ESMP implementation during construction and operation phases requires the involvement of several stakeholders and agencies each with specific roles and responsibilities. Key stakeholders involved in the implementation of the ESMP include the Project Management Unit (PMU), relevant government ministries and agencies, contractors, local leadership, project beneficiaries, etc. The Project steering committee and board has the responsibility for overseeing the implementation of the ESMP and giving strategic direction to any changes that may require changes in project design. The PMU will be responsible for overall coordination in the implementation of the ESMP and ensuring compliance with national, GCF and UNDP SES requirements. The Environmental Management Agency is responsible for providing overall oversight in monitoring the environmental and social performance of the project. The Contractors will be responsible for implementing the Contractor ESMP (C-ESMP) for their work sites.

The Project beneficiaries and local communities have the responsibility to routinely monitor environmental and social performance during all phases of project development to ensure that their constitutional rights are adequately observed and that the contractors effectively implement the mitigation measures provided in the ESMP. The local authorities and other relevant government agencies will be responsible for dealing with specific issues or conflicts that may arise during project implementation. The total cost of implementing the Project’s Environmental and Social Management plan is estimated at $180 000 USD. This mainly involve the cost of training and capacity building for ESMP implementation and monitoring. The cost of implementing mitigation measures by the contractor shall be borne by the contractor as may be stipulated in the contract of works.

ES9: Project Grievance Redress Mechanism

The Project has developed the Grievance Redress Mechanism (GRM) that shall be rolled out to all the Project implementation areas. The aim of the Grievance Redress Management System is to respond to and settle or redress any grievances, complaints, queries or clarifications from stakeholders in a manner that is legitimate, reliable, transparent, cost-effective, accessible and culturally appropriate to all parties. Some examples of possible grievances anticipated from the project relate to theft and vandalisation of assets, quality of irrigation equipment, GBV & SEAH related issues, project implementation delays, etc.

A multi-tier grievance redress structure is proposed to address project related complaints. It represents different stakeholders at the various levels of the conflict resolution process. Grievances will be resolved through committees constituted at four levels, namely: (1) Local/community level, (2) District level (3) Provincial level, and (4) Project focal point. The project stakeholders will be sensitised to take their complaints or grievances to these committees whenever they feel aggrieved. A reporting line of received (and addressed) grievances will be clearly defined, so that the Project Management Unit (PMU) will have a full set of data for monitoring the effectiveness of the GRM. Complaints will be categorized and recorded at each level of the GRM structure and consolidated periodically in a project grievance database. The database will also be an effective management tool to monitor progress and detect potential obstacles in project implementation. The database will be established in an easy-to-use software system (Microsoft Excel) to allow ease of use.

The necessary procedures for grievance resolution shall be activated until the matter is closed, preferably within 30days from the date the complaint is received. In the event that there is dissatisfaction from affected parties that cannot be resolved within the Project’s proposed grievance resolution process, the national Project focal point shall refer the dispute or complaint to arbitration. Where arbitration yield no expected result, then the courts system shall be the final authority to resolve a dispute. Where aggrieved parties feel that UNDP is not complying with its own social and environmental safeguards standards, the aggrieved party has the liberty to take the complaint to the UNDP Accountability Mechanism which has both compliance and grievance functions. Sensitive grievances including those of sexual nature shall be addressed through various actors or institutions such as local leadership, respected elders, clinics, church leaders, civic organisations and UNDP PMU. Under certain circumstances, the aggrieved party can opt to report the case to the Zimbabwe Republic police or seek the legal redress.

ES10: Conclusion

This ESMP concludes that the development of Masholomoshe and Masiyepambili irrigation schemes will generate significant positive impacts such enhancing food security, improving nutrition, employment creation and contribute to local economic development. The negative environmental and social impacts of the Project are likely to be minimal and can be easily addressed through implementing of readily available management measures. However, during the construction and operation of the Project there is need for periodic monitoring of water quantity and quality, soil quality, environmental flows, air quality, waterborne diseases, health and safety aspects and Sexual Exploitation, Abuse and Harassment (SEAH) by different stakeholders which include the beneficiaries, irrigation management committees, relevant government agencies, contractors and the PMU. If the environmental and social safeguard measures proposed in this ESMP are implemented, the benefits that the Project will bring to the communities will far outweigh the negative impacts which also of course shall be avoided, minimized or mitigated. Therefore, a green light should be given to the development of the two irrigation Schemes.

# List of Acronyms

| Abbreviations |  |
| --- | --- |
| AGRITEX | Agricultural, Technical and Extension Services |
| CSO | Civil Society Organization |
| DDC | District Development Coordinator |
| DDF | District Development Fund |
| ESIA | Environmental and Social Impact Assessment |
| EMA | Environmental Management Agency |
| GRC | Grievance Redress Committee |
| GCF | Green Climate Fund |
| GRM | Grievance Redress Mechanism |
| IAASTD | International Assessment of Agriculture Knowledge, Science and Technology for Development |
| IFC | International Finance Corporation |
| IMC | Irrigation Management Committee |
| ISDC | Irrigation Scheme Disciplinary Committee |
| M&E | Monitoring & Evaluation |
| MoLAFWRD | Ministry of Lands, Agriculture, Fisheries, Water and Rural Development |
| NGO | Non-Governmental Organization |
| NMMZ | National Museums and Monuments of Zimbabwe |
| PMU | Project Management Unit |
| SEAH | Sexual Exploitation, Abuse and Harassment |
| SEP | Stakeholder Engagement Plan |
| RDC | Rural Development Council |
| UNDP | United Nations Development Programme |
| ZRP | Zimbabwe Republic Police |

# TABLE OF CONTENTS

EXECUTIVE SUMMARY	3

List of Acronyms	9

TABLE OF CONTENTS	10

LIST OF FIGURES	14

LIST OF TABLES	15

LIST OF PLATES	17

1.0 INTRODUCTION	18

1.1 Introduction	18

1.2 Scope of the ESMP	20

2.0 PROJECT DESCRIPTION	22

2.1 Masholomoshe Irrigation Scheme	22

2.1.1 Project location	22

2.1.2 Irrigation scheme status	22

2.1.3 Current land use	23

2.1.4 Status of existing Infrastructure	23

2.1.5 Revitalization of the Irrigation Scheme	27

2.2 Masiyepambili Irrigation Scheme	30

2.2.1 Project location	30

2.2.2 Irrigation scheme status	30

2.2.3 Status of existing infrastructure	32

2.2.4 Irrigation Infrastructure Revitalization	34

2.3 Construction camps	34

2.4 Project Justification	34

3.0 PROJECT ALTERNATIVES	36

3.1 Consideration of alternatives overview	36

3.2 No Action Alternative	36

3.3 Irrigation Type Alternatives	37

3.3.1 Masholomoshe Irrigation Scheme	37

3.3.2 Masiyepambili Irrigation Scheme	38

4.0 POLICY AND LEGAL FRAMEWORK	40

4.1 Introduction	40

4.2 National policies and strategies	40

4.3 National legislation and regulations	43

4.4 UNDP’s Social and Environmental Standards Policy (2015)	50

4.5 International Commitments	52

4.6 Conclusion	55

4.7 Licenses/Permits required for the project	55

5.0 ENVIRONMENTAL AND SOCIAL BASELINE	56

5.1 Introduction	56

5.2 Climate	56

5.3 Topography	59

5.4 Hydrology	59

5.5 Ecological Assessment	64

5.5.1 Vegetation	64

5.5.2 Fauna	65

5.5.3 Entomology	66

5.6 Soil Assessment	67

5.7 Air Quality	69

5.8 Noise	70

5.9 Archaeology and Cultural Heritage	70

5.10 Baseline Socio-economic setting within the project area	71

6.0 STAKEHOLDER CONSULTATION	92

6.1 Stakeholder engagement methodology	92

6.2 Stakeholder Identification and Analysis	92

6.3 Key stakeholders consulted	92

7.0 IMPACT IDENTIFICATION AND ANALYSIS	118

7.1 Introduction	118

7.2 Environmental and Social Impact Assessment Method	118

7.3 Anticipated positive impactsacts	120

7.4 Anticipated negative environmental and social impacts	120

8.0 ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN	131

8.1 Introduction	131

8.2 Specific Environmental and Social Management Plans	131

8.3 Stakeholder Engagement Fees	173

8.4 Monitoring and Evaluation	173

9.0 ENVIRONMENTAL AND SOCIAL MONITORING PLAN	174

9.1 Monitoring Responsibilities and Reporting	174

9.2 Monitoring Program	174

10.0 CAPACITY DEVELOPMENT AND TRAINING	182

10.1 Introduction	182

10.2 Capacities Required	182

10.3 Objectives of the Capacity Building and Training	183

10.4 Methodology for the Capacity Building and Training	183

11.0 ESMP COST ESTIMATES	186

11.0 Estimated project cost	186

11.2 Estimated cost for implementing the environmental and social management plan	186

12.0 GRIEVANCE REDRESS MECHANISM	187

12.1 Introduction	187

12.2 The Project GRM	188

12.3 Timelines for resolving grievances	191

12.4 Other grievance redress channels	192

12.5 Without Prejudice	193

12.6 Cost of Implementing the GRM	193

12.7 Conclusion	194

13.0 CONCLUSION AND RECOMMENDATIONS	195

13.1 Conclusion	195

13.2 Recommendations	195

APPENDIX 1: LETTER OF CONSENT	196

Appendix 1: Focus Group Discussion Tool	197

Appendix 2: Key informant Questionnaire	208

Appendix 3: Some Stakeholder Responses	211

Appendix 4: Focus Group Discussion Summary Notes	213

Appendix 5: Grievance log sheet at PMU	217

APPENDIX 6: Chance Find Procedure	218

Appendix 7: Voluntary land use agreement forms	222

# LIST OF FIGURES

Figure 1 Locality Map of Masholomoshe Irrigation Scheme	23

Figure 2: Masholomoshe scheme layout	24

Figure 3 Nkashe dam capacities	25

Figure 4:Topographical map of the project area	31

Figure 5:Masiyepambili irrgation scheme map	32

Figure 6: Catchments of Zimbabwe	59

Figure 7: Mzingwanwe catchment showing main rivers, dams and towns	60

Figure 8: Drainage system in Gwanda district including Masholomoshe irrigation scheme area	61

Figure 9 Drainage system in Matobo district including Shashani river	62

Figure 10 Heavy sediment loads in Shashani river	63

Figure 11: Masholomoshe Irrigation scheme distribution of cumulative beneficiaries	71

Figure 12 Distribution of irrigation scheme representation by age group (N=127)	72

Figure 13:Masiyepambili Irrigation scheme distribution of cumulative beneficiaries	73

Figure 14: Masholomoshe attendance among children of school going age	77

Figure 15: Representation of source of labour for farming	78

Figure 16 Estimated monthly household income among scheme beneficiaries	79

Figure 17: Masholomoshe irrigation scheme household expenditure on moveable assets	80

Figure 18: Gender based violence stakeholder mapping	85

Figure 19: Project Resolution Process	190

Figure 20: Schematic procedure for handling sensitive grievances	191

# LIST OF TABLES

Table 1: Risks and remedial actions to climate proofing the scheme	28

Table 2: Evaluation of no action alternative	36

Table 3: Evaluation of irrigation type alternatives	37

Table 4: Evaluation of irrigation type alternatives	39

Table 5: Operational standards triggered under the Project	51

Table 6: Licenses/Permits	55

Table 7: Average monthly climate data	57

Table 8: Projected climate data for the year 2050 Climate Data	58

Table 9: Water Analysis Data	63

Table 10:Flora registerd at Masiyephambili irrigation scheme site	65

Table 11:Species of birds registered in the Project area	66

Table 12: Soil chemistry in the project area	67

Table 13: Soil Profile	69

Table 14: Air Quality Results	70

Table 15: Ambient noise levels at different sites	70

Table 16: Prospective Nkashe dam water users	74

Table 17: Average livestock drinking water requirements	74

Table 18: Current irrigation land allocation status	75

Table 19 :Estimated crop yields	82

Table 20: Determined crop types, land use patterns, crop rotations and water requirements for Masholomoshe Scheme	86

Table 21: Irrigation water management plan for Masholomoshe Scheme	87

Table 22 Determined crop types, land use patterns , crop rotations and water requirements for Masiyepambili Scheme	89

Table 23 Irrigation Water Schedule Master Plan for Masiyepambili Irrigation scheme	90

Table 24 Stakeholder Identification and Analysis	93

Table 25 Key Consulted Stakeholders and their views in relation to Masholomoshe scheme	96

Table 26: List of stakeholders consulted and their views for Masiyepambili scheme	104

Table 27: Rating the ‘Impact’ of a Risk	118

Table 28: Rating the ‘likelihood ‘of a risk	119

Table 29: Determining ‘Significance’ of Risk	119

Table 30 Environmental and Social Impacts Rating	121

Table 31 Pollution Prevention and Resource Efficiency Management Plan	132

Table 32 Water Management Plan	136

Table 33 Biodiversity and Natural Resources Management Plan	139

Table 34: Land Use Management Plan	141

Table 35 Energy Management Plan	144

Table 36 Air Quality Management Plan	145

Table 37 Cultural Heritage Management Plan	147

Table 38: Community Health, Safety and Security Management Plan	148

Table 39: Labour Management Plan	153

Table 40: Child Safety Management Plan	155

Table 41 Gender based violence management plan	155

Table 42 Sexual exploitation, abuse and harassment management plan	158

Table 43 COVID-19 management plan	160

Table 44: Emergency Preparedness and Response Plan	160

Table 45 Stakeholder Engagement Plan	164

Table 46: Stakeholder Engagement Plan implementation schedule	167

Table 47 Stakeholder Engagement Activities	173

Table 48: Environmental Monitoring Plan	176

Table 49: Social Monitoring Plan	179

Table 50: Capacity building and training plan	184

Table 51: Environment and social management plan implementation cost	186

Table 52: Suggested timeframes for grievance redress	192

Table 53: Cost of implementing and operating the GRM	193

# LIST OF PLATES

Plate 1: Conveyance showing cracks and unsealed joints	25

Plate 2: Night storage dam	26

Plate 3: Distribution boxes and field canals	26

Plate 4: Excessive Siphon discharge resulting in ponding at field	27

Plate 5: Typical centre pivot	29

Plate 6: Concrete storage tank and defunct mono-pump and pressure tank	33

Plate 7: Rejuvinating acacia bushes	64

Plate 8: Irrigation scheme nursery (left) and maize field (right)	82

Plate 9: Municipality of Gwanda carrying out gender based violence awareness	84

# 1.0 INTRODUCTION

The United Nations Development Programme (UNDP) has commissioned Sesani (Pvt) Ltd to conduct detailed feasibility and climate proof designs of GCF funded irrigation schemes in Matabeleland South in Zimbabwe

## 1.1 Introduction

The Government of Zimbabwe through the Ministry of Lands, Agriculture, Fisheries, Water and Rural Development (MLAFWRD) in partnership with United Nations Development Programme (UNDP) is implementing a seven-year Project “Building the climate resilience for vulnerable agricultural livelihoods in Southern Zimbabwe”. The Project aims to strengthen the adaptive capacities of vulnerable smallholder farmers, especially women, to climate change induced impacts on their agro-ecosystems and livelihoods through revitalization of irrigation schemes, upgraded water and soil moisture management and water use efficiency, climate-resilient agriculture, improved access to climate information and markets, and partnerships with public and private sector actors. The project is targeting the revitalization of 15 existing irrigation schemes and the establishment of 6 new schemes in 15 priority districts of Manicaland, Masvingo and Matabeleland South Provinces.

The irrigation schemes have been classified into three lots, namely lot 1, 2 and 3. Each lot is defined by a specific hydrological catchment with similar biophysical and socio-economic characteristics. Lot 1 consist of three irrigation schemes and all are situated in Matabeleland South Province. Lot 2 consist of 7 irrigation schemes and all are situated in Masvingo Province. Lot 3 consist of 5 irrigation schemes and all are situated in Manicaland Province. The other 5 remaining schemes (presumably lot 4) are still yet to receive GCF approval. Sor far, a site specific Environmental and Social Management Plan (ESMP) has been prepared for each of the irrigation schemes under Lot 1 and Lot 2. Studies show that that the ESMPs for the schemes that fall in the same lot largely share the same content particularly in terms of baseline information, environmental and social impacts, management measures and grievance redress mechanisms. Based on this, it was therefore logical to cluster the ESMPs by lot to ease the review process.

This ESMP is therefore a consolidation of the site-specific ESMPs developed for Masiyepambili and Masholomoshe Irrigation Schemes which are two of the three irrigation schemes under Lot 1. The two irrigation schemes are part of the 21 irrigation schemes targeted for construction, revitalization and climate proofing under the Project. The two irrigation schemes are located in Matabeleland South Province of Zimbabwe.

Climate proofing of the schemes will be achieved through installation of a water efficient irrigation system, adoption of climate-smart crop value chains, improved irrigation scheduling, crop rotation, improved agri-business practices, soil conservation practices, use of solar energy, capacity building of the IMC, improved access to hydro-meteorological information, robust designs for the civil, structural, and irrigation infrastructures, constituting a WPC for seasonal water allocation, and improved catchment management.

The project was screened against the GCF’s and UNDP’s Social and Environmental Standards and was classified as a moderate risk project. According to the Environmental Management Act Chapter 20:23 of 2002 of Zimbabwe an ESMP is necessary for this project. To comply with the Environmental Management Agency, Green Climate Fund’s and UNDP’s Social and Environmental Requirements, the Ministry of Lands, Agriculture, Fisheries, Water and Rural Development (MLAFWRD) engaged Sesani to assess the environmental and social impacts of the proposed irrigation revitalization project and come up with an Environment and Social Management Plan on its behalf.

The Environment and Social Management Plan was produced as per the requirements of Environmental Management (Environmental Impact Assessment and Ecosystem Protection) Regulations, SI 7 of 2007 and in fulfilment of the requirements of the Environmental Management Act (CAP 20:27) – First Schedule (Section 2 and 97) under the “Prescribed Activities”. Internationally, the United Nations Development Programme Social and Environmental Standards, GCF’s Environmental and Social Policy and International Finance Corporation EHS guidelines for Annual Crop production were made reference to.

Potential environmental and social impacts were identified by overlaying project activities on various components of the environment and social domains to establish potential interactions. Various techniques such as matrices, networks, field observations, expert knowledge, public consultations and checklist as provided in Zimbabwe’s EIA guidelines were used A risk analysis matrix combining the “impact” and “likelihood” of occurrence was used to evaluate the level of significance of the potential impacts. Potential environmental impacts identified include loss of biodiversity;  water resource depletion; land degradation; water, land and air pollution; and social impacts include water conflicts, land conflicts, gender bias, water borne diseases, sexual exploitation, abuse and harassment, poor labor conditions, child labour, health and safety incidences, etc.

The legal framework governing all environmental and social aspects was also identified and taken into account during identification of impacts and mitigation measures. The legislation include the Environmental Management Act CAP 20:27, Water Act (Chapter 20:24), Public Health Act (Chapter 15:17), Labour Act (Chapter 28:01), Forestry Act (Chapter 19:05), Parks and Wildlife Act (Chapter 20:14), Traditional Leaders Act (Chapter 29:17), Farm Feeds, Fertilizer and Remedies Act (Chapter 18:12), Factories and Works Act (14:08), Rural District Councils Act (Chapter 29:13), The Communal Lands Act (Chapter 20:04) and their applicable regulations.

The ESMP process undertaken has identified and assessed a range of potential environmental and social impacts associated with the proposed Project and the associated management measures. Provided that this ESMP is implemented religiously, the potential environment and social risks will be reduced to insignificance level. The project also comes with positive impacts which include employment creation, improvement of community livelihoods, improved food security and nutrition among irrigation scheme members, Appropriate ESMPs were developed to mitigate and minimize the environment and social consequences of identified actions. These are presented in chapter 7. This management plan has been developed to deal with the following specific issues:

- Water quality and water quantity Management

- Waste management;

- Air quality management

- Biodiversity management

- Energy management

- Land use management

- Hazardous Substances Management

- Cultural and Heritage sites management

- Occupational Health and Safety Management

- Community Health, Safety and security management;

- Emergency Preparedness and Response

- Child Safety

- Sexual Exploitation, Abuse and Harassment

- Socio-economic management

The environmental and social impacts can be managed to acceptable levels if the ESMP is fully implemented at all phases of project development. The commitment and participation of all stakeholders is required for the irrigation scheme to be successful.

## 1.2 Scope of the ESMP

An experienced consultant registered with the Environmental Management Agency was engaged to prepare the comprehensive Environment and Social Management Plan. Specific objectives of the assignment include:

- To prepare Environment and Social Management Plan (ESMP) indicating the recommended/proposed structural, biological, and operational measures to incorporate in the design and operation of the schemes to mitigate or reduce the negative impacts identified.

- To assess social-cultural aspects and impacts associated with irrigation revitalisation project.

- Provide the management measures for addressing the identified impacts

- To document an ESMP Report with the following layout

Chapter 1: Introduction

Chapter 2: Project Description – locality, and technical details of the project,

Chapter 3: Alternatives considered.

Chapter 4: Policy, Legal and Administrative Framework

Chapter 5: Baseline Environmental and Social setting

Chapter 6: Public Consultation

Chapter 7: Identification and Assessment of Potential Environmental and Social Impacts

Chapter 8: Environmental and Social Management Plan

Chapter 9: Environmental and Social Monitoring Plan

Chapter 10: Capacity Development and Training

Chapter 11: ESMP Cost Estimation

Chapter 12: Grievance Redress Mechanism

Chapter 13: Conclusion and Recommendations

# 2.0 PROJECT DESCRIPTION

This Chapter describes the status at each irrigation scheme and provides a description of the proposed climate proofing activities to revitalise the scheme. The section concludes by providing a justification for the Project.

## 2.1 Masholomoshe Irrigation Scheme

### 2.1.1 Project location

Masholomoshe Irrigation Scheme is located in Ward 1, Gwanda District, Matabeleland South Province. Access to the scheme from Gwanda town is along the Gwanda-Bulawayo Road (approximately 10km) and turn right into a dirt road and travelling a further 6 km to Masholomoshe Scheme. Figure 1 shows the topographical map of the project area. The UTM geo reference is 7183363mE and 7690895mS.

### 2.1.2 Irrigation scheme status

Masholomoshe irrigation scheme has total irrigated area of 38.8 hectares. Currently, the scheme has 132 members. The scheme has an Irrigation Management Committee (IMC) with a constitution which guides its operations. The scheme is situated in agro-ecological region V, a very dry region characterized by low rainfall and high temperatures. Severe droughts and floods due to climate change effects are apparent in the southern part of the country and has impacted the rural population who are more dependent upon a set of atmospheric and hydrological circulations. Reductions in river runoff and aquifer recharge as well as ever rising temperatures has become a threat to naturally occurring plants which are a source of food to their livestock. The rising temperatures also translate into increased crop water demand. Livelihoods in this rural community and food security are therefore at risk from water-related impacts linked primarily to climate change. The rural poor women, who are the most vulnerable, are disproportionately affected.

Figure 1 Locality Map of Masholomoshe Irrigation Scheme

### 2.1.3 Current land use

Masholomoshe is an existing irrigation scheme which is currently 38.8 hactares in size. The irrigation scheme is divided into 3 blocks. The farmers in the irrigation scheme grow sugar beans, maize, carrots, rape, onion, tomatoes, cabbage, beetroot, lettuce, cornflower and tsunga. Most the crops grown at the scheme are perishables.

### 2.1.4 Status of existing Infrastructure

System Layout

Masholomoshe scheme drains its water from Nkashe Dam which is approximately 2km upstream of the scheme. The water is released through a gate valve into a concrete lined conveyance canal – length 2500 meters. A network of lined canal supplies irrigation water to the three blocks (Block A, B and C) covering an irrigated area of 38.8ha. Figure 2 shows the current irrigation scheme layout.

Figure 2: Masholomoshe scheme layout

The scheme is supplied with water from the Nkashe Dam. Water from the dam is released through a gate valve into a concrete line canal into night storage dam in which the irrigation water is drawn. The earth-fill dam was constructed in 1964 and is still in relatively good condition. The full capacity of the dam is 893 ML. Nkashe dam has a capacity of 750ML in April and 170ML during in August (Figure 3). It was established that the dam rarely spills (only spills when there is good rain season). Currently the dam does not have a device for flow measurements. Gauging steps are proposed. These gauging steps will be calibrated using the elevation-area-storage graph which was generated from the bathymetric survey undertaken in this Study. Figure 3 shows the historical dam levels in April and October for the past 20 years.

Figure 3 Nkashe dam capacities

The graph shows that the dam has been almost emptied (below 5%) in early 1990s, 1997-2000, 2004-2007, 209-2012, 2014-19. Farmers’ concern on the dam water supply reliability was siltation which is happening due to erosion up-catchment.

Water Conveyance System

The water from the dam is conveyed to field edge through a concrete line canal of bed width 500mm (trapezoidal section). The conveyance canal is functional, and rehabilitation is required in places where there was damage due to cyclone induced floods (Plate 1).

Plate 1: Conveyance showing cracks and unsealed joints

Night Storage Dam

A night storage dam (Plate 2) was constructed on a vantage point within the irrigation scheme. The night storage dam is compacted of earth fill and has a capacity of 2500m3. The dam has a length of 50m, width of 48m and a depth of 1.5m. The dam is 30m away from the river. The night storage dam serves to store water during the night hours and in the day the stored water is used for irrigating block A (about 10 hectares). The night storage dam is still intact and requires routine maintenance.

Plate 2: Night storage dam

Infield Structures

The infield system consists of a network of concrete lined canals taking off from the main canal through gated distribution boxes. A total of six field canals supply water for irrigation to blocks A, B and C. The total infield canal network is 2500 meters. The distribution boxes and field canals are in good condition requiring routine maintenance (Plate 3).

Plate 3: Distribution boxes and field canals

Water Management

The drainage network is dilapidated as evidenced by the water logging at the end of the fields. This is further exacerbated by excessive irrigation practice being done in the scheme (Plate 4).

Plate 4: Excessive Siphon discharge resulting in ponding at field

### 2.1.5 Revitalization of the Irrigation Scheme

In addition to managing climate risks and hazards, the project has the unique potential to increase and stabilize crop yields by allowing multiple cropping year-round. Irrigated crop production enhances farm incomes from sale of surplus yields and off-season premiums and provides employment to others through additional on-farm labor. This contributes to improved food security, enhanced livelihoods and increased resilience and wellbeing for vulnerable smallholder communities. Moreover, smallholder farmers will also benefit from access to more accurate, dependable, and tailored information on weather, climate, and hydrological resources, which will allow them to plan agricultural tasks and manage crops, soil and water to reduce water stress or take advantage of rain or irrigation potential to reduce water stress.

A summary of the existing challenges and proposed activities to climate proof the irrigation scheme are provided in the Table 1.

Table 1: Risks and remedial actions to climate proofing the scheme

| Risks | Remedial Actions |
| --- | --- |
| Water supply risk from Nkashe dam from periodic droughts | Match irrigated area to long term yield of the dam. Intensive irrigation to be practiced on 16ha area. The remaining 22.8ha will be utilised in seasons with good rainfalls. |
| Water losses in the water conveyance canals | Install a PVC pipeline to convey water from the dam to a pump station. |
| Water losses from overflowing canals |  |
| Water losses in the night storage dam | By-pass the night storage dam and pump directly into the irrigation scheme |
| Water losses from inefficient irrigation technology | Install a centre pivot irrigation system. This will reduce water application losses from about 50% to about 20%. |
| Cropping system is low value and prone to climate shocks | Adopt climate smart crop value chains. |

Solar system installation

The irrigation scheme will be solar powered. A solar system of 200 Kw capacity will be installed near irrigation scheme office to cater for energy requirements for the irrigation scheme. The solar system will include solar array to convert the irradiation from sunlight to electrical energy required by pumps.

Center pivot irrigation system installation

A center pivot irrigation system will be installed at the irrigation scheme (Plate 5). The existing canals will be decommissioned. To minimise environmental damage, a 1.5m deep and 2m wide trench will be excavated for a distance of 2km and install 300 PVC pipeline at the same position where the canal was decommissioned. The system consists of a pipeline with sprinklers. The pipeline (lateral) is supported by steel frameworks (towers) spaced along the lateral. The towers are mounted on wheels, enabling the system to rotate around a pivot point. The center pivot feed pipe will have a diameter of 200m and two pivots of 190m and 150m radius. The two pivots will irrigate 8ha each. Water pumped from the dam is delivered to the pivot and into the lateral through a supply pipeline. The center pivot irrigation systems will consists of the following:

- Pivot Point – The central point around which the pivot moves. This is where the water enters the pivot pipes and where the control panel is located.

- Control Panel – A piece of hardware attached to the pivot point that gives commands to the center pivot machine. The control panel controls the starting, stopping, changing of directions, running wet versus dry etc.

- Drive Unit/Drive Tower – A drive unit or drive tower is the part of the machine that touches the ground, and contains the necessary components for the machine to move. It consists of a base beam, drive train, wheels, and various structural supports.

- Span - The long pipes between drive units are called spans. Spans consist of the main water line, sprinklers, and a supporting structure of trusses to hold the weight between towers.

- Tower Box – Located at each drive unit is a tower box. This compartment controls the drive unit components, telling it to move in the right direction and for how long.

- Last Regular Drive Unit – The Last Regular Drive Unit is the last tower on a regular pivot or the last tower before a corner arm or pivot add-on that extends irrigated area.

Plate 5: Typical centre pivot

Pump station installation

A pump station will be installed to pump water from Nkashe dam to night storage dam. Three (3) axial flow pumps (delivers considerably more water while significantly reducing fuel energy use) of 22.5 Kw each will be installed. At each given time, one of the three pumps will be on standby. The pump station will be installed on a ground which is 15m higher than the river level to minimize damage of the pump station by flooding.

## 2.2 Masiyepambili Irrigation Scheme

### 2.2.1 Project location

Masiyepambili irrigation scheme is situated in Ward 2, Matobo District, Matabeleland South Province. The irrigation scheme is located at coordinate 643047.56mE and 7676673.09mS approximately 20km from Maphisa growth point. Access to the project site is from Maphisa growth point about 10km towards the Antelope Dam and turning off into Zamanyoni village, then travelling a further 10km to the Shashane River using a poorly gravelled road. Figure 4 and Figure 5 show the satellite image and the topographic map of the project area respectively.

### 2.2.2 Irrigation scheme status

Currently there is no irrigation activity happening onsite since 2006. However, 30 direct beneficiaries are still registered with the irrigation scheme. There is one Project Affected Person (PAP) by the name “Gracius Dube“ who is currently utilising part of the land of the scheme for horticultural production. The PAP was consulted and is willing to be incorporated into the scheme once it is vitalised (See Appendix 1). The project will not result in any physical displacement of persons.

Figure 4:Topographical map of the project area

### 2.2.3 Status of existing infrastructure

System Layout

Masiyepambili irrigation scheme is designed to draw its water from Shashane River, although not functioning currently. Irrigation water was being pumped from riverbed sand abstraction system situated in Shashane River using buried manifold PVC pipe (90mm diameter) connected to the sand abstraction system to a concrete circular holding tank (100m3 in capacity) at a vantage point to the command area. From the tank, a main canal bed width 250mm delivered water to a filed network of canals. Irrigation was by means of siphons abstracting water from the field canals into borders. The pump station is now defunct. The concrete holding tank is still in good condition. Figure 5 shows the layout of Masiyepambili irrigation scheme.

Figure 5:Masiyepambili irrgation scheme map

Infield Structures

Canals

The infield structures consist of a main canal that branches into field canals network supplying irrigation water to a net area of 4.8ha. The field canals are brick lined. They have a bed-width of 250mm, and they are no longer functioning with visible cracks. Plate 6 shows the concrete storage tank and defunct mono-pump and pressure tank.

Plate 6: Concrete storage tank and defunct mono-pump and pressure tank

Pump House

The pump house has since been vandalised and the security fence was stolen with only three poles remaining. The pump house walls have begun to subside into the riverbed as a result of the river extending its banks due to lateral erosion. The pump house renovation works will involve repairing walls, new roof, door, sealing leaks.

Water Storage Reservoir

The water tank reservoir has cracks which need to be repaired. At the time the irrigation scheme stopped functioning, this reservoir did not have any performance or structural issues such as leaks. The concrete reservoir water capacity could not be established.

Office and Storage Shed

The office and storage shed have become totally unusable; the asbestos sheets roof was damaged when the wooden beams supporting it were eaten away by termites. The storage shed is where tools were kept. The Irrigation Management Committee operated from the office, which was also used to conduct trainings and meetings

### 2.2.4 Irrigation Infrastructure Revitalization

The scheme will be revitalised to irrigate an area of 10ha. A sand abstraction system will be installed in the river bed adjacent to the scheme for abstraction of water to supply the scheme. A solid set sprinkler or drip irrigation system will be installed. Submersible pumps will pump water from the river bed into a sump for onward pumping to the irrigation scheme. Solar panels will be used to power the pumps.

## 2.3 Construction camps

Mobile construction camps will be erected at each scheme. The camps will provide accommodation for skilled workforce. Unskilled labour will be recruited from the local community. Temporary cabin that will be manned by locally hired security will be erected for storage of construction material. Alternatively, the contractor will liaise with farmers for material storage as done by previous contractor.

## 2.4 Project Justification

With continued intensification of climate variability and change, Zimbabwe’s current coping strategies for the agriculture and water sectors are becoming increasingly ineffective, requiring essential adaptation investments to achieve lasting climate resilience among vulnerable rural farming households. Adaptation to climate change for vulnerable smallholder farmers requires resources and capacities for adaptive management of their agro-ecosystems with the aim of stabilizing, increasing and sustaining agricultural yields and incomes. In areas that are becoming drier and hotter, particularly the southern part of Zimbabwe, these resources include access to, as a priority, sufficient, dependable water and a diversity of climate-resilient crop varieties and livestock breeds and management practices.

Through irrigated agriculture, the Project will contribute to improved food security, enhanced livelihoods and increased resilience and well-being for vulnerable smallholder communities particularly women. Irrigation has unique potential to increase and stabilize crop yields by allowing multiple cropping year-round. Irrigated crop production enhances farm incomes from sale of surplus yields and off-season premiums and also provides employment to others through additional on-farm labour. Furthermore, problems of malnutrition in children from disadvantaged families will be addressed. Besides, smallholder farmers will also benefit from access to more accurate, dependable technologies and tailored information on meteorological, climate and hydrological resources. This gives the smallholder farmers an opportunity to plan their agricultural activities and manage crops, soil and moisture to reduce water stress during different weather conditions.

# 3.0 PROJECT ALTERNATIVES

## 3.1 Consideration of alternatives overview

The consideration of alternatives is fundamental for managing environmental and social impacts for developmental projects. This technique evaluates alternative sites, methods, materials, designs and other factors incidental to the project context. These factors are subjected to evaluation using multiple criteria, ranging from environmental, economic and social spectra. In this ESMP, consideration of alternatives started with the No revitalisation option, revitalisation alternative and alternative irrigation system. Information from current baselines as well as input from stakeholder consultation were considered in the evaluation process.

## 3.2 No Action Alternative

The Do-nothing alternative considers non-implementation of the project, which would mean no revitalisation of the irrigation schemes and continued water losses in the system as outlined in the need for the Project. Not implementing the proposed project would lead to continued food insecurity and loss of livelihoods in the community. Table 2 provides the evaluation of the no-action alternative.

Table 2: Evaluation of no action alternative

| CRITERIA | DETAILS |
| --- | --- |
| Environmental performance | No changes mean no additional or new impacts to the environment associated with project construction, operation and decommissioning phases. Water and energy demands will remain relatively low |
| Economic Efficiency | Failure to revitalise the irrigation schemes means losses in financial opportunities for the community. |
| Social performance | There will be no improvement in food security, nutrition, and livelihood of the community. No jobs will be created for the community. |
| Climate change adaptation and resilience | No changes means there is no resilience and adaptation to climate change and the effects will keep on worsening to the community |

Overall, “No Action alternative” is not an option at all as it will not help address food insecurity experienced in the country due to the effects of changing climate. The No Action Alternative will not help the country in realising the vision 2030. Nevertheless, the Project the may be implemented in other alternative options with very minimal environmental and social risks. These alternatives are further evaluated below using the above criteria.

## 3.3 Irrigation Type Alternatives

The selection of the best irrigation method for each scheme involved analysing a series of operations that yield the best trade-off between minimization of input requirements (water, energy, labour, etc.) and maximizing the crop harvests. Alternative irrigation methods that use less water, but produce high yields, contribute to agricultural sustainability are the ones needed especially in a time where climate change effects are being witnessed.

### 3.3.1 Masholomoshe Irrigation Scheme

Irrigation technologies

At Masholomoshe irrigation scheme, three irrigation alternatives were analysed which include (1) centre pivot, (2) sprinkler irrigation and (3) surface irrigation. Table 3 provides a multi-criterial evaluation of irrigation alternatives.

Table 3: Evaluation of irrigation type alternatives

| Criteria | Sub criteria | Center pivot | Sprinkler | Surface |
| --- | --- | --- | --- | --- |
| Environmental performance | Sustainability of water use | Use moderate quantity of water | Use moderate quantity of water | Uses high quantities of water as the crop field is flooded with water |
|  | Erosion risk | do not cause much soil erosion due to water coverage under the system is very orderly | moderate risk of soil erosion due to water drops splash effect | There is high risk of erosion due to flowing water in the crop field. |
|  | Soil salinization | There is high risk of salinization due to excessive water use | There is high risk of salinization due to excessive water use | There is high risk of salinization due to excessive water use |
| Economic Efficiency | Financial cost | High costly during | High costly | Low costly |
| Social performance | Employment creation | There is employment creating | There is employment creating | There is employment creating |
|  | Community participation | The community accepted the initiative are willing to support the project | The community accepted the initiative are willing to support the project | The community did not support the initiative due to high water requirements hence competing with other users |
| Adaptation and resilience | Vulnerability | Moderately vulnerable to lower rainfall which may result in reduced water supply | Moderately vulnerable to lower rainfall which may result in reduced water supply | Highly vulnerable to lower rainfall which may result in reduced water supply |

Overall, centre pivot irrigation is mostly viable due to minimal water requirements considering limited water supply available due to the changing climate.

Water Source

Dam water and borehole water options were considered for the two schemes. Analysis of water quality results indicated that the dam is suitable for irrigation but groundwater is not suitable due to the high salinity which can reduce crop yields. Furthermore, groundwater water yields are very low. Hence only dam water will be utilised for revitalisation of Masholomoshe irrigation scheme.

### 3.3.2 Masiyepambili Irrigation Scheme

Three irrigation technologies were evaluated for Masiyepambili irrigation scheme namely (1) drip irrigation, (2) solid set sprinkler irrigation and (3) surface irrigation. Table 4 provides a multi-criterial evaluation of irrigation technologies.

Table 4: Evaluation of irrigation type alternatives

| Criteria | Sub criteria | Drip | Solid Set Sprinkler | Surface |
| --- | --- | --- | --- | --- |
| Environmental performance | Sustainability of water use | Lower quantity of water is used | Lower quantity of water is used | Uses high quantities of water as the crop field is flooded with water |
|  | Erosion risk | There is very low risk of soil erosion as water is only applied on the crop | Low to moderate risk of soil erosion | There is high risk of erosion due to flowing water in the crop field. |
|  | Soil salinization | There is very low risk of salinization as water is only applied on the crop. Little water is used. | Low to moderate risk | There is high risk of salinization due to excessive water use |
| Economic Efficiency | Financial cost | High to moderate capital costs. High operational costs. | High to moderate capital costs. Low operational costs. | Low capital cost. Low operational costs. |
| Social performance | Employment creation | There is employment creation | There is employment creation | There is employment creation |
|  | Community participation | The community accepted the initiative and are willing to support the project | The community accepted the initiative and are willing to support the project | The community did not support the initiative due to high water requirements hence competing with other users |
| Adaptation and resilience | Vulnerability | Low water requirements | Low water requirements | High water requirements |

Overall, drip and solid set irrigation can be considered as viable options for the irrigation scheme based on low water requirements. The lower operational costs of the solid set sprinkler system makes it more suitable.

# 4.0 POLICY AND LEGAL FRAMEWORK

## 4.1 Introduction

This section focuses on the Environmental and Social legislation, policies, standards, guidelines, conventions that have a bearing on the establishment and operations of the proposed project. Sections/clause of the legislation, policies, standards, guidelines and conventions that are likely to affect or have implications for the proposed project are highlighted. Modern day environmental principles and policies are centred on the 1980 World Conservation Strategy which argues that for development to be sustainable, it should support conservation rather than hinder it. It is also argued that local groups need rights over their own environment and should benefit from its development. This was also embraced by millennium development goals (MDGs) in 2000 which was succeeded by sustainable development goals in 2015. For any development to be sustainable it must take into account the environmental, social and economic factors.

## 4.2 National policies and strategies

- Zimbabwe National Environmental Policy (2009)

The vision of the National Environmental Policy of Zimbabwe is to alleviate poverty and improve quality of life of the people of Zimbabwe. The goal of this policy is to avoid irreversible environmental damage, maintain essential environmental processes and preserve the broad spectrum of biological diversity so as to sustain the long-term ability of natural resources to meet the basic needs of people, enhance food security, reduce poverty, and improve the standard of living of Zimbabweans through long term economic growth and creating of employment

- Zimbabwe National Climate Change Response Strategy (2014);

The goal of the Response Strategy is to mainstream climate change adaptation and mitigation strategies in economic and social development at national and sectoral levels through multi-stakeholder engagement. Some of its strategic objectives include the develop climate proofed and environmentally sustainable transport systems that are less carbon intense; promoting sustainable development, management and utilization of water resources under changing climatic conditions and promoting sustainable land-use systems that enhance agricultural production, ensure food security and maintain ecosystem integrity.

- Zimbabwe National Climate Policy (2017)

The policy’s goal is to enable Zimbabwe to establish the legal structures to regulating businesses in climate-related matters, and enable them to reduce their greenhouse gases emissions. More relevant to the project, the focuses on adaptation with regard to rural communities and agriculture.

- Zimbabwe National Biodiversity Strategy and Action Plan (2014)

The objectives of the strategy focus on addressing the underlying causes of biodiversity loss by mainstreaming biodiversity across government and society; to reduce the direct pressures on biodiversity and promote sustainable use; to improve the status of biodiversity by safeguarding ecosystems, species and genetic diversity; to enhance the benefits to all from biodiversity and ecosystem services.

- Zimbabwe National Occupational Health and Safety Policy (2021)

The aim of this policy is to eliminate occupational accidents, injuries, diseases, and fatalities and to promote Occupational Safety and Health (OSH).

- Zimbabwe National Agriculture Policy Framework (2018-2030);

The overall objective of the National Agriculture Policy Framework (NAPF) is to provide policy guidance and direction on how to promote and support the sustainable flow of investments to transform the agricultural sector through increased and sustained agricultural production, productivity and competitiveness. Its goals are to ensure national and household food and nutrition security, increase agriculture’s contribution to the gross domestic product (GDP) and improve agricultural market access and competitiveness.

- Zimbabwe National Renewable Energy Policy (2019);

The policy focusses on energy issues, its impact on other non-energy sectors and significant potential for value creation along the different segments of the renewable technologies. Key socio-economic areas addressed in the policy relevant to the project include affordability and accessibility of energy from renewable sources, employment opportunities, gender equity, and benefits to children, youth participation, food security, and improved access to basic facilities.

- Zimbabwe National Development Strategy (2021-2025)

The objectives of this strategy are to strengthen macroeconomic stability, characterized by low and stable inflation, as well as exchange rate stability, to promote new enterprise development, employment and job creation and ensure sustainable environmental protection and resilience.

- Zimbabwe National Climate Change Response Strategy

It provides a framework for a comprehensive and strategic approach to climate change adaptation, mitigation, technology and finance. The strategy aims to mainstream climate change adaptation and mitigation strategies in economic and social development at national and sectoral levels through multi-stakeholder engagement. The strategy recommends frequent assessment of surface and groundwater resources and promotion of water use efficiency in projects. Measures should be developed to rehabilitate, maintain and protect surface and groundwater resources. The strategy also recommends farmers to use high water and energy efficient irrigation systems.

- Zimbabwe National Gender Policy (2017-2022)

The policy seeks to achieve a gender just society where men and women enjoy equality and equity and participate as equal partners in the development process of the country. The policy focuses on eight priority areas namely: Gender, Constitutional and Legal Rights; Gender and Economic Empowerment; Gender, Politics and Decision Making; Gender and Health; Gender, Education and Training; Gender Based Violence; Gender and Environment; and Gender, Media and ICTS. The policy is underpinned by principles of gender justice, equality, integration and inclusiveness. The policy recognises that Zimbabwe’s economy is agro based, hence subject to climate change risks. The largest group of people in farming activities are women (86%). The policy acknowledges that women are already in a disadvantaged position, and the effects of climate change threaten to further increase the inequality. The reliance of women on natural resources for food and income, limited access to productive resources, combined with their disadvantaged position in society increases their vulnerability to climate change induced distress. The policy states that women have a significant role to play in climate change adaptation and mitigation as they acquired environmental management skills through experience in utilising natural resources. The policy therefore stresses that it is imperative to make gender considerations in climate change and environment conservation strategies. This policy therefore commits to spearheading a specific gender responsive effort in management of the environment particularly focusing on the exacerbation effects of climate change on pre-existing inequalities between men and women.

## 4.3 National legislation and regulations

The following are the laws and regulations relevant to the revitalization of the two irrigation schemes.

- The Constitution of Zimbabwe Amendment Act of 2013

Section 73 of the Zimbabwe Constitution stipulates that every person has a right to an environment that is not harmful to their health or well-being and to have the environment protected for the benefit of present and future generations, through reasonable legislative and other measures that prevent pollution and ecological degradation, promote conservation and secure ecologically sustainable development and use of natural resources while promoting economic and social development.

- Environmental Management Act of (Chapter 20:27)

Section 4 of the Act stipulates that everyone has a right to a clean environment, access to environmental information, to protect the environment for the benefit of present and future generations and to participate in the implementation of the promulgation of reasonable legislative, policy and other measures. The Act outline principles that apply to projects/activities that affect the environment including polluter pays principle, precautionary principle, intergenerational equity principle, proportionality principle, the principles of responsibility, participation, effectiveness and efficiency. Part IX Section 57 requires all activities for the proposed development not to cause water pollution and no toxic or noxious substance shall be disposed of into the aquatic environment in contravention of water pollution control standards. Part IX Section 73 of the Act also prohibits the discharge of hazardous substances, chemicals and materials or oil into the environment from the different activities. Section 63 prohibits any person to emit any substances which cause air pollution (that is, by the emission of substances in excess of a prescribed amount for a particular source). Section 64 prohibits emission of substances or energy which causes or is likely to cause air pollution without an emission license. Section 79 and 80 prohibits emission of noise and vibration into the environment from any source above the prescribed standards. Measures should be put in place to ensure everyone enjoys his/her right to the clean environment.  The environmental monitoring plan should be religiously implemented.

- Environmental Management (Effluent and Solid Waste Disposal) Regulations, SI 6/2007 requires all solid waste sites to have appropriate lining specific to the nature of environmental risk, whether agricultural, industrial, and domestic and or any type of solid waste. Generators of waste are required to prepare waste management plan every year and the Environmental Management Agency may set prevention targets for each generator of waste. Waste bins shall be provided to ensure the environment is kept clean. Waste shall be disposed at a site designated by Matobo Rural District Council.

- Environmental Management (Hazardous Waste Management) Regulations, SI 10 of 2007. The regulation prohibits generation, storage, use, recycling, treatment, transportations and disposal of hazardous waste without a licence. It also prohibits disposal of hazardous waste at any other place except in a licensed hazardous waste disposal site or landfill. The generator of hazardous waste to prepare a waste management plan as per section 13 requirements. Further, it stipulates that hazardous wastes and waste oils shall be collected, stored, transported and treated separately from other wastes and the two shall not be mixed or blended together. Designated waste bins shall be provided to ensure the environment is kept clean. Waste shall be disposed at a site designated by Matobo Rural District Council.

- Environmental Management (Atmospheric Pollution Control) Regulations, SI 72 of 2009. The regulation provides for prevention of air pollution from dust, vehicles air emissions, uncontrolled burning and from fuel burning appliances, monitoring and recording air emissions as well as licensing these discharges to air. The regulations encourage suppression of dust through water sprinkling and other means. It prohibits the uncontrolled burning of used tyres, and other combustible material that pollute the air. Emissions into the atmosphere are likely in the construction, operation and decommissioning phase mainly fugitive dust and vehicle exhaust emissions. The regulation prohibits the use of vehicles which emit emissions that are above the standards prescribes by the regulation. The regulation also prohibits the owner or occupier of land or premises on which there is a disturbed surface area to cause or permit fugitive dust to be emitted into the atmosphere as a result of the active operation or from the disturbed surface area in excess of the amount prescribed by the law. Baseline ambient air quality measurement was done. Baseline ambient air quality measurement was done.

- Environmental Management (Prohibition and Control of Ozone Depleting Substances, Greenhouse Gases, Ozone Depleting Substances and Greenhouse Gases Dependent Equipment) Regulations, SI 131 of 2016. The regulations provide for the control and prohibition of ozone depleting substances, greenhouse gases, ozone depleting substances and greenhouse gases dependent equipment. The containers carrying such substances and equipment shall be clearly labelled and packaged to national and international standards. It also prohibits the unlicensed importation of ozone depleting substances and greenhouse gases and equipment. Additionally, it controls the repair, service, and handling, installation, decommissioning and destroying of equipment.

- Environmental Management (Control of Hazardous Substances) (General) Regulations, S.I. 268 of 2018. The regulations regulate labelling, packaging, sale and handling, special requirements for employers, use of pesticides, accidental spillages, import, manufacturing, transportation, storage, use and manufacturing of hazardous substances. The regulations require the licensing of commercial use of pesticides, sell, import, manufacturing, transportation, storage, use and manufacturing of hazardous substances. The employees who handle hazardous substances should be trained on the products’ hazards and provided with adequate personal protective equipment/clothing. Contractors and farmers shall be trained on hazardous substances management to ensure proper storage, handling, use and disposal of hazardous substances.

- Environmental Management (EIA and Ecosystems Protection) Regulations, SI 7 of 2007. The regulation prohibits the destroying of vegetation and introducing exotic plant species. The projects that require an Environmental Impact Assessment as specified in EMA CAP 20:27, shall be done by Environmental Management Agency registered Environmental and Social Impact Assessment consultant and a licence valid shall be issued after submission to Environmental Management Agency.

- The EIA certificate should be kept and shown to the agency upon request.

- Quarterly, the developer shall submit an environmental monitoring report to the agency.

- No person shall, without a license issued by EMA reclaim or drain, drill a tunnel, introduce any exotic animal or plant species, cultivate or license the cultivation of, or destroy any natural vegetation on, or dig up, break up, remove or alter in any way the soil or surface of: wetlands or, land within 30 meters of the high flood level of any body of water conserved in artificially constructed water storage work on a public stream, or, bed, banks or course of any river or stream.

Section 16 of the regulation ensures fire preventive measures, regulates burning of vegetation, fire extinguishing and investigation.

This environmental and social management plan was developed as per the requirement of the statutory instrument.

- Public Health Act (Chapter 15:17)

The Act prohibits and regulates bathing, washing, erection of structures and depositing of harmful substances for preventing the pollution so as to endanger health of any supply of water to the public. Section 67 requires the owner of any water works to allow the Chief Health Officer or any medical officer of any local authority to enter any water works or gathering ground and inspect and examine any sources of water supply or any such water works, and take such sample of water as he may deem fit. The Act also regulates activities detrimental to health. The owners of the premises responsible for environmental nuisance such as noise and odours, at levels that can affect human health are liable for prosecution under this Act. All the structures to be erected were approved by the council.

- Public health (COVID-19 Prevention, containment and treatment) Regulations, 2020. Regulates the implementation of measures to prevent, contain of COVID -19. The regulation controls gatherings, testing of COVID-19, restriction of movement, burial of dead bodies and disinfection of areas. The contractors will be temperature screened every day before work commences. Sanitisers and mask will be provided to the employees.

- Zimbabwe National Water Authority Act (Chapter 20:25)

The functions of this authority include the formulation of national polices and standards for water quality and pollution control and environmental protection.

- Rural District Councils Act (Chapter 29:13)

Section 73 prohibits the accumulating, dumping, depositing, abandoning or dropping on or in any road, street, sidewalk, drain, land, premises or place of rubbish or waste material of any description including, without derogation from the generality of the foregoing, machinery or vehicles or parts thereof. Provides for the provision and use of water subject to the Water Act Cap. 20:24. Owners and occupiers of land may be charged for services made available where any stand, lot, premises or other area, whether with or without improvements, within the council area is or could reasonably be connected to any sewer, water main or electricity main of the council. The developer is also required to provide means for the control, extinguishing and prevention of bush fires.

- Water Act (Chapter 20:24)

The Act prohibits the abstraction of water without a permit and it stipulates that the holder of an abstraction permit shall install a measuring device of the amount of water abstracted. Sections 46 and 68 of the Act also govern the construction work activities which interfere with river banks, bed, or course of a public stream; or any marshes, springs, swamps, or veils forming the source, or found along the course, of a public stream. A permit shall be applied for to the Catchment council in order to conduct such operations.

- Water (Permits) Regulations, SI 206 of 2001

The regulation provides for the control of abstraction and storage of water.

- Forest Act (Chapter 19:05)

Regulates the burning and destroying of vegetation by any person. It also regulates the provision of fireguards on common boundaries. The Forest Act regulates activities which include protection of vegetation and forests from activities, burning of vegetation. Provision and maintenance of fireguards on common boundaries.

- Traditional Leaders Act (Chapter 29:17)

The Act regulates the appointment and functions of traditional leaders (Chiefs, Headman, Village head) who also have the responsibility to preventing any unauthorised settlement or use of any land and ensuring that the land and its natural resources are used and exploited in terms of the law. The proponent shall consult the traditional leaders (Chiefs, Headmen and Village heads) of the area where he wants to develop.  These locals have a wide range of powers in the field of local administration regarding among other things, grazing, allocation of communal land and communal land use, irrigation and use of natural resources.

- Parks and Wildlife Act (Chapter 20:24)

The Act regulates the management and exploitation of wildlife resources that include indigenous plants, wild animals, fish, etc.

- Labour Act (Chapter 28:01)

The Act promotes fair labour standards by encouraging the participation of employees in decisions affecting the interests in the workplace by giving effects to the fundamental rights of employees. This is mainly done to secure the just, effective and expeditions resolutions of disputes and unfair labour practices. It prohibits forced labour and protects workers from discrimination in a workplace.

- National Museums and Monument Act (Chapter 25:11)

The National Museums and Monuments of Zimbabwe Act is administered by the National Museums and Monuments of Zimbabwe, a parastatal whose responsibility is to provide the establishment of museums that have the mandate among other things to locate document, inventory and preserve all ancient historical monuments artefacts and other objects of historical, scientific, educational value or interest

- National Social Security Authority (Accident Prevention and Workers’ Compensation Scheme) Notice, 1990.

The regulation provides duties of employers, manufacturers, designers, suppliers, supervisors or health and safety representative, and workers in protecting the health and safety of employees.

- Meteorological Services Act (Chapter 13:21)

Part II of the Act provides for free the following which the community is set to benefit from: General weather outlook services for subsistence agricultural purposes, and for social purposes in connection with the provision of water resources and health services. Adverse weather and climate early warning services and advisories.

- Regional Town and Country Planning Act (Chapter 29:12)

The  Act to provide for the planning of regions, districts and local areas with the object of conserving and improving the physical environment and in particular promoting health, safety, order, amenity, convenience and general welfare, as well as efficiency and economy in the process of development and the improvement of communications; to authorize the making of regional plans, master plans and local plans, whether urban or rural; to provide for the protection of urban and rural amenities and the preservation of buildings and trees and generally to regulate the appearance of the townscape and landscape; to provide for the acquisition of land; to provide for the control over development, including use, of land and buildings; to regulate the subdivision and the consolidation of pieces of land; and to provide for matters incidental to or connected with the foregoing.

- Zimbabwe Energy Regulatory Act (Chapter 13:23).

Section 7 of the act stipulates that no person shall in relation to the declared energy source, engage in the generation, procurement, distribution, transportation, transmission and production of the energy source for gain or reward unless that person is licensed. Solar energy shall be used to power the irrigation system.

- Communal Land Act (20:04)

An Act to provide for the classification of land in Zimbabwe as Communal Land and for the alteration of such classification; to alter and regulate the occupation and use of Communal Land; and to provide for matters incidental to or connected with the foregoing.

- Pesticides Regulations, SI 144 of 2012

It regulates the use of pesticides in Zimbabwe. The users of pesticides shall use registered pesticides. It stipulates that pest operators shall be competent and registered with the Ministry of Lands, Agriculture, Fisheries, Water and Rural Resettlement.

## 4.4 UNDP’s Social and Environmental Standards Policy (2015)

This ESMP was developed based on considerations of the guidance of UNDP’s Social and Environmental Standards in order to achieve environmental and social benefits and minimize of any potential environmental and social risks. These Standards underpin UNDP’s commitment to mainstream social and environmental sustainability in its programs and projects to support sustainable development and are an integral component of UNDP’s quality assurance and risk management approach to programming. Through the SES, UNDP meets the requirements of the GCF’s Environmental and Social Safeguards Policy.

UNDP screens and reviews its activities to identify opportunities to advance the principles that are founded on human rights, gender equality and women’s empowerment and environmental sustainability and to identify potential risks that may require measures to avoid, minimize, and/or mitigate potential impacts. At the Project level, UNDP has developed seven (7) operational safeguards Standards which further support implementation of UNDP’s commitments to promote respect for human rights, gender equality, and environmental sustainability. UNDP’s Project-level Standards relate to the following areas:

- Standard 1: Biodiversity Conservation and Sustainable Natural Resource  Management

- Standard 2: Climate Change Mitigation and Adaptation

- Standard 3: Community Health, Safety and Working Conditions

- Standard 4: Cultural Heritage

- Standard 5: Displacement and Resettlement

- Standard 6: Indigenous Peoples

- Standard 7: Pollution Prevention and Resource Efficiency

The Standards set out specific requirements relating to different social and environmental issues. Application of the Standards is determined during UNDP’s social and environmental screening and categorization process. Where it is determined that a Project may present certain risks and/or impacts, requirements of the relevant Standard(s) are triggered.

The screening carried out during project development indicate human rights principle and (5) of the project-level social and environmental standards have been triggered across the project components. This programming principle and the operational standards are described in Table 5.

Table 5: Operational standards triggered under the Project

| Principle/ Project-Level Standard | Safeguard Details | Applicable to the Project |  |
| --- | --- | --- | --- |
|  |  | Yes/No | Basic Details |
| Principle 1. Human Rights | UNDP recognizes the centrality of human rights to sustainable development | Yes | There is a risk that duty-bearers (e.g. government agencies) may not have the capacity to meet their obligations in the project |
| OS 1 | Biodiversity Conservation and Sustainable Natural Resource Management | Yes | This standard has been triggered because the project may generate potential biodiversity impacts through habitat loss or hydrological changes. Further biodiversity impacts may be experienced during the operational phase of the project due to soil and water pollution |
| OS2 | Climate Change Mitigation and Adaptation | Yes | This standard has been triggered because the Project will involve activities that generate greenhouse gas emissions such as vehicular transport. GhG emissions can also be released during handling of crop residues. |
| OS 3 | Community Health, Safety and Working Conditions | Yes | This standard has been triggered because the Project will involve construction of civil works which may pose a threat to human and animal life during the construction and operational phase. There is also potential risk of child labour and poor working conditions including occupational health and safety risks during the construction phase of the project. |
| OS 4 | Cultural Heritage | Yes | This standard has been triggered because, tangible cultural heritage may be unexpectedly encountered during the construction phase. As such, risks and impacts to tangible cultural heritage, and in particular, archaeological material, that may arise from Project activities need to be managed |
| OS 5 | Displacement and Resettlement | No | This standard has not been triggered because the Project will not result in physical or involuntary resettlement of persons during the implementation of Project activities. |
| OS 6 | Indigenous Peoples | No | This standard has not been triggered because there are no Indigenous Peoples situated within the project area of influence |
| OS 7 | Pollution Prevention and Resource Efficiency | Yes | This standard has been triggered because the Project will result in the use of chemicals e.g. (petroleum based fuels, fertilizers and pesticides) which may result in pollution of water, soils and air. The project will also result in water abstraction which necessitates its efficient use and management |

## 4.5 International Commitments

Zimbabwe is a signatory to a number of international and regional treaties which govern the implementation of the Project. The notable agreements and conventions relevant to the project include:

4.5.1 Basel Convention

The Basel convention is a global agreement, ratified by several member countries and the European Union, for addressing the problems and challenges posed by hazardous waste, specifically the ‘transboundary’ movement of hazardous waste and to address uncontrolled movement and dumping of such wastes (Basel Convention 1989). The overall goal of the Basel Convention is to protect human health and the environment against the adverse effects that may result from the generation, transboundary movements and management of hazardous and other wastes The key objectives of the convention are to minimize the generation of hazardous waste in terms of quantity and hazardousness, to dispose of the wastes as close to the source of generation as possible and to reduce the movement of hazardous waste. Categories of hazardous waste covered by the convention include: toxic, poisonous, explosive, corrosive, flammable, eco-toxic and infectious. In line with the principles of this Convention, a priority will be to deal with the waste as close to source as possible.

4.5.2 The Ramsar (1971) Convention

The RAMSAR (1971) Convention on Wetlands of International Importance is an inter-governmental treaty and provides a framework for national action and international cooperation for the conservation and wise use of wetlands, and to contribute to sustainable development worldwide. The proposed project should not result in significant disturbance of the shores of the Nkashe dam which depict seasonal wet areas.

4.5.3 The United Nations Framework Convention on Climate Change and the Kyoto Protocol

The United Nations Framework Convention on Climate Change (UNFCCC) is an international environmental treaty aimed at stabilizing greenhouse gas concentrations in the atmosphere at a level that would prevent dangerous anthropogenic interference with the climate system. The Convention therefore provides the basis for global action “to protect the climate system for present and future generations”. Zimbabwe ratified the Convention in 1992. The Kyoto Protocol is linked to the UNFCCC to prevent anthropogenic interference with the climate. The Protocol establishes legally binding commitments for the reduction of greenhouse gases and fluorocarbons. The revitalization project will need to understand its contributions to greenhouse gas emissions relative to that of the country and if relevant include related mitigation measures for on-going monitoring and assessment.

4.5.4 Convention on Biological Diversity

The Convention on Biological Diversity is the first global agreement on conservation and sustainable use of Biological Diversity. Its objectives are to conserve biodiversity, promote the sustainable use of bio-diversity components and to promote fair and equitable sharing of benefits arising from the use of resources. Zimbabwe ratified the Convention in 1994. The convention requires Zimbabwe to prepare a national biodiversity strategy and to ensure that it is mainstreamed into the planning and activities of all sectors that have an impact on biodiversity. In line with this provision, the contractor who is going to carry rehabilitation works should ensure that the project is planned, developed and operated in a manner that is in compliance with the recommendations of the National Biodiversity Strategies and Action Plans (NBSAP).

4.5.5 Convention to Combat Desertification

The objective of the United Nations Convention to Combat Desertification is to combat desertification and mitigate the effects of drought in countries experiencing serious drought and/or desertification through effective action at all levels and supported by international cooperation. The Convention was established in 1994 and ratified by Zimbabwe in 1997 and developed the Zimbabwe National Action Program in support of the convention. Irrigation projects are associated with salinization of the soil an environmental problem which leads to desertification. The proposed project should be planned, developed and operated in such a manner that promotes the use of organic fertilizers.

4.5.6 Vienna Convention on Protection of the Ozone Layer and Montreal Protocol

The Vienna Convention entered into force in 1980 and outlines responsibilities of states for protecting human health and the environment against the adverse effects of ozone depletion in the stratosphere. It established the framework under which the Montreal Protocol was negotiated. Parties to the Vienna Convention are required to take appropriate measures to protect human health and the environment against these effects. Zimbabwe ratified the Convention in 1992. Activities which result in the generation or distribution of ozone depleting substances associated at the site should be avoided.

4.5.7 Equator Principle

The Equator Principles were developed in 2003 to provide a set of guiding principles for determining, assessing and managing environmental and social risk in project financing. In committing to this the ESMP must address the following:

- An assessment of the baseline environmental and social conditions;

- Requirements under host country laws and regulations, applicable international treaties and agreements;

- Sustainable development and use of renewable natural resources;

- Protection of human health, cultural properties and biodiversity, including endangered species and sensitive ecosystems;

- Use of dangerous substances;

- Occupational health and safety;

- Fire prevention and life safety;

- Socioeconomic impacts;

- Pollution prevention and waste minimization, pollution controls (liquid effluents and air emissions) and solid and chemical waste management.

4.5.8 Convention on the Elimination of all forms of Discrimination against Women (1979)

The Convention defines discrimination against women as “…any distinction, exclusion or restriction made on the basis of sex which has the effect or purpose of impairing or nullifying the recognition, enjoyment or exercise by women, irrespective of their marital status, on a basis of equality of men and women, of human rights and fundamental freedoms in the political, economic, social, cultural, civil or any other field.” This project aims to support more women than men thus promote gender balance in the socio-economic aspect.

4.5.9 Proponent’s Responsibility with Regards to the Environment

In addition to complying with the various pieces of legislation as outlined above, this study recommends that the contractor develops an environmental monitoring and management program for the project life cycle. The proponent will also provide an Occupational, Safety, Health and Environmental policy that provides for the health and safety of persons at work in connection with the use of equipment and machinery; the protection of persons other than persons at work, against hazards to health and safety arising out during operation stage and ensure continuous improvement. This will ensure that the environment in and around the area is well conserved and measures will be put in place to reduce environmental contamination.

## 4.6 Conclusion

These are the legislative and international commitment instruments that shall be used to guide the proposed project in order to be environmentally sustainable throughout the project lifecycle. The contractor and project beneficiaries are encouraged to adhere to all the legislations throughout the project development phases.

## 4.7 Licenses/Permits required for the project

The project has to acquire the permits provided in Table 6.

Table 6: Licenses/Permits

| ITEM | LICENSE/PERMIT | STAGE | REGULATION |
| --- | --- | --- | --- |
| 1. | ESMP License | Pre-Construction | EMA |
| 2 | Water abstraction Permit | Operational phase | ZINWA |

# 5.0 ENVIRONMENTAL AND SOCIAL BASELINE

## 5.1 Introduction

Collection of baseline data is an essential aspect for conducting an environmental and social assessment process. The baseline data reflects the present status of the environment before commencement of any revitalization activity of the proposed project. The environmental and social baseline forms a benchmark for determination of the impacts of the developmental project. In this context, the impacts of the project will manifest in changes to the environmental and social characteristics from the pre-project baseline. More so, the baseline surveys depict the suitability and sensitivity of the proposed project area, a key step in impact prediction and recommendations for feasibility of the project. In the present Project, baseline surveys included the following facets of the environment: geology, hydrogeology, terrestrial and aquatic ecological assessments, soils, water quality, ambient air quality, archaeological, land use and socio-economic. Baseline surveys covered the proposed location for the project and the immediate environment where impacts are likely to be felt.

## 5.2 Climate

The Project lies in agro-ecological region V and this is characterized by low and erratic rainfall which averages between 450–650 mm/year and hot spells during the dry summer months. Precipitation falls mainly between November and March and the common type of precipitation received in the area is conventional influenced by the southern Frontier of the Inter-Tropical Convergence Zone (ITCZ) mainly in January. The temperature average is around 28ºC. Climate change has worsened the already extreme weather patterns in the district. Drought is an example of this phenomenon that has negatively affected the Project area. Prolonged droughts have dried up natural water points such as streams, rivers, and swampy areas. Table 7 and Table 8 show the average monthly climate data and the projected climate data for the year 2050 respectively.

Table 7: Average monthly climate data

| Month |  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Average | Totals |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min Temp | °C | 15.38 | 14.2 | 13.5 | 12.2 | 7.8 | 10.8 | 10 | 11.9 | 12.1 | 14.1 | 16.1 | 17.5 | 13 |  |
| Max Temp | °C | 28.08 | 28.3 | 27.7 | 26.1 | 24 | 22.3 | 22.8 | 24.4 | 27.4 | 29.7 | 27.4 | 26.6 | 26.2 |  |
| Humidity | % | 54.59 | 54.6 | 54.6 | 54.6 | 51.5 | 51.5 | 51.5 | 48.4 | 46.4 | 48.4 | 51.5 | 53.6 | 51.8 |  |
| Wind | m/s | 2.78 | 2.7 | 2.8 | 2.7 | 2.3 | 2.5 | 2.7 | 2.9 | 3.3 | 3.5 | 3.3 | 2.9 | 2.8 |  |
| Sun | hours | 6.65 | 6.5 | 7.1 | 7.9 | 8.3 | 8.4 | 8.6 | 9.2 | 9.3 | 8.7 | 7.3 | 6.1 | 7.8 |  |
| Rad | MJ/m²/ day | 20.52 | 19.8 | 19.5 | 18.3 | 16.4 | 15.4 | 16.2 | 19 | 21.7 | 22.7 | 21.4 | 19.7 | 19.2 |  |
| Eto | mm/day | 5.5 | 5.3 | 5.9 | 4.8 | 3.9 | 3.5 | 3.8 | 4.8 | 7.1 | 8.1 | 7 | 6.4 | 5.5 | 1,853 |
| Rain | mm/ month | 144.23 | 110.4 | 60 | 28.6 | 6.7 | 4.9 | 0 | 0 | 2.2 | 22.3 | 81.8 | 132.5 | 49.5 | 594 |
| Eff rain | mm/ month | 104 | 98 | 49 | 21 | 0 | 0 | 0 | 0 | 0 | 15 | 21 | 95 | 38 | 441 |
| Ieff | mm/day | 2.03 | 2.04 | 4.23 | 4.14 | 3.93 | 3.5 | 3.82 | 4.84 | 7.1 | 7.6 | 6.29 | 3.22 | 4.25 | 57 |

Table 8: Projected climate data for the year 2050 Climate Data

| Month |  | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min Temp | °C | 16.38 | 15.2 | 14.5 | 13.2 | 8.8 | 11.8 | 11.0 | 12.9 | 13.1 | 15.1 | 17.1 | 18.5 | 14.0 |
| Max Temp | °C | 29.08 | 29.3 | 28.7 | 27.1 | 25.0 | 23.3 | 23.8 | 25.4 | 28.4 | 30.7 | 28.4 | 27.6 | 27.2 |
| Humidity | % | 54.59 | 54.6 | 54.6 | 54.6 | 51.5 | 51.5 | 51.5 | 48.4 | 46.4 | 48.4 | 51.5 | 53.6 | 51.8 |
| Wind | m/s | 2.78 | 2.7 | 2.8 | 2.7 | 2.3 | 2.5 | 2.7 | 2.9 | 3.3 | 3.5 | 3.3 | 2.9 | 2.8 |
| Sun | hours | 6.65 | 6.5 | 7.1 | 7.9 | 8.3 | 8.4 | 8.6 | 9.2 | 9.3 | 8.7 | 7.3 | 6.1 | 7.8 |
| Rad | MJ/m²/day | 20.52 | 19.8 | 19.5 | 18.3 | 16.4 | 15.4 | 16.2 | 19.0 | 21.7 | 22.7 | 21.4 | 19.7 | 19.2 |
| Eto | mm/day | 5.83 | 5.6 | 6.2 | 5.1 | 4.2 | 3.7 | 4.0 | 5.1 | 7.5 | 8.6 | 7.4 | 6.8 | 5.8 |
| Rain | mm/month | 115.38 | 88.3 | 48.0 | 22.9 | 5.3 | 3.9 | 0.0 | 0.0 | 1.8 | 17.8 | 65.4 | 106.0 | 39.6 |
| Eff rain | mm/month | 93.60 | 88.20 | 44.10 | 18.90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 13.50 | 18.90 | 85.50 | 38.00 |
| Ieff | mm/day | 2.71 | 2.69 | 4.74 | 4.50 | 4.16 | 3.71 | 4.04 | 5.13 | 7.53 | 8.14 | 6.78 | 3.92 | 4.58 |

## 5.3 Topography

The topography in Masholomoshe and Masiyepambili irrigation schemes is characterized by a gently undulating slope. Masiyepambili is 13m above Shashane river.

## 5.4 Hydrology

Masholomoshe and Masiyepambili irrigation schemes fall under Shashe sub catchment of Umzingwane catchment. Mzingwane catchment occupies a land area of 63 000 km2. Mzingwane Catchment is divided into 4 sub-catchments namely Lower Mzingwane, Upper Mzingwane, Mwenezi and Shashe. Figure 6 shows the major catchments of Zimbabwe. Water source for Masholomoshe irrigation is Nkashe Dam situated north of the project site approximately 2 kilometres upstream of the irrigation scheme. Table 8 show water quality data for Nkashe dam. The dam was constructed by Rhodesian Government in 1964.

Figure 6: Catchments of Zimbabwe

The main drainage system constitutes seven main rivers namely Ramaquabane, Ingqwesi, Sansukwe, Simukwe, Shashani, Tuli and Umzingwane (Figure 7). These rivers flow southwards towards the Shashi and Limpopo rivers and are prone to desiccation especially in the long dry season (April to October). During the dry season, the pools and sandy beds become the alternative source of water for both humans and livestock. During the hottest part of the year (September - mid-November) these regularly cease to be adequate. Figure 8 illustrates the drainage system in Gwanda district including the Masholomoshe irrigation scheme area.

Figure 7: Mzingwanwe catchment showing main rivers, dams and towns

Figure 8: Drainage system in Gwanda district including Masholomoshe irrigation scheme area

Like Masholomoshe irrigation scheme, Masiyepambili irrigation scheme also falls in Shashe sub-catchment. In the Shashe sub-catchment lies Shashane River which is the source of irrigation water for Masiyepambili irrigation scheme. Shashane river flows southwards towards the Shashi River and is prone to desiccation especially in the long dry season (April to October). Figure 9 shows the drainage pattern in Matobo district including the Masiyepambili irrigation area. Figure 10 shows the heavily silted Shashane river. Table 9 provides water quality analysis results from Shashane river and Nkashe dam.

Figure 9 Drainage system in Matobo district including Shashane river

Figure 10 Heavy sediment loads in Shashane river

Table 9: Water Analysis Data

| Parameter | Shashani River | Nkashe Dam | FAO Standards |
| --- | --- | --- | --- |
| Ph | 6.8 | 6.7 | 6.4 to 8.4 |
| Conductivity (μS/cm) | 185 | 193.00 | Max 700 |
| TDS (mg/l) | 129.50 | 135.10 | Max 450 |
| Total Hardness (mg/l) | 45.80 | 19.66 | Max 75 |
| Total Alkalinity (mg/l) | 60.00 | 90.00 | N.A |
| Calcium (mg/l) | 12.20 | 4.90 | N.A |
| Copper (mg/l) | 0.01 | < 0.01 | Max 0.2 |
| Iron (mg/l) | 0.93 | 5.15 | Max 0.1 |
| Pottasium (mg/l) | 10.8 | 2.80 | N.A |
| Magnesium (mg/l) | 3.72 | 1.80 | N.A |
| Manganese (mg/l) | 0.06 | 0.23 | Max 0.1 |
| Sodium (mg/l) | 11.13 | 2.45 | Max 70 |
| Zinc (mg/l) | 0.22 | 0.02 | Max 2 |
| Nitrate (mg/l) | 40.29 | 1.63 | Max 5 |
| Sulphate (mg/l) | 2.56 | 61.34 | N.A |
| Chloride (mg/l) | 6.00 | 5.00 | Max 100 |
| Turbidity (ntu) | 0.20 | 11.00 | N.A |
| Phosphate (mg/l) | 0.78 | 6.91 | N.A |
| S.A.R. | 0.72 | 0.24 | Max 3 |

## 5.5 Ecological Assessment

### 5.5.1 Vegetation

The vegetation in the project area is generally categorized in the Jesse mixed woodland. Vegetation growth in this area has been retarded by poor soil quality, erratic rains, and harsh temperature. The common plant species include: fabaceae spp. family, Combretum family, Monticola spp. Family and Colophospernum. The present form of vegetation inside the disused scheme is dominated by rejuvenating Acacia karoo. Plate 7 shows the rejuvinating acacia bushes. Immediate locations outside the irrigation scheme boundary and the area proposed for extension is dominated by Colophospernum mopane, Camiphora mallotii, and Azora reticulata, Cassia abreviata, Combretum collinum and a wide range of seasonal grasses which include Heteropogon contortus, Aristida spp., Digitaris eriantha, Sporobolus stapfianus, Sporobolus ioclados and Chloris gayana. Table 8 show the flora registerd at Masiyephambili irrigation scheme site

Plate 7: Rejuvinating acacia bushes

Table 10:Flora registerd at Masiyephambili irrigation scheme site

| Botanical Name | IUCN Classification |
| --- | --- |
| Erythrophleum africanum | Least Concern |
| Piliostigma thornningii | Least Concern |
| Combretum collinum | Least Concern |
| Lonchocarpus Capassa | No data available |
| Grewia monticola | Least Concern |
| Acacia Karroo | Least Concern |
| Azora reticulate | No data available |
| Commiphora marloti | No data available |
| Cassia Abbreviata | Least Concern |
| Colophospernum mopane | No data available |
| Dichrostachys cinerea | Least Concern |
| Lonchocarpus Capassa | Least Concern |
| Combretum imberbe | Least Concern |
| Ziziphus mucronata | Least Concern |
| Sclerocarya birrea | No data available |

**Least Concern (LC) is a category containing species that are pervasive and abundant after careful assessment.

### 5.5.2 Fauna

Faunal assessments were done through use of key informants, search for spoors, dung, nests, visual sightings in areas used for vegetation assessment and around the proposed project site. Animal species reported in the area include Common diuker, Scub hare, night ape, Impala, warthogs, vervet monkey and Chacma Baboons. Dung recorded was from cattle and goats. Birds were identified based on expert knowledge of gained over a number of years such as, vocalizations, presence of eggs, nests and feathers and physical observations from a short distance. Key informants noted the presence of snakes such as black mamba, seen around the project area and other repitiles such as chameleon, tree agamas. However, small mammals like rodents had visible denning signs. Insects identified were Lucastana pardalina (brown locust), Sceliphron spirifex (zingizi) and Apis melifera (honeybee) and insectivores, frugivores and piscivores were dominating. Very few reptiles are known to exist in the area, and these include, twig snake, black mamba, chameleon, tree agamas, and puff adder snake. Table 11 provides the common bird species spotted in the project area.

Table 11:Species of birds registered in the Project area

| Scientific name | IUCN Classification |
| --- | --- |
| Quelea quelea | Least Concern |
| Corythaixoides concolor | Least Concern |
| Plooceus velutus | No data available |
| Streptopelia senegalensis | Least Concern |
| Uraeginthus angolensis | Least Concern |
| Dicrunus adsimilis | Least Concern |
| Pternitis swaisonni | Least Concern |
| Oena capensis | Least Concern |
| Camaraptera brevicaudata | Least Concern |

### 5.5.3 Entomology

Above ground (crawling and aerial macro-invertebrates) as well as below ground macro-invertebrates were sampled. The macro-invertebrates were sampled in the irrigation field and surrounding area. Crawling insects were sampled using pitfall traps. The pitfall traps were set in randomly marked points in a 5m x 4m grid fashion. Traps were filled with soapy water in order to avoid escape of trapped macro-invertebrates and were checked after 24 hrs. The captured organisms were then placed in vials filled with 70% ethanol. For below ground macro-invertebrates soil monoliths were collected sifted in trays for macro-invertebrates. The remaining macro-invertebrates were then flushed out using berlese funnels. Aerial insects were captured using sweep nets and the contents were emptied into glass bottles and taken to the National museum for identification. Identified invertebrates were bees, butterfly, locusts, and aphids.

## 5.6 Soil Assessment

A soil survey was conducted concurrently with vegetation assessments. Using hand soil auger, soil samples were collected from points where vegetation assessments were being done covering block A, B and C. Three samples were taken at each point and were mixed and thoroughly homogenised to make a composite sample. About 500g of the composite sample was subsequently sealed in a well-labelled paper bag and sent to lab for physicochemical analyses. Parameters targeted for lab analysis depended on those likely to be impacted by the proposed project e.g. nitrogen, pH and phosphorus. Table 11 provides the results of soil chemistry. Existing excavation to a greater extent were used for soil profile descriptions. The project area is characterized by Kaolinitic soils from Fersiallitic group. The soils are mainly greyish brown to reddish brown clay to sandy – loam and the predominant loam soils have the ability to store organic matter. The soils are suitable for most crops using any irrigation systems. The pH is ideal for most crops. Exchangeable sodium percentage is very low making the soils most ideal for irrigation without the risk of soil degradation and salinity build up. Table 12 and 13 shows soil analysis results and soil profile.

Table 12: Soil chemistry in the project area

| Parameter | 0-25 cm | 25 – 55cm | 55 – 90cm |
| --- | --- | --- | --- |
| Clay % | 8 | 17 | 16 |
| Silt % | 5 | 12 | 13 |
| Fine sand % | 41 | 33 | 28 |
| Medium sand % | 33 | 25 | 26 |
| Coarse sand % | 13 | 13 | 17 |
| Gravel % | * | * | * |
| pH (CaCl2) | 6.1 | 6.0 | 6.1 |
| Carbonates % | * | * | * |
| Ex Ca (me %) | 5.5 | 7.0 | 9.5 |
| Ex Mg (me %) | 3.3 | 3.9 | 5.3 |
| Ex Na (me %) | 0.11 | 0.16 | 0.12 |
| Ex K (me %) | 0.30 | 0.13 | 0.09 |
| TEB (me %) | 2.7 | 9.6 | 6.2 |
| CEC (me %) | 2.7 | 9.6 | 6.2 |
| base sat % | 100 | 100 | 100 |
| E/C | 32.3 | 57.9 | 38.9 |
| S/C | 32.3 | 57.9 | 38.9 |
| ESP | 4.0 | 1.7 | 2.0 |
| EKP | 11.0 | 1,3 | 1.5 |

Table 13: Soil Profile

| Depth(cm) | Description | Picture |
| --- | --- | --- |
| 0-25 cm | Reddish brown, 5YR 5/4m; loamy medium grained sandy (mLS); dry soft, very friable moist, slightly sticky, and non-plastic wet consistence; weakly developed fine sub- angular blocky structure; rapid permeability and very well drained; few very fine and fine roots; few very fine and fine interstitial pores; gradual smooth transition to: |  |
| 25 - 55 cm | Yellowish red ,5YR 5/6m; dark yellowish brown, 10YR4/4m sandy loam(SaL);,friable moist, slightly sticky, and plastic wet consistence; weakly developed medium sub-angular blocky structure; good permeability and well drained; few fine and very fine roots; few very fine and fine interstitial pores; gradual smooth transition to: |  |
| 55 - 90 cm | Yellowish red ,5YR 5/6m; dark yellowish brown, 10YR4/4m sandy loam(SaL);friable moist, slightly sticky, and plastic wet consistence; weakly developed medium sub-angular blocky structure; good permeability and well drained; few fine and very fine roots; few very fine and fine interstitial pores; gradual smooth transition to: |  |

## 5.7 Air Quality

The irrigation schemes are situated in a communal area and there are no major human activities that generate air pollutants. Fugitive dust emissions are caused by traffic entrainment of dust from unpaved roads and agricultural operations. The air emissions were measured on real time and were verified for three consecutive days. All the measured pollutants fall within the stipulated limits  provided by the World Health Organisation (WHO) 2021 ambient air quality standards as shown in Table 14.

Table 14: Air Quality Results

| Parameter | Crop field | Pump and solar area | WHO 24hrs | WHO annual |
| --- | --- | --- | --- | --- |
| NO2 (ug/m3) | 0.1 | 0.3 | 25 | 10 |
| CO (mg/m3) | 0.0 | 0.0 | 4 | - |
| SO2 (ug/m3 ) | 0.0 | 0.0 | 40 | - |
| PM10 (ug/m3) | 0.6 | 0.2 | 45 | 15 |
| PM2.5 (ug/m3) | 0.1 | 0.1 | 15 | 5 |
| O3 (ug/m3) | 0.1 | 0.1 | 100 | 60 |

## 5.8 Noise

A noise survey was conducted using a sound level meter: Model: ET-953. Due to lack of specific environmental noise limits in Zimbabwe, the results for ambient noise were compared against the International Finance Corporation Environmental Health and Safety Guidelines stipulated limit of 70 dBs. The results are within the limits as shown in Table 15.

Table 15: Ambient noise levels at different sites

| Sampling station | Result (dB) |
| --- | --- |
| Dam wall | 54 |
| Pump and solar area | 53 |
| Between dam and field | 49 |
| Field point 1 | 48 |
| Field point 2 | 48 |

## 5.9 Archaeology and Cultural Heritage

An archaeological survey was conducted with the Project area and no archaeological or cultural materials were identified. However, in the event that any suspected archaeological and cultural material is encountered during the site development activities, UNDP SES standard 4 Cultural heritage which triggers chance find procedures should be activated. Appendix 6 outlines the chance find procedure that must be activated in case of a chance find.

## 5.10 Baseline Socio-economic setting within the project area

Population and Demography

Demographic data described here include population, age distribution, ethnic makeup as well as literacy and education rates, provided both on a district level to give a background, as well as specific to each of the two schemes . The relevant information applicable to each site was extracted from the primary and secondary data sources and assessed within the context of project life cycle.

The population of Gwanda Rural District where Masholomoshe irrigation scheme is situated was estimated to be 116000 as per 2012 census. The Masholomoshe Irrigation scheme cumulative beneficiaries are estimated to be 850 direct beneficiaries represented by the 127 heads of household. The average number of persons per household is 6.8 persons. The average age of irrigation scheme beneficiary members disaggregated by gender is 63 years for women and 59 years for men. The general beneficiary membership at Masholomoshe irrigation scheme comprises an elderly age range averaging 57.6 years. Figure 11 shows the scheme member distribution by age.

Figure 11: Masholomoshe Irrigation scheme distribution of cumulative beneficiaries

Another key observation is that the males across both age groups have a higher proportion of irrigation scheme membership than females. Figure 12 shows the distribution of irrigation scheme representation by age group (N=127). The findings during community engagement were that because initial membership was based on voluntary labour during trenching and land clearing, this became a gender assigned role that men could create more time to participate in the activity and are more inclined to take on heavy duties such as trenching. A lot of the irrigation scheme members are taking care of minors left in their custody by older children who have migrated to South Africa and Botswana for work opportunities.

Figure 12 Distribution of irrigation scheme representation by age group (N=127)

The general population of Matobo Rural District stood at 93421 after the 2012 census. The Food Atlas Survey of 2016 indicated that the total population in ward 02 was 5993. The total number of currently registered beneficiaries is 30, and 26 members participated and were represented in the survey; the irrigation membership is made up of 12 females and 18 males. The Masiyepambili Irrigation scheme cumulative beneficiaries can be estimated to be in the range 160 – 200 direct dependents, with total 149 persons represented by the 26 heads of household and irrigation scheme members interviewed. Outcomes of the HH survey indicate that the average number of persons per household is 5 persons. The average age of irrigation scheme beneficiary members disaggregated by gender is 54 years for women and 60 years for men. The general beneficiary membership at Masiyepambili comprises an elderly age range because in the period that Masiyepambili was established, household heads only were registered as members and everyone under that household head would then become a direct beneficiary. Figure 13 show the age distribution at Masiyepambili Irrigation scheme.

Figure 13:Masiyepambili Irrigation scheme distribution of cumulative beneficiaries

Vulnerable groups

Eleven persons were identified in the Project area living with some form of disability. These are disaggregated as 7 under the age of 18 and 4 above the age of 19. Others present in this beneficiary community are orphans and widows, as well as the elderly. Revitalization of the irrigation schemes will assist them to have better opportunities in their lives in terms of food security as they are predominantly subsistence farmers.

Prospective water users

The water from Nkashe dam and Shashane river is used for non-consumptive use by community members. The water is mainly used for crop irrigation, livestock drinking, bathing and clothes washing. The quantity of water consumed by livestock depends on environmental factors (e.g. ambient temperature and humidity), drinking water temperature, water quality, ration composition (nature of food and dry matter content), feed intake, size of the animal and individual variation between animals. Table 16 shows the current water users and use at Nkashe Dam. Table 17 show the average livestock drinking water requirements in the Project area.

Table 16: Prospective Nkashe dam water users

| Water User | Water Uses |
| --- | --- |
| Non irrigation scheme members | Bathing, washing |
| Irrigation scheme | Crops irrigation |
| Domestic animals | Drinking |

Table 17: Average livestock drinking water requirements

| Animal | Estimated number in the area | Daily water requirement/animal | Monthly water requirement/animal | Total required/month (approximate) |
| --- | --- | --- | --- | --- |
| Cattle | >2100 | 38 litres | 1178 litres | 24738 hectolitres |
| Goats | >3200 | 5 litres | 155 litres | 4960 hectolitres |
| Donkeys | >200 | 23 litres | 713 litres | 1460 hectolitres |

Land tenure system

Zimbabwe has four main systems of land tenure which are freehold land that is private, state land, communal and leasehold resettlement systems. The land allocated for the two irrigation schemes is communal land which is governed by the Communal Land Act (Chapter 20:04). This means the land allocated for irrigation is vested in the State President who has powers to permit its occupation and use in accordance with the Act. According to this Act, the Irrigation Scheme beneficiaries have land use rights. The Gwanda and Matobo Rural District Councils have the power to allocate the land on behalf of the State as per Act. Hence, everything is above the board since the beneficiaries within the two schemes were allocated the land by the Rural District Councils for the purpose of irrigation development.

The landholders have voluntary agreed to cooperate with the relevant stakeholders including the Ministry of Agriculture of Lands, Agriculture, Fisheries, Water and Rural Development to develop the proposed scheme (refer to Appendix 7 for the voluntary land use agreements ). No individual properties or assets will be lost or destroyed during the process of developing the two schemes. Once the schemes are fully developed, all the landholders will benefit from the scheme and each beneficiary will have a plot of 0.3 in size under irrigation. In Masiyepambili irrigation, there is potential to increase the number of direct project beneficiaries. The process of allocating some additional plots to individuals in the scheme could present some disputes if the process is not participatory and inclusive, and it is therefore recommended that local leadership must be involved and that the process be inclusive, gender and disability sensitive & culturally appropriate.

The general land ownership is through inheritance under customary law, and that each person allocated a piece of land can utilise it across generations (in the event of death a willing family member is given precedence in taking over). Table 18 shows the current land allocation status at the two irrigation schemes.

Table 18: Current irrigation land allocation status

| Scheme Name | Area (Ha) | Number of female plot holders | Number of male plot holders | Total Number of plot holders | Plots Size (Ha) |
| --- | --- | --- | --- | --- | --- |
| Masholomoshe | 38.8 | 62 | 69 | 131 | 0.3 |
| Masiyepambili | 8 | 12 | 18 | 30 | 0.267 |

Irrigation Scheme management and their capabilities

Administrative structures at the schemes

An Irrigation Management Committee (IMC) for Masholomoshe irrigation scheme was formulated based on nominations and followed by a voting process. The Management Committee (IMC) has a constitution which guides the irrigation scheme operations. Currently, the scheme has 131 beneficiaries each with a land allocation of 0.3 hectares. All members contribute an amount equivalent to or more than USD 1.00 towards operations, repair and maintenance of irrigation scheme infrastructure. The Irrigation Management Committee (IMC) is made up of 7 females and 2 males comprising of a Chairperson, Secretary, Vice Chairperson, Treasurer, Manager, Caretaker and two committee members.

On the other hand, Masiyepambili scheme was managed by a nominated Irrigation Management Committee (IMC) but the committee collapsed following the collapse of the irrigation scheme. Failure of the Masiyepambili irrigation scheme administrative and other internal structures to buffer against and cope with hyperinflation (2007-2008) resulted in the scheme activities being suspended and then stopped all together as beneficiaries were struggling to meet their own personal financial obligations at household level.

Education and Training

Literacy rates

There are well distributed schools around Masholomoshe scheme with students walking a distance of 1 – 5 kilometers in a one-way trip to get to school. The schools include Magedleni Primary, Matshetsheni Primary, Matshiya Primary, Masholomoshe Primary, Nkashe Primary and Secondary. Some of the beneficiary member’s children and dependents attend these local schools. However, there are a few schools in around Masiyepambili with students walking as far as 5-10 kilometres in a one-way trip to get to school. The schools found near and around the scheme are Emhlonhlweni Primary School, Sinkamaswe Primary School, Tudi Primary school, and Zamanyoni Secondary School. Some of the beneficiary member’s children and dependents of school going age attend these local schools.

Education level among irrigation scheme beneficiaries

Education levels among the irrigation scheme members is low, over 60% said they struggle to read and write and would prefer to have training materials and training sessions administered in vernacular with visual aids such as pictures for demonstrations.

Education among dependants of school going age

The majority have some varying levels of basic education ranging from primary to secondary, with less than 15% of the interviewed population having attended a centre of higher learning and completed a diploma, national certificate or degree. All interviewed beneficiaries expressed a willingness to learn and attend any farming or other training courses that help them improve yield, fight pests and disease. Most of the beneficiaries (50%+) expressed that illustrative and materials in vernacular are easier for them to follow even on their own, so training materials must have pictures and be in Ndebele.

Over 75% of the beneficiaries interviewed expressed that they are struggled to get their children to attend and stay in school because the little income they generate goes towards the collective family survival. However, when the irrigation scheme was operational, issues such as school fees and household needs were addressed timeously and without fail as priorities. Now the priority has shifted to basic survival. Figure 14 provides Masholomoshe irrigation scheme school attendance among children of school going age (N=137)

Figure 14: Masholomoshe attendance among children of school going age

In 2006 the Masholomoshe IMC received training in Irrigation Management. Trainings are often individual and there is a need to build confidence for those who have been trained to train others and share information received or lessons learnt. All trainings are free, and members of the irrigation scheme are permitted to participate. More training was received in December 2011 and January 2022 by an Agritex extension officer Mrs Ngwenya, all beneficiaries were trained. We receive training in the growing and care for each crop as we follow the crop calendar

Access to agriculture extension services

Extension officers are in charge of assisting farmer efforts in both schemes. These were observed to be well known and active among the beneficiary members. This is an additional asset for the programme as the community will have guaranteed access to Agritex Extension services.

Access to labour

Among the irrigation scheme beneficiaries, it is a common practice to use members of the household of varying ages from 5 years and above to provide unpaid labour at the small holder plot where mostly subsistence farming is done for household consumption, community garden or even irrigation scheme (particularly on school holidays). It was also observed that the elderly have a higher inclination to hire seasonal labour to work on the smallholder plots, while work in the community garden and on the irrigation scheme is reserved for family members or household members. For work in communal plots, there are few incidents of hiring children in the age range 10 to 14 years. In retrospect, while a household would not hire a person in the age range 10 to 14 years of age, these children if they are there in the household will do field work like tilling and ploughing especially on school holidays as unpaid labour in their own home. Figure 15 shows the representation of labour for farming. In the following illustration yes represents a response above 60% of population asked, while No represents 40% outcome and sometimes, was often on the fence (neither strongly agree nor disagree).

Figure 15: Representation of source of labour for farming

Economic Activities

The local economy in Gwanda and Matobo districts is dominated by agriculture, a few people are into formal trade of goods and services. The most common type of trade among the farming community is barter trade, most prevalent in the dry season. The irrigation scheme beneficiaries interviewed all listed agriculture (irrigated) as their primary source of income, with surplus money going towards household needs such as education and healthcare. Cattle ranching also contribute to the economy. The land is also less fertile than other provinces, as commercial crops cannot be grown and rural farmers usually cannot produce enough rain fed maize to feed their families. There is a small tuckshop which is run by the irrigation scheme to supplement sales from harvests.

Household income among irrigation scheme members

Five general income streams were identified namely, collective and individual sale of surplus crops from irrigation scheme, general trade in goods and services, small holder farming production and sales (rain-fed), casual labour and sale of livestock or other related projects. The main source of income for over 89% of the irrigation scheme members was listed as the irrigation scheme. There appeared to be a number of beneficiaries who are also on pension, thus increasing their monthly income. A lot of households were unable to quantify their income as they said they do small cumulative daily sales. Figure 16 shows the estimated monthly household income among scheme beneficiaries.

Figure 16 Estimated monthly household income among scheme beneficiaries

Expenditure assets by irrigation scheme member households

The irrigation scheme members have expressed greater financial freedom to purchase additional assets that facilitate their improved livelihoods, in the areas of transport, communication and energy for cooking. The community generally favours transport means that can facilitate moving produce from the farm to the market or the main road, thus the investment in scotch carts. In addition, mobile phones are a big purchase across all age groups and genders because the phones are used for communication, for lighting and sometimes to charge other low voltage devices. Figure 17 illustrates the disaggregation of assets procurement by beneficiary household age and gender.

Figure 17: Masholomoshe irrigation scheme household expenditure on moveable assets

Agricultural practices at Masholomoshe Irrigation scheme

Agronomic practices

The beneficiaries highlighted that they practice subsistence farming and that the following agronomic practices are practiced in the area:

- Crop rotation

- Bucket watering system

- Manual weeding and harvesting

- Irrigation agriculture & Rain fed agriculture

- Retained seed planting

- Mulching using tree leaves and crop residues

Water supply and conveyance

Masholomoshe irrigation scheme runs on water harvested from Nkashe dam, which is feared to be heavily silted and losing storage capacity to evaporation. That adding to the water conveyance losses faced when using the leaking canals poses a major risk of water shortages in the middle of a cash crop period. There is a borehole with a bush pump that is used to supply community members with water for domestic uses. However, Water supply for Masiyepambili was from Shashane River through sand abstraction system using a submersible pump inserted into the riverbed which drew up water. Shashane River has perennial pools throughout the year, indicating that base flow is consistently high through the rain season, the dry summer period to the time of the next rain season. There were challenges moving water to irrigate sections of land on slightly higher ground as the irrigation scheme was using gravity and concrete lined canals to move the water nearer the crops then use siphons for flood irrigation.

Cropping & Harvest

The cropping calendar for Masholomoshe is developed and shared by Agritex officers and the beneficiaries begin their land preparations, and they put together contributions for seed or seedlings and begin planting. All planting  begin at the same time for pest control and harvest management. At present in the gardens there are maize and sugar beans. Due to the small size of the farming area, this will mostly be for immediate consumption with a few sales to neighbours. Sugar beans can be dried and stored for household use. Maize if produced in sufficient quantities is sometimes sold to the GMB. It is harvested collectively. Other crops grown by the irrigation scheme include carrots. Rape, onion, tomatoes, cabbage, beetroot, lettuce, tsunga, cornflower. Plate 8 shows nursery and maize at Masholomoshe irrigation scheme. Table 19 shows estimated crop yields for Masholomoshe irrigation scheme.

Plate 8: Irrigation scheme nursery (left) and maize field (right)

Table 19 :Estimated crop yields

| Crops | Crop Recovery Rate (%) | Projected Three Year Production (Mt/ha) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Year 1 |  | Year 2 |  | Year 3 |  |
|  |  | Gross Yield | Net Yield | Gross Yield | Net Yield | Gross Yield | Net Yield |
| Maize | 95 | 5 | 4.75 | 6.5 | 6.18 | 8 | 7.60 |
| Sunflower | 90 | 2 | 1.80 | 2.3 | 2.07 | 2.5 | 2.25 |
| Sugar Beans | 90 | 1.5 | 1.43 | 2 | 1.90 | 2.5 | 2.38 |
| Groundnut | 90 | 2.5 | 2.25 | 3.5 | 3.15 | 4.5 | 4.05 |
| Garlic | 90 | 2.5 | 2.38 | 4 | 3.80 | 6 | 5.70 |
| Ginger | 90 | 15 | 13.50 | 17.5 | 15.75 | 20 | 18.00 |

Sales and marketing of produce

There is no formal means of transporting the produce from Masholomoshe scheme but it is often carried on foot or donkey drawn carts or bicycles to the area of the market. Livestock like donkeys and cattle are draught power using carts. The absence of a motorized vehicle has greatly affected the quantity of produce that can be transported by the irrigation scheme members. The other challenge faced is that because production is uniform (everyone grows the same crops at once), and most of the farm produce is perishable, they face severe losses when they fail to find a market to absorb their harvested produce. Beneficiaries would like training in; marketing, creating market linkages through networking with green grocers even as far as Bulawayo, value addition to perishable farm produce such as drying and packaging or other preservation means such as pickling and tinning.

Ethnicity/ language

Local languages spoken in the area include Ndebele, Zulu, Kalanga and Suthu. Although different languages are spoken within the area the beneficiaries work with each very well. The participants concurred that there was teamwork among them before the collapsed and even now and differences in language was not a barrier when working together.

Health status of the area

Most prevalent diseases in the Project area are diarrhoea, malaria, COVID-19, schistosomiasis, sexually transmitted infections (STIs). The diseases may not only pose risks to employees but may also add risk to the local people near the project site. The HIV prevalence of Gwanda district stands at 21.9%. The incidence rate per 1000 people for diarrhoea is 41.5, for dysentery its 0.9 and for schistosomiasis its 2.1. In Matobo district, the HIV prevalence stands at 18% while the incidence rate per 1000 people for diarrhoea, dysentery and schistosomiasis is 35.3, 2.6 and 2.1 respectively.

Transport and communication

The most common modes of transport in the area include vehicles, carts, motorbikes, bicycles.  Communication is being done by broadcast (television and especially radio), group (video, CDs, audio-cassettes, posters), and Interpersonal channels (community leaders, contact farmers, extension workers).

Gender Based Violence (GBV)

Gender based violence data collection was a combination of both qualitative and quantitative approaches that include household visits, document review, focus group discussions, key informant interviews and face-to-face interviews with community based organisations, NGOs and other organisations working on GBV service provision in the area. Prior to the implementation of the field research, a desk review was conducted to consolidate the existing data and information on GBV in Gwanda  and Matobo Rural District Council. Due to the sensitivity of data collection on GBV, enumerators were trained on how to handle this information to ensure the safety and confidentiality of the individual. The data collection was aimed at identifying knowledge, attitudes and practices around GBV, services availability, referrals, access challenges and prevalence rate of gender-based violence. Targets were community members and key informants, including multi-sectoral service providers and religious leaders.

GBV Key findings

The GBV analysis baseline assessment revealed the following trends related to GBV in the area:

- Gender based violence Statistics - Between December 2020 and February 2021, 141 GBV cases were reported. Per every 15 household, there is either child marriage or exploitation of girls from rich.

- Barriers and challenges - Mostly Gender Based Violence cases are often concealed by families and community leaders.

- Most frequent types of GBV - intimate partner and domestic violence, physical violence, harassment in public spaces, child marriage and denial of resources

- At-Risk Groups - women and girls headed households were identified as particularly vulnerable and at risk of GBV.

- The level of responsiveness by the police, community elders and elected leaders remains low in addressing rape and GBV cases at community level.

Plate 9 shows the Municipality of Gwanda carrying out gender based violence awareness in the project area. Figure 18 shows the GBV service providers in the Project area.

Plate 9: Municipality of Gwanda carrying out gender based violence awareness

Figure 18: Gender based violence stakeholder mapping

Determined crop types, land use patterns, crop rotations and water requirements

This section provides the determined crop types, land use patterns, crop rotations and water requirements for Masholomoshe irrigation scheme (Table 20) and the  irrigation Water Schedule Master Plan (Table 21). On the other hand, the determined crop types, land use patterns, crop rotations and water requirements for Masiyepambili irrigation scheme are provided in Table 22 and Table 23.

Table 20: Determined crop types, land use patterns, crop rotations and water requirements for Masholomoshe Scheme

| Name of Block | Area (Ha) | Parameter | Jan |  |  | Feb |  | Mar |  | Apr |  | May |  | Jun |  | Jul |  | Aug |  | Sep |  | Oct | Nov | Dec | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 4.00 | Crop Irrigation Requirements | Maize |  |  |  |  |  |  |  |  | Maize |  |  |  |  |  |  |  |  |  |  | Maize |  |  |
|  |  |  | 104.0 |  |  | 113.4 |  | 183.1 |  | 0.0 |  | 83.3 |  | 155.6 |  | 194.1 |  | 205.2 |  | 120.4 |  | 129.4 | 93.9 | 129.7 | 1,512 |
| B | 4.00 | Crop Irrigation Requirements | Sugar Beans |  |  |  |  |  |  |  |  | Sugar Beans |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 37.8 |  |  | 108.7 |  | 64.1 |  | 70.9 |  | 66.6 |  | 170.5 |  | 64.7 |  | 205.2 |  | 301.0 |  | 194.1 | 107.4 | 0.0 | 1,391 |
| C | 4.00 | Crop Irrigation Requirements | Sunflower |  |  |  |  |  |  |  |  | Garlic/Ginger |  |  |  |  |  |  |  |  |  |  | Sunflower |  |  |
|  |  |  | 94.6 |  |  | 108.7 |  | 183.1 |  | 70.9 |  | 66.6 |  | 170.5 |  | 64.7 |  | 205.2 |  | 301.0 |  | 194.1 | 107.4 | 115.2 | 1,682 |
| D | 4.00 | Crop Irrigation Requirements | Groundnut |  |  |  |  |  |  |  |  | Garlic/Ginger |  |  |  |  |  |  |  |  |  |  |  | Groundnut |  |
|  |  |  | 108.7 |  |  | 103.9 |  | 91.5 |  | 0.0 |  | 66.6 |  | 170.5 |  | 145.6 |  | 102.6 |  | 120.4 |  | 0.0 | 0.0 | 72.0 | 982 |
| TOTAL | 16.00 |  | 345.1 |  |  | 434.7 |  | 521.7 |  | 141.8 |  | 283.2 |  | 667.0 |  | 469.1 |  | 718.3 |  | 842.9 |  | 517.5 | 308.6 | 316.9 | 5,567 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL ANNUAL |  |  |  | 167,007 |

Table 21: Irrigation water management plan for Masholomoshe Scheme

| CROP IRRIGATION 2020 SCHEDULING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | Peak Drip | Peak Sprinkler | Peak Centre Pivot | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| Field Daily water requirement | m³/day | 1,197.38 | 1,025.80 | 1,025.80 | 383.0 | 485.1 | 621.3 | 170.9 | 344.6 | 811.8 | 570.9 | 874.1 | 1,025.8 | 627.5 | 373.2 | 365.3 |
| Irrigation Cycle | Days | 4.00 | 4.00 | 4.00 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 |
| Total Available Moisture (TAM) | Mm | 110.00 | 110.00 | 110.00 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 |
| Root Zone Depth (RZD) | M | 0.70 | 0.70 | 0.70 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| Allowable Depletion (AD) | % | 0.50 | 0.50 | 0.50 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Overall System Efficiency | % | 0.86 | 0.76 | 0.81 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 |
| Reticulation System Efficiency | % | 0.97 | 0.97 | 0.97 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 |
| Distribution Efficiency | % | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 |
| Irrigation System Efficiency | % | 0.92 | 0.82 | 0.87 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 |
| Dam Water Delivery Requirement | m³/day | 1,397.66 | 1,343.40 | 1,266.19 | 494.1 | 625.8 | 801.5 | 220.5 | 444.5 | 1,047.2 | 736.4 | 1,127.6 | 1,323.3 | 809.4 | 481.5 | 471.3 |
| Solar pumping 8 hours | m³/hr | 174.71 | 167.93 | 158.27 | 61.77 | 78.23 | 100.19 | 27.56 | 55.57 | 130.90 | 92.05 | 140.95 | 165.41 | 101.18 | 60.19 | 58.91 |
| Power Requirements |  | 55m head | 55.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Solar pumping 8 hours | Kw | 32.70 | 31.43 | 29.62 | 11.56 | 14.64 | 18.75 | 5.16 | 10.40 | 24.50 | 17.23 | 26.38 | 30.96 | 18.94 | 11.26 | 11.03 |

Table 22 Determined crop types, land use patterns , crop rotations and water requirements for Masiyepambili Scheme

| Name of Block | Area (Ha) | Parameter | Jan | Feb | Mar |  |  | Apr |  | May |  | Jun |  | Jul |  | Aug |  | Sep |  | Oct |  | Nov |  | Dec |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 2.50 | Crop Irrigation Requirements | Maize |  |  |  |  |  |  | Maize |  |  |  |  |  |  |  |  |  |  |  | Maize |  |  |  |  |
|  |  |  | 56 | 61 | 106 |  |  | - |  | 49 |  | 92 |  | 114 |  | 121 |  | 71 |  | 76 |  | 55 |  | 72 |  | 874 |
| B | 2.50 | Crop Irrigation Requirements | Sugar Beans |  |  |  |  |  |  | Sugar Beans |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20 | 59 | 37 |  |  | 41 |  | 39 |  | 101 |  | 38 |  | 121 |  | 178 |  | 114 |  | 63 |  | - |  | 811 |
| C | 2.50 | Crop Irrigation Requirements | Sunflower |  |  |  |  |  |  | Garlic/Ginger |  |  |  |  |  |  |  |  |  |  |  | Sunflower |  |  |  |  |
|  |  |  | 51 | 59 | 106 |  |  | 41 |  | 39 |  | 101 |  | 38 |  | 121 |  | 178 |  | 114 |  | 63 |  | 64 |  | 974 |
| D | 2.50 | Crop Irrigation Requirements | Groundnut |  |  |  |  |  |  | Garlic/Ginger |  |  |  |  |  |  |  |  |  |  |  |  |  | Groundnut |  |  |
|  |  |  | 58 | 56 | 53 |  |  | - |  | 39 |  | 101 |  | 86 |  | 61 |  | 71 |  | - |  | - |  | 40 |  | 565 |
| TOTAL | 10 |  | 186 | 235 | 301 |  |  | 83 |  | 167 |  | 393 |  | 277 |  | 424 |  | 497 |  | 304 |  | 181 |  | 177 |  | 3,224 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL ANNUAL |  |  |  | 96,711 |  |  |  |  |  |

Table 23 Irrigation Water Schedule Master Plan for Masiyepambili Irrigation scheme

| Parameter | Unit | Peak Sprinkler | Peak Drip | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field Daily Water Requirement | m³/day | 748.36 | 641.12 | 239.4 | 303.2 | 388.3 | 106.8 | 215.4 | 507.4 | 356.8 | 546.3 | 641.1 | 392.2 | 233.3 | 228.3 |
| Irrigation Cycle | Days | 4.00 | 4.00 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 | 4.0 |
| Total Available Moisture (TAM) | Mm | 110.00 | 110.00 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 | 110.0 |
| Root Zone Depth (RZD) | M | 0.70 | 0.70 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| Allowable Depletion (AD) | % | 0.50 | 0.50 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Overall System Efficiency | % | 0.86 | 0.76 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 | 0.78 |
| Reticulation System Efficiency | % | 0.97 | 0.97 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 | 0.95 |
| Distribution Efficiency | % | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 |
| Irrigation System Efficiency | % | 0.92 | 0.82 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 | 0.85 |
| Water Delivery Requirement | m³/day | 873.54 | 839.63 | 308.8 | 391.1 | 501.0 | 137.8 | 277.8 | 654.5 | 460.3 | 704.7 | 827.0 | 505.9 | 300.9 | 294.6 |
| Solar pumping 8 hours | m³/hr | 109.19 | 104.95 | 38.60 | 48.89 | 62.62 | 17.23 | 34.73 | 81.81 | 57.53 | 88.09 | 103.38 | 63.23 | 37.62 | 36.82 |
| Solar pumping 8 hours | kW | 20.44 | 19.64 | 7.22 | 9.15 | 11.72 | 3.22 | 6.50 | 15.31 | 10.77 | 16.49 | 19.35 | 11.83 | 7.04 | 6.89 |

Crop enterprises selected through participatory approach

The following crop enterprises selected are through the participation of beneficiaries, irrigation engineers and extension agronomists:

- Carrots is a root vegetable that is rich in nutrients. There is high demand of carrots in the local and are good for rotation purposes with onions.

- Rape and tsunga has high demand both local and export markets is one such profitable agricultural enterprise. The enterprise is only feasible and profitable in nitrogen and phosphorus rich soils.

- Onion - onions are a crop that lends itself well to small-scale and part-time farming operations. Multiple markets exist for growers with small acreages. Several marketing alternatives are available to the onion grower: wholesale markets, cooperatives, local retailers, and roadside stands, farmers markets.

- Maize is a staple crop which is strategically maintained in reserves by Grain Marketing Board and has a stable market in Zimbabwe. It also contributes to household food and nutritional security. It was agreed to grow maize using the Pfumvudza/Intwasa concepts for climate proofing.

- Sunflower is a valuable crop used in cooking oils and stock feeds.  The Government of Zimbabwe is promoting the compulsory adoption of sunflower as a strategic crop. It also has relatively low water requirements.

- Groundnuts and sugar beans have high demand in both local and export markets and are good for rotation purposes with cereals (maize) and nitrogen fixation.

- Garlic and ginger are high value crops that have high demand in both local and export markets and are good cash crops.

- Tomatoes – have a high demand, they are used on daily bases by households and companies.

- Beetroot – market is anticipated to expand at a commendable pace over driven by the shifting preference from synthetic nitrates and nitrites to natural beetroot powder and other extracts across the meat industry worldwide.

- Lettuce - demand will mostly pick up during the approaching holidays with salads on the dinner menu.

The economic analysis of the crop enterprises were conducted as per clause 7 of Terms of Reference for detailed information see the economic analysis embedded in the Feasibility study Report.

# 6.0 STAKEHOLDER CONSULTATION

## 6.1 Stakeholder engagement methodology

A mixed methods approach was adopted in stakeholder engagement. This involved collecting, analysing and integrating quantitative and qualitative data. Data was collected by conducting in-depth interviews, administering questionnaires, meetings and focus group discussions with stakeholders (project-affected parties, other interested parties and vulnerable groups). Key informants who included experts/officer from Agritex, ZINWA, MSD, Forestry, Ministry of Health and Child Care, Environmental Management Agency, Zimbabwe Republic Police, Ministry of Women Affairs, Community, Small and Medium Enterprise, Local District Development Coordinator Office, farmers were interviewed and given questionnaires to understand how land management and irrigation agriculture is currently being done and their view with regards to climate, food security and sustainable environmental management.

Meetings and focus group discussions were carried out with irrigation and community members to seek local knowledge on the status of socioeconomic, environmental and irrigation scheme issues or concerns. Transect walks were conducted with stakeholders to assess the level of environmental status including land degradation and its causes. The data collection methods provided extensive baseline information on the project beneficiary group to facilitate informed decisions in the proposed project planning and implementation.

## 6.2 Stakeholder Identification and Analysis

Project stakeholders were identified and determined their priorities and objectives in relation to the project. Stakeholders were analysed on how they will be affected by the projects in order to plan engagement accordingly. Stakeholders have been and will continue to be identified on a continuing basis by identifying various stakeholder categories that may be affected by, or be interested in, the Project considering the project area and nature of impact. Table 24 provide the results of stakeholder analysis.

## 6.3 Key stakeholders consulted

Table 25 and Table 26 provides a list of stakeholders consulted and their views and concerns in related to the revitalization of Masholomoshe and Masiyepambili irrigation schemes.

Table 24 Stakeholder Identification and Analysis

| KEY |  |  |
| --- | --- | --- |
| High positive impact |  | HPI |
| High negative impact |  | HNI |
| Low positive impact |  | LPI |
| Low negative impact |  | LNI |
| High positive influence/Resource |  | HPI/R |
| High negative influence/Resource |  | HNI/R |
| Low positive influence/Resource |  | LPI/R |
| Low negative influence/Resource |  | LNI/R |
| Stakeholder | Stakeholder interest |  |  | Impact | Influence /resource | Resource | Impact/ Influence group | Project Consultation Stage |
| CORE DECISION MAKERS |  |  |  |  |  |  |  |  |
| UNDP/ GCF | Project funding |  |  | HPI | HPI/R | Project funds and technical expertise | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| Ministry of Lands, Agriculture, Water, Climate and Rural Resettlement | Provision of technical, extension, advisory, regulatory and administrative services. Land Ownership and Land Use |  |  | HPI | HPI/R | Project funds and technical expertise | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| Department of irrigation | Provision of agricultural engineering, mechanization and irrigation services |  |  | HPI | HPI/R | Technical expertise and irrigation schemes | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| ZINWA | Water resources management |  |  | LPI | HPI/R | Water resources | Group 4 | - Inception - Design - Project implementation - Decommissioning |
| Environmental Management Agency | Environmental protection |  |  | HPI | HPI/R | Technical expertise | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| Matobo Rural District Council | Local governance administration |  |  | HPI | HPI/R | Land | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| Contractors and consultants | Installation and maintenance of the irrigation scheme |  |  | LPI | HPI/R | Technical expertise and equipment | Group 4 | - Inception - Design - Project implementation - Decommissioning |
| DIRECTLY AFFECTED GROUP |  |  |  |  |  |  |  |  |
| Irrigation scheme project members | Irrigation scheme employees |  |  | HPI | HPI/R | Technical expertise and land | Group 2 | - Inception - Design - Project implementation - Decommissioning |
| Project workers/ representatives | Employment |  |  | HPI | LPI/R | Technical expertise | Group 1 | - Project implementation - Decommissioning |
| OTHERS |  |  |  |  |  |  |  |  |
| AGRITEX | Crop production through research and extension services |  |  | LPI | LPI/R | Technical expertise | Group 3 | - Project implementation |
| Meteorological services department | Advisory on matters pertaining to meteorology, climate and seismology |  |  | LPI | LPI/R | Meteorological data | Group 3 | - Project implementation |
| Forestry department | Sustainable utilization of forest resources |  |  | LPI | HPI/R | Technical expertise | Group 4 | - Inception |
| Parks and wildlife | Wildlife protection |  |  | LPI | HPI/R | Technical expertise | Group 4 | - Project implementation |
| Councilor | Consultation on community issues |  |  | LPI | LPI/R | Administrative coordination | Group 3 | - Inception |
| District Development Coordinator | Development coordination |  |  | HPI | LPI/R | Administrative coordination | Group 1 | - Inception |
| ZRP | Law and order maintenance |  |  | LPI | HPI/R | Technical expertise | Group 4 | - Inception - Design - Project implementation - Decommissioning |
| Ministry of health and child care | Public Health services administration |  |  | HPI | HPI/R | Technical expertise | Group 2 | - Inception - Design - Project implementation |
| Farmers associations | Coordinating farming programs within the area |  |  | LPI | LPI/R | Technical expertise | Group 3 | - Inception |
| Ministry of Women Affairs, Community, Small and Medium Enterprises | Gender and community issues |  |  | HPI | LPI/R | Technical expertise | Group 1 | - Inception - Design - Project implementation - Decommissioning |
| Chief | Consultation on community issues |  |  | HPI | LPI/R | Cultural knowledge | Group 1 | - Inception - Project implementation |
| Community members | Employment opportunities |  |  | HPI | LPI/R | Labour | Group 1 | - Inception - Design - Project implementation - Decommissioning |

Table 25 Key Consulted Stakeholders and their views in relation to Masholomoshe scheme

| Date | Location | Stakeholder | Method of engagement | Issues discussed /Concerns raised | Response | Information disclosed | Method of disclosure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| March –July 2022 | Harare and Gwanda | UNDP | - Workshops - Meetings | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report and ESMPs - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Email |
| March –July 2022 | Harare | Ministry of Lands, Agriculture, Water, Fisheries and Rural Resettlement | - Meeting - Interview | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Email |
| March –July 2022 | Gwanda | Department of irrigation | - Letter of consultation and prospectus - Transect walks | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Meetings and presentations |
| March –July 2022 | Gwanda | Irrigation scheme project members (Group 1) Women = 26 Men = 39 | - Focus group discussion - Questionnaire - Meeting | - Water loses due to breakdown of irrigation infrastructure - The dam water finishes before the rain season, low water table - We need a storage tank for plot A so that it complements the dam - Lack of resources to do repairs in between harvests e.g. cement, pipes, fencing. | - Incorporate concerns in Draft feasibility and ESMP reports - Irrigation engineer to present on irrigation technology to ensure farmers are well informed about irrigation technology | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Meetings and presentations |
| March –July 2022 | Gwanda | Irrigation scheme members (Group 2) Women = 28 Men = 34 | - Focus group discussion - Questionnaire - Meeting | - We have no filters for sand and debris when we pump water - We need additional siphons - We are unable to replace the siphons when they are in disrepair - The roof of the shed is leaking and the walls are falling apart, - Not in favour of drip and solid set irrigation technology | - Incorporate concerns in - Draft feasibility and ESMP reports - Irrigation design - Irrigation engineer to present on irrigation technology to ensure farmers are well informed about irrigation technology | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Meetings and presentations |
| March 2022 | Gwanda | Environmental Management Agency | - Key informant interview | - To provide ESMP that meets Environmental Management Act and EIA and ecosystem Regulation | - Final ESMP to be submitted for approval | - Environmental and social prospectus - Final ESMP to be disclosed | Telephone Submission of hard copies |
| March – July 2022 | Gwanda | AGRITEX | - Meeting - Interview - Transect walks | - Masholomoshe has no perimeter pig fence and domestic animals and wildlife can easily gain access. - There is need to install a new fence and gates to ensure security of the crops against animals and theft - Concerned with crops to be grown and cropping practices - We prefer center pivot technology | - A perimeter pig fence will be installed around the scheme perimeter. | - Project concept, irrigation technology and Social and Environmental Standards screening procedure - Draft assessment finding, management plans and irrigation technology - Draft ESMP | Meetings and presentations |
| March 2022 | Gwanda | Gwanda Rural District Council | - Interview | - The project is a welcome development in the district as it will further create employment. - There is need for proper disposal, handling or storage of waste during the project lifecycle. | - Waste bins will be provides and waste will be disposed in a designated site by rural district council. | - Project concept, - Irrigation technology | Telephone |
| March 2022 | Gwanda | Councilor | - Interview | - -We welcome the development as it will improve livelihoods of many people in the area. - Locals should be considered first when employing | - Women and youths will be considered in every activities of the irrigation scheme. - Locals are prioritised when recruiting. | - Project concept - Irrigation technology | Telephone |
| March 2022 | Gwanda | District Development Coordinator | - Interview | - The project is being welcomed into the community as it will empower the local people |  | - Project concept | Telephone |
| March 2022 | Gwanda | ZRP | - Interview | - They are happy with the project as it will help to reduce crime rate |  | - Project concept | Telephone |
| March 2022 | Gwanda | Ministry of health and child care - District medical office | - Questionnaire | - A source of safe water for drinking should be provided onsite - Livestock may contract measles from surrounding pasture if the irrigation is not provided with adequate latrines. - Mismanaged water storage and pool from leakages provide breeding places for mosquitos |  | - Project concept | Face to face and telephone |
| March 2022 | Gwanda | Farmers association | - Interview | - No comments |  | - Project concept | Telephone |
| March 2022 | Gwanda | Ministry of Women Affairs, Community, Small and Medium Enterprises | - Interview | - Gender equality and upholding of human rights |  | - Project concept | Telephone |
| March 2022 | Gwanda | Chief | - Interview | - Employment priority for the irrigation project should be locally based especially non-skilled labour. - Need for consultations with village heads in order to provide lists of community members fit for employment. | - Community members will be prioritized for labour - village heads will be consulted when the need for recruitment arises. | - Project concept | Telephone |
| March 2022 | Gwanda | Community members | - Questionnaire | - Local employment creation | - To give first preference to locals during employment of labour. | - Project concept | Oral |
| March 2022 | Bulawayo Gwanda | ZINWA | - Key informant interview | - There should be regular monitoring of water quality on the downstream side of the irrigations scheme. - The major concern of stakeholders is to make sure that nearby surface and ground water sources are not going to be affected in terms of quality and quantity. - Water is a major source of conflict among communities; access to water should be equitable. - There is also need for rehabilitation of the Nkashe dam which is heavily silted this also makes it shallow and affects the amount of water retained during rain events through increased surface for mass evaporation. | - Water quality monitoring will be done as per water monitoring plan (upstream, project site and downstream) | - Project concept and water quantity requirements | Telephone and email |
| March 2022 | Harare | Climate department | - Interview | - The irrigation project must incorporate viable climate proofing interventions |  | - Project concept | Telephone |
| March 2022 | Gwanda | Forestry commission | - Interview - Questionnaire | - The used of timber for fencing and protecting irrigation scheme crops causes a lot of tree cutting. - The project needs to provide secure, long standing fencing to ensure communities do not need to substitute fencing with logs and tree trunks. | - Secure fence will be installed and live fencing will be considered. | - Project concept | Telephone |
| March 2022 | Harare | Meteorological department | - Interview | - Meteorological data is available at a cost | - The metrological data was bought | - Project concept | Telephone |
| June 2022 | Gwanda | - DOI- Provincial Irrigation Engineer - ZINWA – Catchment Manager and Hydrologist - AGRITEX - Officer - Forestry Commission - Officer - Environmental Management Agency – Officer - Gwanda RDC - Natural Resources Officer and CEO - Contractors and consultants - Village Head / Chief - District Development Coordinator - Ministry of Women Affairs, Community, Small and Medium Enterprises ZRP | - Meeting | - ESMP and Feasibility study validation | - Incorporate comments on the ESMP and feasibility | - Project information - Draft Feasibility study and ESMP presentation | Meetings and presentations |

Table 26: List of stakeholders consulted and their views for Masiyepambili scheme

| Date | Location | Stakeholder | Method of engagement | Issues discussed /Concerns raised | Response | Information disclosed | Method of disclosure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| March –July 2022 | Harare and Matobo | UNDP/ GCF | - Meeting - Interview | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Email |
| March –July 2022 | Harare | Ministry of Lands, Agriculture, Water, Fisheries and Rural Resettlement | - Meeting - Interview | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Email |
| March –July 2022 | Matobo | Department of irrigation | - Letter of consultation and prospectus - Transect walks | - Terms of reference - Review comments - Reviewed feasibility study report and ESMP - Reviewed irrigation design | - Revised draft - Revised feasibility study report - Revised irrigation design | - Draft and final Inception report - Draft and final Feasibility study report and ESMP - Draft and final Irrigation design | Meetings and presentations |
| March 2022 | Matobo | Environmental Management Agency | - Key informant interview | - To provide ESMP that meets Environmental Management Act and EIA and ecosystem Regulations | - Final ESMP to be submitted for approval | - Environmental and social prospectus - Final ESMP to be disclosed | Telephone Submission of hard copies |
| March – July 2022 | Matobo | AGRITEX | - Meeting - Interview - Transect walks | - Water might get depleted at the expense of other villagers and livestock. - It is necessary that some trees will be cut to make way for the extension of the scheme - If no chemicals are used in the scheme, animals will not be affected and will gain feed from crop residues and fodder crops that will be planted in the scheme. - As long as all villagers are consulted, there is voluntary membership, there will be no conflicts and the locals will grow crops they want. - There should be a provision for non-members to use water for domestic purposes as they could vandalise the system if they deem themselves to be outsiders - If the scheme is not fenced, crops may be destroyed by livestock hence conflicts. | - Incorporate concerns in Draft feasibility and ESMP reports - Scheme members will be assisted byIAGRITEX officers - Training will be conducted as per ESMP training plan | - Project concept, irrigation technology and Social and Environmental Standards screening procedure - Draft assessment finding, management plans and irrigation technology - Draft ESMP | Meetings |
| March 2022 | Matobo | Village Head | - Questionnaire | - There is enough land that can be used for agricultural activities if there are enough inputs that can foster irrigation - There is a reliable water source for crop production. The scheme can suck its water from the Tshashane river bed which provides water throughout the year. - Animals are facing serious drought. Some which were common are now rare. - Scheme can enhance food production for the community as well as helping the community to manage the land properly - Improves nutrition due to variety of crops grown - Most GBV cases are triggered by economic situation in the area | - Incorporate concerns in Draft feasibility and ESMP reports - Project to kick off | - Project concept - Irrigation technology | Meeting |
| March 2022 | Matobo | Matobo Rural District Council | - Questionnaire | - Not much air pollution is expected - Land degradation due to land clearing - Biodiversity loss due to land clearance - There will be reduced grazing land for livestock production - Less future land for other uses such as cropping and residential - Livestock may vandalise irrigation solar equipment - Proponent should follow all EIA recommendations. - System should guard against veld firs and theft - Beneficiaries should be trained on irrigation management | - Incorporate concerns in Draft feasibility and ESMP reports - Implement management plans in the ESMP | - Project concept, - Irrigation technology | Telephone |
| March 2022 | Matobo | ZRP | - Interview | - It might lead to reduction in crime. The youth will be kept occupied and will also get some income out of the use of land - The project will make water to be easily accessible rather than what is currently present. - Potential of reducing stock theft cases. - The project is likely to address gender based violence by providing income as most cases are a result of lack of money in families. | - Incorporate concerns in Draft feasibility and ESMP reports | - Project concept | Telephone and questionnaire |
| March 2022 | Matobo | Matobo South Development Initiative | - Questionnaire | - Crop residues should be put in a compost that burning so as to prevent air pollution - Farmers should use organic fertilizers to improve soil quality - Where possible, farmers should plant trees within scheme - Farmers should not use pesticides to deal with lethal toxins as these will lead to bioaccumulation where these toxins end up affecting other animals that are not pests. - Some communities may lose land so its is important to consult with the community leaders - Some people might want to use water from the irrigation for household or commercial use. | - Incorporate concerns in Draft feasibility and ESMP reports - Implement management and monitoring plans to manage risks and impacts | - Project concept | Telephone |
| March 2022 | Matobo | Ministry of Women Affairs, Community, Small and Medium Enterprises | - Interview | - Women should be included in the project. - Gender equality and upholding of human rights |  | - Project concept | Telephone |
| March 2022 | Matobo | Chief | - Interview | - Employment of locals during construction and operational phases of the projects - The project is a good initiative. | - Incorporate comments in the draft document - Community members will be prioritized for labour | - Project concept | Telephone |
| March 2022 | Matobo | Councilor | - Interview | - We welcome the development as it will improve livelihoods of many people in the area. - The councillor participated in securing additional land for the scheme. | - Incorporate comments in the draft document. | - Project concept - Irrigation technology | Telephone |
| March 2022 | Matobo | District Development Coordinator | - Interview | - We welcome the project. The community will benefit a lot. |  | - Project concept | Telephone |
| March 2022 | Matobo | Gracious Dube | - Questionnaire | - Gracious Dube was concerned of loss of his farming business | - Include the farmer in the irrigation scheme (Appendix 1) | - Project concept | Telephone and oral |
| March 2022 | Matobo | Community members | - Questionnaire | - Consider locals for employment | - To give first preference to locals during employment of labour. | - Project concept | Oral |
| March 2022 | Bulawayo | ZINWA | - Key informant interview | - Regular monitoring of surface and ground water quality downstream. - Always conserve water | - Water quality monitoring will be done as per water monitoring plan (upstream, project site and downstream). | - Project concept and water quantity requirements | Telephone and email |
| March 2022 | Harare | Climate department | - Interview | - The climate is changing, the irrigation project must incorporate viable climate proofing interventions. | - Climate proofed infrastructure to be designed | - Project concept | Telephone |
| March 2022 | Matobo | Forestry commission | - Interview - Questionnaire | - Salinization may occur - Pollution of surface and ground water from pesticides use - Land degradation from cutting down of trees - Irrigation scheme will improve land use. - Less grazing area for livestock due to extension of the irrigation scheme - Consider gender balance when selecting irrigation scheme members. | - Implement management and monitoring plans to manage the risks and impacts | - Project concept | Telephone |
| March 2022 | Harare | Meteorological department | - Interview | - Meteorological data is available at a cost | - The metrological data was bought | - Project concept | Telephone |
| June 2022 | Matobo | - Department of irrigation - Provincial Irrigation Engineer - ZINWA – Catchment Manager and Hydrologist - AGRITEX - Officer - Forestry Commission - Officer - Environmental Management Agency – Officer - Matobo Rural District Council – Natural Resources Officer and CEO - Contractors and consultants - Village Head / Chief - District Development Coordinator - Ministry of Women Affairs, Community, Small and Medium Enterprises ZRP | - Meeting | - ESMP and Feasibility study validation | - Incorporate comments on the ESMP and feasibility | - Project information - Draft Feasibility study and ESMP presentation | Meetings and presentations |

# 7.0 IMPACT IDENTIFICATION AND ANALYSIS

## 7.1 Introduction

An “impact” is defined as a change to the environment or social domains, whether adverse or beneficial, wholly or partially resulting from a project activity. Environmental and social impacts were identified through a mixed methods approach. These include stakeholder consultations, checklist, matrices, literature review, safety data sheets, professional judgement, etc. The environmental and social aspects were determined following the life cycle approach from construction to decommissioning. Subsequently these were rated for significance with the established criteria covering likelihood and impact rating in line with the UNDP social and environmental screening procedure.

## 7.2 Environmental and Social Impact Assessment Method

The overall rating of the project is moderate. The rating of environmental and social aspects and impacts to determine significance was done using the following matrix which considers the following:

- Impact (e.g. consequences if the risk were to occur) (Table 27) and

- Likelihood (e.g. the chance of the risk occurring) (Table 28) for each identified risk.

- Matrix for determining ‘Significance’ of Risk (Table 29)

Table 27: Rating the ‘Impact’ of a Risk

| Score | Rating | Social and environmental impacts |
| --- | --- | --- |
| 5 | Extreme | Significant adverse impacts on human populations and/or environment. Adverse impacts of large-scale magnitude and/or spatial extent (e.g. large geographic area, large number of people, transboundary impacts, cumulative impacts) and duration (e.g. long-term, permanent and/or irreversible); areas adversely impacted include areas of high value and sensitivity (e.g. valuable ecosystems, critical habitats); adverse impacts to rights, lands, resources and territories of indigenous peoples; involve significant levels of displacement or resettlement; generates significant quantities of greenhouse gas emissions; impacts may give rise to significant social conflict |
| 4 | Extensive | Adverse impacts on people and/or environment of considerable magnitude, spatial extent and duration, but more limited than Extreme (e.g. more predictable, mostly temporary, reversible). Impacts of projects that may affect the human rights, lands, natural resources, territories, and traditional livelihoods of indigenous peoples are to be considered at a minimum potentially Extensive |
| 3 | Intermediate | Impacts of medium magnitude, limited in scale (site-specific) and duration (temporary), can be avoided, managed and/or mitigated with relatively uncomplicated accepted measures |
| 2 | Minor | Very minor impacts in terms of severity and magnitude (e.g. small affected area, very low number of people affected) and duration (short), may be easily avoided, managed, mitigated |
| 1 | Negligible | Negligible or no adverse impacts on communities, individuals, and/or environment |

Table 28: Rating the ‘likelihood ‘of a risk

| Score | Rating |
| --- | --- |
| 5 | Expected |
| 4 | Very likely |
| 3 | Moderately |
| 2 | Low likelihood |
| 1 | Not likely |

Table 29: Determining ‘Significance’ of Risk

7.3 Anticipated positive impactsacts

- Greater food security than rain-fed production and increase in the monetary income will reduce poverty and food shortage.

- Improved beneficiary community resilience to climate change impacts on subsistence farmers

- Employment Creation

- Improved infrastructure and irrigation facilities

- Improved livelihoods

- Enhanced agricultural practices and farmers’ organizational capacity

- Improved market access and access to rural financial services

- Creation of job opportunities and possible improvement in living standards.

## 7.4 Anticipated negative environmental and social impacts

Table 30 provides the identification and analysis of the potential environmental and social impacts during all phases of project development.

Table 30 Environmental and Social Impacts Rating

| Activity | Aspect | Impact | Likelihood | Impact rating | Score | Significance of risk | Control measure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRE-CONSTRUCTION AND CONSTRUCTION PHASE |  |  |  |  |  |  |  |
| Topographical surveys and earthworks | Vegetation clearance | Loss, fragmentation and degradation of habitat, and severance of animal migration routes and pathways | 5 | 2 | 10 | Moderate | - Removal of vegetation will be limited to the project area and working areas only - Careful siting of all project infrastructure and components - Rehabilitation of cleared areas with native species |
| Potential excavations and earthworks | Displacement or damage to cultural heritage artefacts | Loss of tangible forms of cultural or archaeological importance | 2 | 3 | 6 | Low | - An archaeological survey was carried out and no artefacts were found in the project implementation areas. - Activate a “Chance Finds” procedure in case tangible forms of cultural or archaeological importance in encountered |
| Construction and earthworks activities | Poor construction management practices | Adverse effects on safety, human health and wellbeing | 5 | 2 | 10 | Moderate | - Good construction site “housekeeping” and management procedures (including site access). - Risk assessments and emergency response planning to consider impacts on local communities. |
| Domestic activities at the camp | Use of fire for cooking and lighting of cigarettes | Loss of life and property | 3 | 3 | 9 | Moderate | - Designate smoking zones which should be away from flammable and combustible materials - Provision of a fire guard |
|  | Generation of waste (Domestic waste & Human waste) | Land and water pollution | 2 | 4 | 8 | Moderate | - Implement the waste management hierarchy (prevent, reuse and dispose) - Provide waste bins for disposal of waste - Provide Mobile ablution facilities - Provide training on waste management |
| Worker recruitment | Recruitment of persons under 18 years of age | Child labour | 5 | 2 | 10 | Moderate | - Stop hiring children under the minimum age - Provide training and awareness programs for employees at all levels of vulnerability and build accountability by assigning clear responsibilities at all levels |
| Labour influx | Movement of plant and workforce into areas could introduce invasive | Loss of fauna, flora and ecosystem services, | 5 | 2 | 10 | Moderate | - Where possible, provide mechanical clearance of invasive species particularly during routine maintenance of water distribution systems. - No introduction of exotic species (e.g. for site rehabilitation) without specialist vetting and government approval |
| Labour influx | Labour influx may lead to sexual , exploitation, abuse and harassment (SEAH) of women and girls | -Depression - Anxiety -Trauma -Suicide -Discrimination -Social stigma -Rejection | 2 | 5 | 10 | Moderate | - Employment of local people. - Contractor workforce to sign the code of conduct on Sexual Exploitation, Abuse and Harassment (SEAH) - Implement the Grievance Redress Mechanism - Training and Awareness raising on SEAH related issues - Health centres near the site to be equipped with adequate, personnel, medication, utilities |
|  | Interaction between workforce and local communities | Increased occurrence of communicable diseases, including HIV/AIDS and sexually transmitted Infections (STDs). | 4 | 2 | 10 | Moderate | - Implementation of a health management system for the construction workforce, to ensure it is fit for work - Training and awareness raising for workforce on HIV/AIDS and other STDs, and communicable diseases; - Health awareness raising campaigns for communities on similar topics - Provision of Voluntary Counselling and Testing and –Treatment services |
|  | Increase spread of COVID-19 infections | Ill health, Death | 2 | 3 | 6 | Low | - Practising social distancing. - Improved personal hygiene. - Wearing of face masks - Daily temperature screening - Providing COVID-19 awareness training to contractors and famers - Vaccination. |
| Recruitment of workforce by contractors | Poor working conditions | accidents, injuries, illnesses, depression etc. among workers | 4 | 2 | 10 | Moderate | - Employment practices and working conditions should conform to International Labour Organisation (ILO) Standards and national requirements - Clear and comprehensive health and safety reporting and grievance procedure system should be established, and be freely available to all of the workforce. - Issue work contracts aligned to national laws and ILO standards |
| Vegetation clearance during construction | Loosening of the topsoil | Soil erosion | 4 | 2 | 10 | Moderate | - Revegetation of certain areas to reduce soil erosion. |
| Vegetation clearance | Generation of waste | Land pollution | 4 | 2 | 10 | Moderate | - Removed vegetation will be used for firewood by the community. |
| Construction activities | Dust and emissions from construction and maintenance activities | Air pollution Human health risks e.g respiratory illnesses | 2 | 1 | 2 | Low | - Spraying water during the excavation phase - Sensitive site selection and siting of construction facilities. - Use of modern equipment meeting appropriate emissions standards, and regular preventative maintenance. - No use of ozone depleting substances during construction |
| Vehicle and machinery use | Fuel usage | Natural resources depletion | 5 | 1 | 5 | Low | - Vehicles will be switched off when not in use - Implement speed controls - Planning on proper route selection |
| Vehicle and machinery use | Accidental release of hazardous substances (spills and leaks) | Land and water pollution | 2 | 2 | 4 | Low | - Regular vehicle inspection and maintenance - Implement materials handling and control procedures, use of storage and containment equipment meeting international standards. - Control of construction vehicle movements and prohibition of vehicle washing in watercourses, and similar practices. - Implement emergency response plans during construction (contractors) |
|  | Health and safety | Accidents/incidents | 4 | 2 | 10 | Moderate | - Implement a health and safety plan |
| Vehicle and machinery use | Generation of noise and vibration | Noise and Vibration | 3 | 2 | 6 | Low | - Construction works will be carried out during the day only. - Serviced equipment should be used - Sensitive local access road route selection and siting of construction facilities - Observance of seasonal sensitivities (e.g. breeding seasons), and alteration of activity to reduce noise levels at that time. |
| Vehicle and machinery use | Vehicle Emissions | Air pollution | 5 | 1 | 5 | Low | - Vehicles will be switched off when not in use. - Reducing number of trips and frequency of operation of the vehicles. |
| Cement mixing | Generation of cement dust | Air pollution | 2 | 2 | 4 | Low | - Water will be sprinkled during mortar mixing to reduce dust emissions |
|  | Water usage | Depletion of natural resources | 5 | 1 | 5 | Low | - Provide training on efficient use of natural resources - Monitoring water usage. - Reusing water from other plant processes |
|  | Generation of packaging waste | Land pollution | 5 | 1 | 5 | Low | - Adequate litter bins will be provided - Cement bags shall be reused by the community |
| OPERATIONAL PHASE |  |  |  |  |  |  |  |
| Irrigating crops | Water use | Depletion of natural resources | 5 | 2 | 10 | Moderate | - Maintenance of the irrigation systems to prevent water leaks - Crop irrigation plan should be adhered to - Abstraction to take place with approval of relevant authorities - Water study prior to any abstraction, to inform a Sustainable Water Management Plan. - Regular inspection of all system components to ensure minimal wastage of water |
| Water abstraction from Nkashe dam and Shashane River | Reduced environmental flows resulting in affected downstream ecosystems and riverine vegetation | Biodiversity loss | 4 | 2 | 8 | Moderate | - Sustainable use of dam water to avoid over extraction which might affect downstream ecosystems. |
|  |  | Conflict with non-irrigation scheme members over water for livestock and domestic uses | 4 | 2 | 8 | Moderate | - Sustainable use of river water to avoid over extraction |
| Plant and crop maintenance | Application of fertilizer | Soil degradation (physical and chemical properties) | 5 | 2 | 10 | Moderate | - Conduct periodic soil analysis to detect changes in soil fertility - Practicing crop rotation - Use of organic fertilizers |
| Irrigation | Excessive use of poor-quality water | Salinization | 3 | 3 | 9 | Moderate | - Do not over irrigate crops. Crop irrigation plan should be adhered to |
| Crop irrigation | Interruption or disruption of environmental flows | Biodiversity loss | 3 | 3 | 9 | Moderate | - Design to take account of local hydrological conditions - Minimise the loss of water caused by leaks and evaporation - Install water efficient irrigation systems |
| Weeds and pest control | Application of pesticides and insecticides | Soil and water pollution leading to biodiversity loss Conflicts due to reduced river water quality downstream | 5 | 2 | 10 | Moderate | - Practice integrated pest/vector management programme - Avoid use of products that fall in Classes Ia (extremely hazardous) and Ib (highly hazardous) of the WHO Recommended Classification of Pesticides by Hazard - Use only approved pesticides - Ensure that pesticides are handled, stored, applied and disposed of in accordance with FAO International Code of Conduct on the Distribution and Use of Pesticides - Training of farmers on safe handling, storage, use and disposal of agrochemicals. |
| Unpacking of fertilizers, seeds and other raw materials and equipment | Generation of plastic waste | Land and water pollution | 3 | 2 | 6 | Low | - Examine alternative product formulations and packaging (e.g., biodegradable material). - Agricultural extension to train farmers in appropriate, sustainable application of fertiliser and other agrochemicals |
| Crop Irrigation | Changes in exposure to water borne and water related diseases, | Increased incidences of water borne diseases | 5 | 2 | 10 | Moderate | - Provide information, education and communication about safe uses of irrigation water and occupational safety. - Implement environmental management measures for vector control e.g. focal insecticide and molluscicide application. |
| Labour influx | Workforce-Community Interactions | Perceived disruption to normal community life, through the physical presence of a workforce | 3 | 2 | 6 | Low | - Adoption of a Stakeholder Engagement Plan, as a framework for early and ongoing community consultation. - Implementation of a Grievance Procedure (see Grievance Procedure - Works procedures, defining a Code of Appropriate Conduct for all workers, including acceptable behaviour with respect to community interactions |
| Harvesting | Generation of crop residues | Land and water pollution | 3 | 2 | 6 | Low | - Recycle residues and other organic materials by leaving the materials on site or through composting. |
| Application of pesticides | Generation of aerosols | Air pollution | 3 | 2 | 6 | Low | - Avoid applying of pesticides and herbicides during windy days. |
| Crop production | Planting crops | Introduction of invasive species | 2 | 2 | 4 | Low | - Sourcing planting material from reliable suppliers |
| Water runoff | Soil erosion | Siltation of rivers and dams | 5 | 2 | 10 | Medium | - Afforestation program for the catchment area |
| Crop irrigation | Excessive irrigation. | Loss, damage or disruption of soil including waterlogging | 3 | 2 | 6 |  | - Implement Irrigation scheduling - Install water efficient irrigation technologies e.g. center pivot, drip or solid set sprinkler system - Minimisation of cleared areas and soil disturbance, with revegetation as soon as feasible - Retention of topsoil for restoration - Careful consideration of timing of works (overall duration and seasonality). - Maintain drainage network and diversion structures |
| DECOMMISSIONING PHASE |  |  |  |  |  |  |  |
| Demolition of structures | Noise and dust generation | Noise Pollution | 1 | 1 | 1 | Low | - Demolition should be done during the day only - Implement speed controls |
| Demolition of structures | Generation of demolition waste | Land pollution | 3 | 1 | 3 | Low | - Waste will be reused/recycled. |
|  | Health and Safety | Accidents &incidences | 3 | 1 | 3 | Low | - Implement the Health and Safety Plan |
| Vehicle movement Vehicle movement | Vehicles emissions | Air pollution | 3 | 1 | 3 | Low | - Switching off vehicles when not in use |
|  | Fuel usage | Natural resources depletion | 4 | 1 | 4 | Low | - Use of services vehicles and monitoring fuel consumption of each vehicle - Speed controls |

# 8.0 ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

## 8.1 Introduction

The project proponent has established a detailed and comprehensive Environmental and Social Management Plan (ESMP) which she is committed to implement. With respect to the environmental and social aspects and impacts that have been identified in chapter 7 of this report. The ESMP provides the linkage between each aspect, impact, the relevant mitigation measures and the monitoring approach. The ESMP is the guiding tool that will ensure the protection of the environment by project proponent management, employees, and the irrigation scheme farmers through integrated approach of implementation of mitigation measures. The project proponent and the community shall be responsible for implementation and maintenance of the ESMP. The ESMP will thus be part of the contract documents to be signed by the contractors, binding them to implement the management plans during the construction operational and decommissioning phase of the project. After completing the construction phase, the sites will be handed over to the beneficiary community members for operation.

The Objectives of the ESMP are to:

- Promote environmental and social management and communicate the aims and goals of the ESMP.

- Incorporate environmental and social management throughout the proposed project lifecycle.

- Address concerns and issues that were raised during stakeholder engagement.

- Serve as an action plan for environment and social management.

- Provide a framework for implementing project environment and social commitments i.e. mitigation measures identified for all environmental and social impacts.

- Identify mitigation measures required to address identified social and environmental risks and impacts

- Provide measures for stakeholder engagement& grievance redress, capacity development & monitoring as relates to the project’s environment and social matters.

## 8.2 Specific Environmental and Social Management Plans

Table 31 to Table 46 provides information for managing specific environmental and social issues.

Table 31 Pollution Prevention and Resource Efficiency Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Construction and setting up of irrigation components and infrastructure | Solid waste generation (plastic, paper, construction debris & scrap metal) | Land pollution | - Apply the waste management hierarchy (Avoid, Reduce, Re-use and Recycle) - Encourage waste segregation and provide designated waste bins - Use construction waste such as rubbles to rehabilitate any open pits created during the construction period; - Provide training on waste management - Erect waste management signage at suitable sites - Dispose waste at designated disposal sites as directed by the Matobo Rural District Council | - Contractor | $500.00 |
| Erection of site camp | Risks associated with use of fire for cooking and lighting of cigarettes | Destruction of property and life | - Designate smoking zones which should be free from flammable or combustible materials - Provision of a fire guard | - Contractor | 1000.00 |
|  | Generation of waste (Domestic waste & Human waste) | Soil and water pollution Disease outbreaks | - Provide waste disposal facilities - Provide mobile ablution facilities - Provide training on waste management | - Contractor | 1000.00 |
| Vehicle movement and machinery use | Fuel and oil leaks | Soil and water pollution | - Servicing of vehicle to be done at approved dealers with proper containment facilities - Regular inspections of vehicles, mobile equipment and machinery to check for oil and fuel leaks - Provide drip trays to stationary vehicle and machinery to prevent drips and small leaks onto the ground. - Develop and Implement the spill prevention control procedure | - MLAFWRD - Contractor - PIU | $1000.00 |
| Vegetation clearing at project site | Generation of vegetative waste (trees and grasses) | Land pollution | - Confine clearing to project site only to reduce quantities of waste generated. - Cut trees and grasses will be given to the local community for firewood. - Vegetative waste not suitable for use as biomass will be used to backfill trenches | - Contractor | $500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Weeds and pest control | Application of pesticides and herbicides | Soil and water pollution | - Practice integrated pest/vector management programme - Avoid use of products that fall in Classes Ia (extremely hazardous) and Ib (highly hazardous) of the WHO Recommended Classification of Pesticides by Hazard - Use only approved pesticides and avoid the use of banned pesticides - Ensure that pesticides are handled, stored, applied and disposed of in accordance with FAO International Code of Conduct on the Distribution and Use of Pesticides - Training of farmers on safe handling, storage, use and disposal of agrochemicals. - Testing the soil for pesticides residues - Chemical warehouses/stores should be situated from water sources as well as livestock and food storage areas. - Ensure products are stored in their original packaging; - Regular inspection of stored products to ensure their condition - Keep SDS at appropriate locations in storage facilities - Ensure chemical stores have appropriate ventilation | - MLAFWRD - IMC - Farmers | $5500.00 |
| Planting and crop maintenance | Application of fertilizers | Soil and water pollution | - Practice crop rotation to mitigate infestation of pests and diseases; - Practice intercropping; - Draw up balanced fertilizer programs for each soil management unit based on soil fertility results. - Use organic fertilizers where appropriate - Conduct periodic soil analysis to detect changes in soil fertility. - Provide farmers with training in nutrient management. | - Farmers - Agritex - Irrigation Management Committee (IMC) | $500.00 |
| Unpacking of fertilizers, seeds and other raw materials and equipment | Plastic waste | Land and water pollution | - Examine alternative product formulations and packaging (e.g., biodegradable material). - Promote plastic waste reusing - Dispose waste at designated dumping sites | - IMC and Farmers | $1000.00 |
| Application of pesticides and herbicides | Pesticides containers and packaging material | Land pollution | - Promote waste minimisation through efficient application of fertilisers - Provide waste bin for the temporary storage of empty pesticides and herbicides containers. - The containers should be disposed at designated places agreed by Matobo and Gwanda Rural District Councils - Any unused dilute pesticides and herbicides that cannot be applied to the crops, along with rinse water, and those out of-date should be disposed of as a hazardous waste. - Do not burn or bury empty packaging - Empty pesticide containers, foil seals, and lids should be triple rinsed, and washings must be used sprayed back onto the field or disposed of as hazardous waste. | - IMC and Farmers | $1000.00 |
| Harvesting | Burning of Crop residues | Air Pollution | - Practice composting of crop residues - Consider using crop residues as animal feed. | - IMC and Farmers | $2000.00 |
| In cooperation of crop residue in the soil and burning of crop residue | Addition to annual or intermittent residues Burning of crop residue | Extended release of nutrients during the development (growth) phase of the next crop cycle | - Develop and implement a residue management plan in combination with results from nutrient management research and planning - Consider using crop residues for other beneficial purposes - Avoid using harmful residual chemicals at end of crop life when preparing for removal. | - IMC and Farmers | $2000.00 |
| Crop harvesting | Decomposing chemical laden crop residues | Soil pollution | - Avoid using harmful residual chemicals at end of crop life when preparing for removal | - Farmers | $1500.00 |
| DECOMMISSIONING PHASE |  |  |  |  |  |
| Vehicle movement and machinery use | Fuel and oil leaks | Soil pollution | - Regular maintenance of vehicles, mobile equipment and machinery to prevent oil leaks. | - IMC - Farmers | $500.00 |
| Demolition and uprooting of equipment | Demolition waste | Land and water pollution | - Solid waste from demolition activities should be disposed at a site approved by Matobo Rural District Council - Any material that is reusable will be donated to the community for reuse. | - MLAFWRD | $250.00 |

Table 32 Water Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |  |
| --- | --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Concrete mixing | Water usage | -Depletion of natural resources -Water pollution | Adopt water use efficient practices e.g. - Train employees on water conservation techniques. - Water usage data shall be recorded - Onsite water reuse | Contractor | $3000.00 |  |
| - OPERATIONAL PHASE |  |  |  |  |  |
| Crops irrigation | Water use | Depletion of water resources and water use conflicts | Promote water use efficient practices e.g. - Determine crop water requirements based on local and internationally recognized guidelines to avoid over irrigation. - Develop an appropriate irrigation plan and schedule, and monitor consumption and compare regularly with set targets based on available supplies of water - Adopt soil and water conservation practices such as mulching - Adopt climate proofed water irrigation technology (e.g center pivot) - Conduct inspections of water irrigation infrastructure to detect any possible leakages and attend to them promptly - Provide training on irrigation operation and maintenance - Conduct periodic environmental flow analysis | IMC and Farmer MLAFWRD ZINWA | $1000.00 |  |
| Water runoff | Soil erosion | Siltation of Nkashe Dam and Shashane river dam | - Collaboration with other partners with watershed management scope e.g. Global Environmental Facility 7 implemented by FAO. - Afforestation program of the catchment area - Use of water efficient irrigations systems to reduce the risk of erosion - Avoiding disturbing riverine vegetation. | - IMC and Farmer | $5000 |  |
| Crop irrigation | Nutrient leaching & Water logging | Ground water pollution | - Implement irrigation scheduling - Use water efficient irrigation systems to decrease the risk of waterlogging, erosion and inefficient water use - Promote use of grass cover crops, adding a legume to a rotation, and adding crops that more fully utilize the soil-water resources. - Ensure appropriate soil moisture by active monitoring of soil humidity. | - IMC and Farmers - Agritex | $1000.00 |  |
| Crop irrigation | Water leaks | Depletion of water resources | - Ensure regular maintenance of the irrigation system and associated infrastructure - Regular equipment inspections | - IMC - MLAFWRD | $1000.00 |  |
| Pest and weeds control | Application of pesticides and insecticides | Water pollution | - Efficient application of pesticides - Training of small holder farmers on safe handling, use and disposal of agrochemicals. - Practice integrated pest/vector management programme - Avoid use of products that fall in Classes Ia (extremely hazardous) and Ib (highly hazardous) of the WHO Recommended Classification of Pesticides by Hazard - Ensure that Pesticides are handled, stored, applied and disposed of in accordance with FAO International Code of Conduct on the Distribution and Use of Pesticides - Training of farmers on safe handling, storage, use and disposal of agrochemicals. - Regular water monitoring (water physicochemical, biological and pesticide residue parameters) - Practice crop rotation | - IMC and Farmers - EMA | $2000.00 |  |
| DECOMMISSIONING PHASE |  |  |  |  |  |  |
| Watering of rehabilitated areas | Water use | Depletion of water resources | - Only rehabilitated areas will be watered. | IMC Farmers | $500.00 |  |

Table 33 Biodiversity and Natural Resources Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Site clearing during topographic and soil surveys and earthworks | Earthworks and clearance may lead to loss of plant species and habitats of conservation interest | -Habitat fragmentation -Habitat loss - direct animal disturbance from Noise and general human presence | - Confine clearing to project site - Careful planning of phasing and timing of construction activities. - Clearing shall be restricted to the proposed work site and routes only to allow smaller birds, mammals, reptiles an opportunity to migrate into undisturbed areas close to their similar habitats. - Ensure that no faunal species are disturbed, trapped, hunted or killed during the activities | Contractor Farmers | $500.00 |
| Site construction and clearing | Movement of plant and workforce into areas could introduce invasive species which adversely impact fauna, flora, ecosystems, and crops | Several impacts including habitat loss, local hydrology changes , and biodiversity loss | - Adopt physical and mechanical methods of invasive species control - Staff training and awareness raising in communities. - No introduction of exotic species (e.g. for site rehabilitation) without specialist vetting and government approval. - Where possible, clearance of invasive species during routine maintenance of water storage and distribution systems | Contractor Farmers | $500.00 |
| Trenching | Open trenches | Human and animal fall ins causing injuries or death | - Conduct community sensitisations - Ensure that all pits are barricaded and when animals found trapped in the pits, they shall be rescued. - Pits and trenches will be backfilled soon after installations | Contractor | $500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Crop production | Introduction of invasive species | Several impacts including habitat loss, nutrient cycling, local hydrology, and biodiversity loss | - Use of certified seeds - Sourcing planting material from reliable suppliers - Implementing machinery cleaning programs when moving between fields to remove soil and seeds that may carry invasive or alien species. - Clear invasive species using mechanical or physical means | IMC Farmers | $500.00 |
| Water abstraction and use for crop irrigation | Changes in environmental flows and salinity balance | Alteration in species distribution and breeding patterns of fish leading to biodiversity loss | - Monitoring environmental flows - Maintain adequate environmental flows through installation of water efficient irrigation system | MLAFWRD ZINWA | 1000.00 |
| Weed control and soil fertility management | Application of herbicides and fertilizers | Non-point source pollution leading to loss of aquatic life | - Agricultural extension to train farmers in appropriate, sustainable application of fertiliser and other agrochemicals. - Use selective pesticides, where appropriate, rather than broad-spectrum products to minimize impacts on non-target species. - Adopt mechanical weeding | IMC Farmers | $500.00 |
| DECOMMISSIONING PHASE |  |  |  |  |  |
| Uprooting of equipment and demolition of canals | Vegetation disturbance | Loss of vegetation and disturbance of fauna habitats | - Decommissioning activities will only be concentrated to the project site where installations were made - Revegetation of the disturbed environment | MLAFWRD | $500.00 |
| Removal of underground pipes | Creation of trenches | Trapping of fauna | - Backfilling of trenches after removing installations | IMC Farmers | $500.00 |

Table 34: Land Use Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| PRE CONTRUCTION AND CONSTRUCTION PHASE |  |  |  |  |  |
| Land utilisation of the Development of the Scheme | Land holders donate land for the development of the scheme | Land use conflict | - Land holders to sign land use consent forms - Involve the local leadership in the process of plot allocation - Inform the farmers about the benefits of the project | - MAWLFD | $$500.00 |
| Vehicle movement and machinery use | Vehicle movement on top soil | Soil compaction | - The movement of mobile equipment shall be limited to the areas of operation and will make use of designated routes only. - At the end of construction works, level off the soils and facilitate vegetation regeneration. | - Contractor | $$500.00 |
| Vegetation removal and excavations. | Loosening top soil | Soil erosion. | - Limit construction activities within designated area - Immediate backfilling of trenches - Avoiding trench excavation during rainy season where possible | - Contractor | $500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Planting and crop maintenance | Application of mineral fertilizers | Physical and Chemical degradation of the soil. | - Incorporate nitrogen-fixing legume crop plants and cover crops in the cropping cycle - Draw up balanced fertilizer programs for each soil management unit based on soil fertility results. - Conduct periodic soil analysis to detect changes in soil fertility. - Provide farmers with training in nutrient management. | - IMC and Farmers - Agritex | $500.00 |
| Crop maintenance | Irrigation | Soil erosion | - Use green manures, cover crops, or mulching techniques to maintain soil cover. - Consider using center pivot irrigation method | - IMC and Farmers | $500.00 |
| Land preparation | Loosening the topsoil | Soil erosion | - Practice reduced and zero tillage if possible. - Minimize soil compaction, disturbance by using appropriate land preparation machinery at the right time of year. - Replenish soil organic matter by recycling crop residues, compost, and manures. - Employ erosion control management practices (e.g., contour and strip planting and grass barriers) in sloping areas. | - IMC and Farmers | $500.00 |
| Irrigation | Excessive use of poor-quality water | Salinization | - Optimise irrigation (employ water efficient irrigation technology (e.g center pivot irrigation), and irrigation scheduling) - Practice soil moisture conservation practices e.g. application of organic manure and use cover crops or mulch to protect the ground surface. | - IMC and Farmers | $2000.00 |
| Crop harvesting | Decomposing chemical laden crop residues | Soil pollution | - Avoid using harmful residual chemicals at end of crop life when preparing for removal | - IMC and Farmers | $1500.00 |
| Weeds and pest control | Application of pesticides and insecticides | Soil pollution and reduction in soil fertility | - Practice organic farming practices - Training of farmers on safe handling, use and disposal of agrochemicals. - Testing the soil for pesticides residues - Practice crop rotation | - IMC and Farmers | $500.00 |
| Land tillage | Use of cultivators or manual hoes | Destruction of the soil structure and destruction of field crop. | - The cultivator should be well experienced so as in not to destruct crops. - To work with the soil when it is not too wet or too dry hence soil structure is maintained | - IMC and Farmers | $1500.00 |
| DECOMMISSIONING PHASE |  |  |  |  |  |
| Vehicle movement and machinery use | Fuel and oil leaks | Soil pollution | - Regular maintenance of vehicles, mobile equipment and machinery to prevent oil leaks. - Topsoil and subsoil shall be protected from contamination through use of center pivot tray during maintenance. | - MLAFWRD | $500.00 |

Table 35 Energy Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Use of vehicles and machinery | -Use of fuel (diesel) | Depletion of natural resources | Promote energy efficient and conservation practices e.g. - Switch off vehicles and moving equipment during periods of inactivity. - Reduce number of deliveries by combining loads where possible - Implement speed controls - An education program will be developed and delivered to the construction personnel to promote energy-efficient work practices. - Vehicle servicing and maintenance | Contractor MLAFWRD PIU | $500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Use of vehicles and machinery | -Use of fuel (diesel and petrol) | Depletion of natural resources | Promote energy efficient and conservation practices e.g. - Shutting down of vehicles and moving equipment during periods of inactivity. - Implement speed controls - Vehicle servicing and maintenance | MLAFWRD | $500 |
| DECOMMISSIONING PHASE |  |  |  |  |  |
| Demolition, transportation of removed equipment | -Use of fuel (diesel) -Use of lubricants and oil | Depletion of natural resources | - Shutting down of vehicles and mobile equipment during periods of inactivity. | Proponent | $0.00 |

Table 36 Air Quality Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Use of vehicles and machinery | Vehicle emissions | Air pollution | - Use of well serviced vehicles - Switch off vehicles when not in use - Enforce speed limits | MLAFWRD Contractor | $0.00 |
| Site preparation | Generation of dust | Air pollution | - Application of dust control measures such as reducing traffic movements and water sprinkling on exposed areas to reduce dust. - Land clearing, removal of topsoil and excess materials, location of haul roads, tips and stockpiles should be planned with due consideration to meteorological factors (e.g. precipitation, temperature, wind direction, and speed) and location of sensitive receptors. - Environmental dust emissions monitoring shall be done. | MLAFWRD IMC | $1500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Land preparation | Burning of crop residues, weeds, | Air pollution | - Avoid burning of crop residues - Consider using crop residues as animal feed | Farmers | $200.00 |
|  | Burning pesticides containers | Air pollution | - Prohibit burning of pesticide-treated agricultural wastes and by-products - Reduce the risk of fire by reducing the build-up of potential fuel sources and controlling weeds and invasive species - Provide training on the impacts of burning pesticides containers on human health | Farmers Agritex EMA | $200.00 |
| DECOMMISIONING PHASE |  |  |  |  |  |
| Use of mobile equipment | Vehicle emissions | Air pollution | - Use of well serviced vehicles - Switch off vehicles when not in use | MLAFWRD | $0.00 |
| Uprooting of irrigation equipment | Generation of dust | Air pollution | - Application of dust control measures such as reducing traffic movements and water sprinkling on exposed areas to reduce dust. - Removal of topsoil and excess materials, should be planned with due consideration to meteorological factors (e.g. precipitation, wind direction, and speed). - Environmental dust emissions monitoring shall be done. | MLAFWRD | $1000.00 |

Table 37 Cultural Heritage Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION, OPERATIONAL AND DECOMMISIONING PHASE |  |  |  |  |  |
| Land clearing and earthworks excavation | - Displacement or damage to cultural heritage sites by construction activities, harm to local setting, amenity value, -Change to intangible cultural heritage due to access, and interaction with workforce. | Loss of cultural and heritage artefacts | - Careful site selection and siting of all project components, taking account of community consultation/specialist surveys. - Implement the Chance Find Procedure if encounter any tangible forms of cultural or archaeological importance | Contractor | $0.00 |

Table 38: Community Health, Safety and Security Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Site preparation and construction activities | Poor construction management practices | Injuries and deaths or adverse effects on safety, human health and wellbeing. | - Good construction site “housekeeping” and management procedures (including site access). - Risk assessments and emergency response planning to consider impacts on local communities. | Contractor | $500.00 |
| Labour migration into the project area | Increased Interaction between workforce and local communities | Increased occurrence of communicable diseases, including HIV/AIDS and sexually transmitted diseases (STDs). | - Implementation of a health management system for the construction workforce, to ensure it is fit for work - Development of workplace policy on HIV & AIDS in line with the national policy and legislation and making it known to all. - Training and awareness raising for workforce on HIV/AIDS and other STDs, and communicable diseases; health awareness raising campaigns for communities on similar topics. - Promoting condom use and respect for sexual rights. - Provision of Voluntary Counselling and Testing and Treatment services. - Recreation facilities at workplace to encourage exercise. - Support groups. | Contractor MoHCC | $500.00 |
| Site preparation and construction activities | Noise generation | Noise induced hearing loss | - Noise is unlikely to be significant or noticeable if the construction vehicle or equipment are kept in optimal working conditions. - All equipment to be used in construction shall be regularly serviced. - Conduct personal noise monitoring to determine noise exposure levels and implement appropriate mitigation actions - Provide adequate PPE e.g. ear plugs | Site manager and contractor | $500.00 |
| Site preparation and construction activities | Generation of dust | Respiratory problems | - Conduct personal dust monitoring to determine respirable and inhalable dust levels - Implement speed control - Provision of adequate personal protective equipment | Site manager and contractor | $500.00 |
| Site preparation and construction activities | Awkward & prolonged postures | Musculoskeletal disorders | - There shall be planning and organizing of work and setting up laydown areas so that materials are handled, moved or carried for the minimum distances and number of times. - Setting up work so that it can be done above knees and below the shoulders. - Breaks or trade off with workers doing different jobs for repetitive or long duration jobs - Reduce the weight of a load to limit force exertion - Training on good ergonomic practices shall be done | Contractor | $500.00 |
| Site preparation and construction activities | Flying objects | Eye injuries | - Wetting down of surfaces to reduce dust emissions. - Awareness training on the risks associated with flying objects shall be provided - Safety goggles shall be provided. | Contractor | $300.00 |
| Site preparation and construction activities | Contact with person infected with a communicable diseases such as Covid-19, | Ill-health | - Ensure adhering to Covid-19 regulations - Testing of employees for Covid-19 shall be mandatory before construction works commence. - Temperature checks shall be a routine daily exercise for EVERYONE upon entrance at the project site. - Social distancing of > 1m apart shall be maintained | Contractor | $500.00 |
| Site preparation and construction activities | Open defecation (faeces and urine) | Spread of water borne diseases such as cholera and typhoid | - Provide training on water, sanitation and hygiene - Discourage employees from improper disposal of human waste. | Contractor | $1000.00 |
| Site preparation and construction activities | Harsh weather | Ill-health | - The Contractor must provide adequate cool drinking water, and encourage job rotations and well as work-rest regimen during hot summer weather conditions. - Adequate warm clothing shall be provided during cold winter weather conditions. | Contractor | $1000.00 |
| Site preparation and construction activities | Sharp/ rough objects | Injuries | - Appropriate PPE such as PVC gloves, hard hats, safety shoes, overalls shall be provided to prevent injuries from accidental contact with sharp objects. - hazard identification and risk assessments shall be carried out to determine existing hazards and putting adequate and appropriate controls. - Conduct daily site inspections - Enforce the use of PPE | Contractor | $2500.00 |
| Site preparation and construction activities | Trenching and digging | - Risk of buried hazards - Fall ins | - Conduct risk assessment of buried hazards - Barricading work sites - Implement an emergency preparedness and response plan - Conduct Safety, Health and Environment induction trainings as informed by the risk assessment study - Provide adequate PPE and enforce its use | Contractor | 500.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Crop Irrigation | Changes in exposure to water borne and water related diseases, especially those associated with water dwelling disease vectors (new areas of standing water created) or poor sanitary conditions. | Increased prevalence of waterborne diseases e.g. bilharzia, malaria, etc. | - Provide information, education and communication about safe uses of irrigation water and occupational safety. - Facilitate programmes/measures to ensure appropriate sanitary and medical facilities are available. - Implement environmental management measures for vector control: e.g. monitoring for key vectors; contact avoidance via site selection; focal insecticide and molluscicide application. | MoHCC | $500.00 |
| Irrigation | Noise generation | Noise induced hearing loss | - Noise is unlikely to be significant or noticeable if the water pump is kept in optimal working conditions. - Noise monitoring and testing shall be done on site to determine noise levels. If found to be above the maximum permissible limit of 90dB in an 8 hour shift then appropriate noise management solutions such as mentioned below are implemented. | IMC | $500.00 |
| Crop production manual activities | Awkward & prolonged postures, extensive gripping, overexertion, defective tools | Musculoskeletal disorders | - Provision of appropriate tools and equipment in good condition - Training of good ergonomic practices | Proponent and farmers | $1000.00 |
| Use of hazardous substances (pesticides and herbicides) | Inhalation, ingestion and contact with hazardous chemicals | Ill health | - The hazardous chemicals should be used as per manufacturer instructions - Farmers should be trained on how to use pesticides and herbicides | IMC | $500.00 |
| DECOMMISIONING PHASE |  |  |  |  |  |
| Demolition | Sharp objects/ rough materials | Injuries | - Provide demolition workers with adequate tools and PPE such as gloves, safety shoes, hard hats etc. | MLAFWRD |  |
|  | Awkward & prolonged postures, extensive gripping, overexertion, defective tools | Musculoskeletal disorders | - Provision of appropriate tools and equipment in good condition - Training of good ergonomic practices | MLAFWRD | $1000.00 |

Table 39: Labour Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Labour influx into the Project Implementation Area in search of employment opportunities | Recruitment of contract workers | Poor labour conditions | - Reduce labour influx by tapping into the local workforce; - Promotion of fair and equitable labour practices for the fair treatment, non-discrimination, equitable worker opportunities, minimum wage and prohibition of employment of children in line with the Employment Act; - Provision of Terms and Conditions of Employment which should be given to every employee which should include; working hours, date of commencement, rate of remuneration, intervals at which remuneration will be paid, normal working hours, provision of termination and disciplinary rules applicable to employees; - Establishment, management and promotion of a healthy, management-worker relationship; - Promotion of health, safe, secure and comfortable accommodation that does not impact negatively on the employees and the communities in the surrounding areas; and - Protection of workers’ rights including migrants and third-party workers in line with Labour Laws | - Contractor - MAWLRD | $500.00 |

Table 40: Child Safety Management Plan

| Activity | Aspect | Impact | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Labour influx into the Project Implementation Area in search of employment opportunities | Child labor and Child abuse | Child safety risks. e.g. SEA, accidental harm, physical abuse, Psychological / emotional Abuse and online abuse. | - Conduct risk assessment of all construction activities and identify risks management options - Child safety Training - Reference Checking - Pre-screening interviews - Criminal History checks - Working with children checks - Probation period - Code of Conduct - Training for all staff, volunteers, leaders etc. - Grievance procedures in place - First Aid kit must be readily available on site | - Contractor - MAWLRD | $500.00 |

Table 41 Gender based violence management plan

| Form of GBV | Social Impact | Mitigation/Control | Responsibility | Estimated cost $USD |
| --- | --- | --- | --- | --- |
| Construction, Operation and Demolition |  |  |  |  |
| Physical violence (such as slapping, kicking, hitting, or the use of weapons) | Physical health -Acute injuries - Pain -Bruises -Scars Mental health -Depression - Anxiety -Trauma -Suicide -Discrimination -Social stigma -Rejection | - Awareness training on the issues associated with gender based violence and sexual exploitation shall be provided with the help of ZRP. - Formulation and operation of project disciplinary committee. - Empower women and girl child with more responsibilities in the irrigation scheme. - Address grievances at an early stage. - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions - Report any act of gender based violence to project disciplinary commit and to the police. - Investigation of GBV cases reported. - Arresting perpetrators involved in gender-based violence. | Contractor Victim, Project Disciplinary committee ZRP | $1000 |
| Emotional abuse (such as systematic humiliation, controlling behaviour, degrading treatment, insults, and threats) | Mental health -Depression - Anxiety -Trauma -Suicide -Discrimination -Social stigma -Rejection | - Incorporate law enforcer in all steps of the project. - Ongoing monitoring to determine any gender-based issues. - Report any act of gender-based violence to project disciplinary commit and to the police. - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions | Victim, Project Disciplinary committee ZRP | $1000 |
| Sexual violence, which includes any form of non-consensual sexual contact, rape. | Sexual reproductive -Sexually transmitted infections -Unwanted pregnancy -Induced abortions Mental health -Depression, Anxiety, Trauma, Suicide, Discrimination, Social stigma, Rejection | - Implement the Code of Conduct on SEA and GBV - Incorporate law enforcer in all steps of the project. - Ongoing monitoring to determine any sexual abuse cases. - Reporting of any act of sexual abuse to project disciplinary commit - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions - Investigation of sexual abuse cases reported. - Arresting perpetrators involved in sexual abuse crimes | Victim, Project Disciplinary committee ZRP | $1000 |
| Early/forced marriage | Mental health -Depression, Anxiety, Trauma, Suicide, Discrimination, Social stigma, Rejection | - Awareness training on the issues associated with gender-based violence and sexual exploitation shall be provided with the help of ZRP. - Reporting of any acts of early child marriages to project disciplinary commit and to the police. - Give support and protection against discrimination | Victim, Project Disciplinary committee ZRP Court | $1000 |
| Economic abuse and the denial of resources, services, and opportunities (e.g. restricting access to financial, health, educational, or other resources) | Financial wellbeing -Reduced income -Lack of control of income and resources | - Provision of fair remuneration to workers - Provision of services such as health and education to all - Reporting of human rights abuse to project disciplinary commit and to the police. | Victim, Project Disciplinary committee | $1000 |
| Trafficking and abduction for exploitation | Sexual reproductive -Sexually transmitted infections -Unwanted pregnancy -Induced abortions | - Incorporate law enforcer in all steps of the project. - Ongoing monitoring to determine any human trafficking and abduction cases. - Reporting of any human trafficking and abduction to project disciplinary commit and to the police. - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions - Arresting and prosecution of perpetrators involved in human trafficking and abduction crimes | Victim, Project Disciplinary committee ZRP Court | 1000 |

Table 42 Sexual exploitation, abuse and harassment management plan

| Form of SEAH | Social Impact | Mitigation/Control | Responsibility | Estimated cost |
| --- | --- | --- | --- | --- |
| Construction, Operation and Decommissioning |  |  |  |  |
| Sexual exploitation | Increased risk of rape, sexual assault, physical violence, sexually transmitted infections, trafficking and pressure to use drugs and alcohol. Mental health Depression Anxiety Trauma Suicide Discrimination Social stigma Rejection | - Implement the code of conduct on SEAH - Awareness training on the issues associated with sexual exploitation shall be provided with the help of Ministry of Woman Affairs. - Formulation and operation of project disciplinary committee. - Empower women and girl child with more responsibilities in the irrigation scheme. - Address grievances at an early stage. - Attend counselling and support group sessions - Report any act of sexual exploitation to project disciplinary commit and to the police. - Investigation any sexual exploitation cases reported. - Arresting and prosecuting perpetrators involved in gender based violence. | Contractor Victim, Project Disciplinary committee Ministry of Woman Affairs ZRP | $5000 |
| Sexual abuse | Physical health Acute injuries Pain Bruises Scars Mental health Depression Anxiety Trauma Suicide Discrimination Social stigma Rejection | - Incorporate law enforcer in all steps of the project. - Monitoring to determine any gender based issues. - Report any act of sexual abuse to project disciplinary commit and to the police. - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions | Victim, Project Disciplinary committee ZRP Court |  |
| Sexual harassment eg attempted or actual sexual assault, including rape; sharing or displaying sexually inappropriate images or videos in any format; sending sexually suggestive communications in any format; sharing sexual or lewd anecdotes or jokes; making inappropriate sexual gestures, such as pelvic thrusts; unwelcome touching, including pinching, patting, rubbing. | Sexual reproductive Sexually transmitted infections Unwanted pregnancy Induced abortions Mental health Depression Anxiety Trauma Suicide Discrimination Social stigma Rejection | - Implement the code of conduct on SEAH - Incorporate law enforcer in all steps of the project. - Monitoring to determine any sexual abuse cases. - Reporting of any act of sexual abuse to project disciplinary commit and to the police. - Seek medical help from health care givers from nearest clinics - Attend counselling and support group sessions - Give support and protection against discrimination - Investigation of sexual abuse cases reported. - Arresting perpetrators involved in sexual abuse crimes | Victim, Project Disciplinary committee ZRP Court |  |

Table 43 COVID-19 management plan

|  | Impact | Mitigation measure | Responsibility | Estimated cost (USD) |
| --- | --- | --- | --- | --- |
| Construction, Operational and decommissioning phase |  |  |  |  |
| Infected with COVID-19 | -Straining of medical local medical facilities -Loss of human life (death) -Upset in the demand/supply of farming products. -Significant slowing down in the income growth for the community. -Anxiety, depression, stress, loneliness and depression during quarantine. | - Practising social distancing. - improved personal hygiene. - Wearing of face masks - Daily temperature screening - Providing COVID-19 awareness training to contractors and famers - Vaccination. | Project manager Local clinics Farmers Contractors | $1000 |

Table 44: Emergency Preparedness and Response Plan

| Emergency | Risk | Mitigation Measures | Responsibility | Estimated Cost (USD) |
| --- | --- | --- | --- | --- |
| Dam collapse or breach/ flooding Piping/Seepage failure/ increase amounts of seepage Widespread flooding Dam Slope failure/ movement of dam crest | Death/injury of farmers Destruction of the irrigation equipment. Destruction of crops Death of animals | Preparedness - Irrigation members, community and workers shall be made aware of the risk through training. - Emergency contact list shall be posted onsite - Monthly dam assessment by the irrigation management committee. ( tacking photos) - Dam water level monitoring - Irrigation equipment and infrastructure will be installed outside the floodplain. - Introduce early warning systems - Periodic observation of seepage flow - Periodic recording of reservoir water levels - Spillway inspection to make sure capacity is not compromised - Ensure maintenance of embankments to prevent tree growth - Define safe zones in case of evacuation - Continue at a reduced level until repairs can be made. Response - Notifying the Civil Protection Unit - Initiating physical works to reduce the risk - Community should move to higher ground (low risk zone) in case of flooding or dam breach. - Limit access to the area - Mobilize the emergency response team - Take action to lower water level in the reservoir by placing filter/rockfill against slope. - Arrange for temporary rectification/protection works e.g. placing filter/rock fill over the seepage area - In case of evacuation, the management committee shall identify access routes to the dam which are compatible during periods of darkness, adverse weather conditions and transport disruptions. - The incident shall be recorded and kept for future references and record keeping | - Irrigation management committee - Farmers - ZINWA - MSD | 2000 |
| Hazardous substances spill | Water pollution Animal poisoning Reduced crop productivity | - Notify the emergency committee - Report observed abnormalities in water eg colour and odour - Conduct water quality sampling - Isolate reservoir to prevent inflows and outflows - Notify the Environmental Management Agency, ZRP, District Development Coordinator as stipulated by Environmental Management - Arrange for remediation operations e.g. removing contaminated soil. - The incident shall be recorded and kept for future references and record keeping | - Contractor - Irrigation management committee - Environmental Management Agency - ZINWA |  |
| Fire | Death/injury of farmers Destruction of crops Destruction of the irrigation scheme. Death of animals | - Burning will not be utilised as a waste/land management practice - Fire guards (9m wide) will be constructed around the scheme. - Educate all farmers and workers on potential causes of fire and how to extinguish fire - Use of fire beaters | Irrigation management committee Farmers | 500 |

Table 45 Stakeholder Engagement Plan

| Stakeholder group | Stakeholder | Stakeholder interest | Participation Method | Responsibility | Costs (USD) |
| --- | --- | --- | --- | --- | --- |
| Core decision makers | UNDP/ GCF | Project funding | Document Review and validation Meetings Field Visits | ESIA Team Leader |  |
|  | Ministry of Lands, Agriculture, Water, Climate and Rural Resettlement | Provision of technical, extension, advisory, regulatory, and administrative services | Interview Key Informant Questionnaire Meeting | Social Scientists |  |
|  | Department of irrigation | Provision of agricultural engineering, mechanization, and irrigation services | Interview Key Informant Questionnaire Meeting | Social Scientists |  |
|  | ZINWA | Water resources management | Key Informant Interview | Social Scientist | $500.00 |
|  | Ministry of Environment, Climate, Tourism and Hospitality Industry | Wildlife protection | Key Informant Interview | Social Scientist |  |
|  | Environmental Management Agency | Land and Air quality management | Key Informant Interview | Social Scientist | - |
|  | Matobo Rural District Council | Regulatory authority on Land use management | Key Informant Interview | Social Scientist | 500.00 |
|  | Contractors and consultants | Installation and maintenance of the irrigation scheme | Stakeholder Meetings | Social Scientist | - |
| Directly affected group | Irrigation scheme project members | Irrigation scheme employees | Stakeholder Meetings | Social Scientist | - |
|  | Project workers/ representatives | Employment | Stakeholder Meetings | Social Scientist | - |
| Others | AGRITEX | Agricultural Land Management | Key Informant Interview Stakeholder Meeting | Social Scientist |  |
|  | Meteorological services department | Advisory on matters pertaining to meteorology, climate, and seismology | Key Informant Interview | Social Scientist |  |
|  | Forestry department | Sustainable utilization of forest resources | Key Informant Interview Stakeholder Meeting | Social Scientist | $30.00 |
|  | Parks and wildlife | Wildlife protection | Key Informant Interview | Social Scientist | - |
|  | Councilor | Consultation on community issues | Key Informant Interview Stakeholder Meeting | Social Scientist | - |
|  | District Development Coordinator (DDC) | Development coordination | Key Informant Interview | Social Scientist | $500.00 |
|  | ZRP | Law and order maintenance | Key Informant Interview | Social Scientist | - |
|  | Ministry of Health and Child Care | Public Health | Key Informant Interview | Social Scientist | - |
|  | Farmers Associations | Coordinating farming programs within the area | Key Informant Interview | Social Scientist | - |
|  | Ministry of Women Affairs, Community, Small and Medium Enterprises | Gender and community issues | Key Informant interview | Social Scientist | - |
|  | Chief | Consultation on community issues | Key Informant Interview Stakeholder Meeting | Social Scientist | - |
|  | Village headman | Consultation on community issues | Key Informant Interview Stakeholder Meeting | Social Scientist | - |
|  | Community members | Employment opportunities | Stakeholder Meeting | Social Scientist | - |

Table 46: Stakeholder Engagement Plan implementation schedule

| Objective of Stakeholder Engagement | Engagement method | Location | Timeline | Stakeholders | Output | Responsibility |
| --- | --- | --- | --- | --- | --- | --- |
| To disseminate information about the project concept and information on the methodology and Irrigation technology The creation of a road map for implementation and a shared knowledge of the project's goals, roles, and duties among the stakeholders. | Telephone Interviews and Face to Face meetings with Stakeholders | Matobo | March 2022 | - Department of irrigation - Provincial Irrigation Engineer - ZINWA – Catchment Manager and Hydrologist - AGRITEX - Officer - Forestry Commission - Officer - Environmental Management Agency – Officer - Matobo Rural District Council – Natural Resources Officer and CEO - Contractors and consultants - Village Head / Chief - District Development Coordinator - Ministry of Women Affairs, Community, Small and Medium Enterprises - ZRP | Meeting Minutes | Project Team Leader and ESIA Team Leader |
| To gather information on socioeconomic and environmental baseline assessments by involving the directly affected stakeholders and Interested Parties. | Key Informants Interviews Field visit (Transect Walks) Focus Group Discussions Questionnaires | Matobo | March 2022 | - ZINWA - Department of irrigation - Irrigation scheme project members - Project workers/ representatives - AGRITEX - Forestry department - Parks and wildlife - Councillor - District Development Coordinator (DDC) - ZRP - Ministry of Health and Child Care - Farmers Associations - Ministry of Women Affairs, Community, Small and Medium Enterprises - Councillor - Village headman - Community members - Ministry of Environment, Climate, Tourism and Hospitality Industry | Minutes Completed Questionnaires Baseline reports | Project team Leader and ESIA Team Leader |
| Draft Baseline assessment findings, management Plans. Irrigation technology and Crops | Meetings and Emails | Matobo | March 2022 | - Department of irrigation - AGRITEX - Irrigation scheme members - Consultants | Draft ESMP | ESIA Team Leader and Project Team Leader |
| Key Stakeholders to review Draft Environmental and Social Management Plans, project information and Irrigation Technology | Meeting Emails Field visit | Matobo | April 2022 | - UNDP - Department of irrigation- Provincial Engineer - AGRITEX – Provincial Agronomist and Officers - Irrigation scheme members - Consultants | Reviewed ESMP | ESIA Team Leader and Project Team Leader |
| To deliberate on the suitable irrigation design | Key Informants interviews and Emails Field Visit | Matobo | May 2022 | - Department of irrigation – Provincial Engineer - AGRITEX - Provincial Agronomist - Irrigation scheme members - Consultants | Draft Irrigation Design | Project Engineer |
| Approval of the ESMP Documents, Crops, and Irrigation Technology | Validation Meeting | Matobo | June 2022 | - UNDP - Contractors and Consultants - Environmental Management Agency - ZINWA - Department of irrigation - Irrigation scheme project members - Project workers/ representatives - AGRITEX - Forestry department - Parks and wildlife - Councillor - District Development Coordinator (DDC) - ZRP - Ministry of Health and Child Care - Farmers Associations - Ministry of Women Affairs, Community, Small and Medium Enterprises - Meteorological services department - Councillor - Village headman Community members | Approved ESMP | Project team Leader |
| Sharing of Project Information and ESMP with contractors | Meeting Emails | Harare | Year 1 to Year 2 | - Contractors and Consultants - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Minutes Attendance register | UNDP PMU and Consultant Project Team Leader |
| Deliver ESMP, Grievance redress mechanism training to the Irrigation scheme project members using the approved guidelines. | Stakeholder Meeting | Matobo | Year 2 | - UNDP - Contractors - Irrigation scheme Management committee - Community Members - Project workers/ representatives - AGRITEX - Department of Irrigation - ZRP - Village head - Councillor - Community Development Officer – Ministry of Women Affairs - District Development Coordinator | Training Register and Minutes | Project Manager UNDP PMU |
| Introducing Irrigation Construction contractor to Stakeholders and Site Appreciation | Field Visits Meeting | Matobo | Year 2 | - UNDP - Contractors - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Minutes Attendance register | Project Manager UNDP PMU |
| Deliver trainings through seminars to the Irrigation scheme project members using the approved guidelines. | Stakeholder Meeting | Matobo | Year 2 – 6 | - UNDP - Contractors - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Training Register and Minutes | Project Engineer |
| Monitoring progress during construction of the Irrigation project | Field visits Meetings | Matobo | Year 2 | - UNDP - Contractors - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Minutes and Progress reports | Project Manager UNDP PMU |
| Commissioning of the Project | Field Visits | Matobo | Year 2 | - UNDP - Contractors - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Project Documentation | Project Engineer |
| Convene regular meetings, offline and online, for sharing knowledge and best practices on the implementation of the project. | Field Visits | Matobo | Entire Duration of the Project (Quarterly) | - UNDP - Contractors and Consultants - Irrigation scheme project members - Project workers/ representatives - Department of irrigation - AGRITEX | Project Progress Reports | Project Team Leader |

## 8.3 Stakeholder Engagement Fees

The breakdown of Stakeholder engagement Fees allocated for implementation of the stakeholder engagement plan is provided in Table 47

Table 47 Stakeholder Engagement Activities

| Stakeholder Engagement Activities | Amount (USD) |
| --- | --- |
| Conducting of socio-economic assessment and Environmental Baselines | 7,000 |
| Travel expenses | 4,000 |
| Communication materials (audio, visual and print production costs) | 2,000 |
| Total | 13,000 |

## 8.4 Monitoring and Evaluation

Throughout the project's duration, the stakeholder engagement plan's implementation will be monitored and assessed. Details about stakeholder engagement will be included in project reports, meeting minutes, and other knowledge outputs. Adaptive management strategies will be implemented as necessary to modify the plan considering the results of monitoring and evaluation activities. The project's monitoring and evaluation plan (M&E) includes contributions from co-financing as well as monitoring and evaluating how the stakeholder engagement plan is being implemented.

# 9.0 ENVIRONMENTAL AND SOCIAL MONITORING PLAN

## 9.1 Monitoring Responsibilities and Reporting

Monitoring the identified impacts is one of the important elements of this ESMP and serves a number of functions. It is the instrument that will determine whether the environmental mitigation measures are fulfilling their intended purpose or not. Environmental monitoring will help to provide the basis for rational decision-making regarding impact control. Information collected during the monitoring exercise will help to improve environmental management plans by adapting measures to ensure that the identified environmental and social issues are mitigated. If environmental monitoring identifies some environmental concerns during scheme revitalization, then the activities will be modified or adjusted, accordingly. The overall aim of this environmental and social monitoring is to qualitatively and quantitatively measure effectiveness of mitigation measures, and develop appropriate responses to incompliances with Project standards, and emerging environmental and social issues.

## 9.2 Monitoring Program

Basically, monitoring of the irrigation schemes will involve physical measurement of selected water and soil parameters before and after climate proofing and revitalization project to establish the nature and extent of induced changes.

The contractor will ensure that the monitoring programs comprise the following:

- A formal procedure;

- Appropriately calibrated irrigation equipment where equipment is required;

- Where samples require laboratory analysis, they will be preserved according to laboratory specifications;

- An accredited, independent, commercial laboratory will undertake sample analyses (soil and water);

- Parameters to be monitored will be identified in consultation with a specialist in the field and/or the relevant authority and in line with relevant guidelines/licenses/permits;

- If necessary, following the initial monitoring results, certain parameters may be removed from the monitoring program in consultation with a specialist and/or the relevant authority;

- Monitoring data will be stored in a structured database;

- Both the data and the reports will be kept on record for the life of the projects.  As a general comment, if monitoring points become damaged or redundant then they can be replaced with new points with the input of an appropriately qualified personnel.

- Environmental monitoring data will be maintained in an electronic data base and Geographical Information system (GIS). This will be the key management tool to monitor historical and current environmental performance as well as environmental compliance.

The environmental monitoring plan (Table 48) and Social Management plan (Table 49) indicate key issues that will be monitored during construction, operation and decommissioning of the scheme. The monitoring plan also includes the type of monitoring indicators, frequency of monitoring and responsibility for each monitoring activities.

Table 48: Environmental Monitoring Plan

| Environmental Issue | Monitoring Parameters | Frequency | Monitoring Site | Means of verification | Responsibility | Cost (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| PRE-CONSTRUCTION PHASE |  |  |  |  |  |  |
| Vegetation and fauna | - Vegetation intact at inactive sites of project area | Daily | -Project area | Observation | Proponent | $100.00 |
| Air quality/Noise | -Observation of air borne particulates (dust) and exhaust fumes -Complaints on noise nuisance | Daily | -Around project area | Observation | Proponent, EMA, MRDC | $500.00 |
| Surface water | -Observable change in turbidity of water in drains/water bodies | Daily | -Canals/drains/ water bodies | Observation | Proponent, EMA, MRDC | $100.00 |
| Waste management | -Records on tree and shrub stems collected by communities -Availability and use of bins -Records on frequency and location of waste disposal sites of domestic and construction waste | Monthly | Construction site | Record keeping and analysis | Proponent, EMA, MRDC | $100.00 |
| Water quality | pH, Electrical Conductivity, Dissolved Oxygen, Total Dissolved Solids, turbidity, Total Nitrogen, Total Phosphorus, oil | Monthly (except TN, TP) Biannually (TN, TP) | Same points as the baseline survey | Laboratory analysis | Proponent | $1000.00 |
| OPERATIONAL PHASE |  |  |  |  |  |  |
| Soil fertility and Quality | Laboratory analysis of soil sample parameters include texture; pH; organic carbon; total nitrogen; available phosphorus; available potassium; exchangeable cations like Ca, Mg, Na, K; - Record of integrated weed and pest management practices implemented; - Record of type and quantity of agrochemicals used by farmers | Biannually | Farms | -Laboratory analyses | Site Manager, EMA | $2000.00 |
| Water quality Degradation | -Physicochemical parameters -Bacteriological parameters -Pesticide residue parameters Groundwater - pH, conductivity, TDS, chloride, iron, nitrate, coliforms, phosphate, pesticides, fluorides, heavy metals (leads, arsenic, cadmium, mercury, etc.), pesticide loads | Biannually (different seasons) | Upstream, midpoint and downstream of project are | Laboratory analyses | Proponent | $2000.00 |
| Air quality | TSP, PM10, NOx, SOx, CO | Quarterly | Project site/project communities | Hand-held dust samplers (TSP, PM10)/ diffusion tubes (NOx, SOx, CO) | Proponent | $1000.00 |
| Noise | Noise level | Bi annually | Project site | Hand-held noise level meter | Proponent, EMA, MRDC | $1000.00 |
| Waste management | -Availability and use of bins for collection of plastic and polythene material. -Availability and use of separate labelled bins for agrochemical containers. -Records on disposal of plastic and polythene material (frequency and location of disposal site) -Records on agrochemical containers returned to suppliers (quantity, frequency, name of supplier) | Monthly | Farms | Record keeping and analysis | Proponent, EMA, MRDC | $200.00 |
| Revitalization of water pipes | Water flow | Twice a year during wet and dry season | Entire scheme including the pipes from the dam area | Physical observations | Proponent | $1000.00 |
| Control over use of pesticides and herbicides in the scheme | Water Quality | On quarterly basis | At the intake site Outlet discharge points for drained water | Monitoring Inspection Reports | Contractor’ Agritex and EMA | $500.00 |
| Proper management of center pi vot irrigation pipes | Water Quality and pipe condition | On quarterly basis | At the intake site Outlet discharge points | Flow Records | Proponent | $1000.00 |
| Efficient agro-chemical application technology | Crop pests and Diseases | Monthly | Crop fields/plots | Physical observations | Proponent | $100.00 |
| Surface and Ground water | Water quantity level | Twice a year during onset of cool dry and end hot dry season season | Pump station and borehole drill hole point | Physical observations using monitoring sensors | ZINWA | $2000.00 |

Table 49: Social Monitoring Plan

| Impacts/Issues | Mitigation Measure(s) | Monitoring Indicator(s) | Monitoring Frequency/ Methods | Responsible Parties | Estimated cost $USD |
| --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |
| Conflict with other non-irrigation scheme members over land | Dialogue with non-beneficiary irrigation scheme community members in which they are informed of procedures for becoming a member and the transparency behind the process | Broad community knowledge on irrigation scheme membership criteria and joining requirements | Community meetings | IMC MLAFWRD Traditional leadership | $500.00 |
|  | Clear defined boundary fence that demarcates the irrigation scheme area | Demarcation of land use between communal rain fed irrigation and formal irrigation scheme plots | Periodic checks to ensure the livestock have not compromised the security fence | IMC | $10000.00 |
|  | Use of a grievance redress mechanism to assist communities to raise any issues that may arise | Grievances are presented using the grievance redress mechanism | As and when grievances are presented | MLAFWRD IMC Traditional leadership | $2000.00 |
| Gender based violence | Raise awareness and use grievance redress mechanism to assist victims | Number of gender based violence cases reported | When cases are reported | IMC Traditional leadership | $200.00 |
| OPERATIONAL PHASE |  |  |  |  |  |
| Conflict with non-irrigation scheme members over water for livestock and domestic uses | Ensuring water abstraction in the dry season leaves sufficient water for the environment downstream | Ground water levels tested weekly in the dry season and bi-weekly in the rain season | Weekly in the dry season, monthly in the rain season | ZINWA IMC DoI | $5000.00 |
|  | Use of a grievance redress mechanism to address any issues that may arise that could escalate beyond verbal conflict | Sufficient dialogue between upstream and downstream water users to engage in dialogue on water demands and equity in the dry season | Monthly | IMC Traditional leadership MLAFWRD | $500.00 |
| Reduced base water flow resulting in affected downstream ecosystems and riverine vegetation | Ensuring there is sufficient water for the environment downstream when water abstraction is underway in both dry and rain seasons | Levels of water downstream of the irrigations scheme pump area of influence | Weekly in the dry season, monthly in the rain season | ZINWA IMC DoI | $500.00 |
| Gender based violence | Raise awareness and use grievance redress mechanism to assist victims | Number of gender based violence cases reported | When cases are reported | IMC Traditional leadership | $500.00 |
| DECOMMISIONING PHASE |  |  |  |  |  |
| Reduced river water quality downstream due to long term pollution by pesticides and fertilizer used at the irrigation scheme | Controlled use and application of synthetic fertilizers for increasing land productivity | Water quality downstream of the irrigation scheme | Monthly in the dry season, bi-weekly in the rain season | IMC AGRITEX | $3000.00 |
|  | Promote use of organic fertilizers and crop rotation as well as manure and compost as alternative and accessible means of improving soil fertility and texture | Increased uptake of organic fertilizer use by irrigation scheme members | Quarterly and when crops are rotated | IMC AGRTEX | $2000.00 |

# 10.0 CAPACITY DEVELOPMENT AND TRAINING

## 10.1 Introduction

Smallholder farmers in the southern Zimbabwe mostly women in the rural areas are facing the increasing climate risks and impacts.  Rainfall received in this region is very erratic and as a result smallholder famers are confronted with food shortages and water supply. The management of scarce resources such as water becomes important. Understanding the fundamental principles by which resources are managed and the relationships between people, governing institutions and the environment is central to the development of viable approaches to natural resources management within river catchments/basins in these dry regions.

The management of irrigation infrastructure generally requires skills from government agencies, including a shift from direct management of irrigation systems to regulation of the irrigation and water sectors, capacity building and provision of support services in cost sharing, technical advice, management training, and dispute resolution for the smallholder farmers.

However, capacity development for irrigated agriculture has always suffered from lack of support particularly agricultural extension officer who normally offers training services to the smallholder famers in the rural areas.  Extension services are affected by lack of transport (motor bikes) which enable the Agritex officers to visit sites and carry out regular training services. Capacity development initiatives can only be successful with the assistance or involvement of government agencies, politicians and donors. More importantly the capacity building of smallholder farmers should be in the portfolio of Ministry of Lands, Agriculture, Fisheries, Water and Rural Development. The Ministry should come up with extensions programs geared at offering trainings and visits for evaluation and monitoring. The major constraints facing smallholder irrigation farmers include shortage of water as well as market availability and unpredictability both locally and abroad.

## 10.2 Capacities Required

The capacities that need development range from effective communication skills, through the application of advanced technical techniques (center pivot irrigation and moisture monitoring), to the development of analytical skills and decision-making. Capacity building efforts should not only target rural smallholder farmers for skills development but other target groups who use same water resources, technicians, technical service managers.

Areas that need capacity building through training include the following;

- Identification, adaptation and promotion of new, climate smart irrigation technologies

- Water reforms, i.e., strategies for improving irrigation water management through the involvement of other community water users for the resource management, distribution and equitable sharing

- Chemical fertilizer application and its effects. This type of fertilizer does not improve soil texture and it’s expensive for most of the poor farmers. Farmers can be trained on the benefits of using compost manure and appropriate planting against salinity problems.

- Planting density in case different crop types

- Sustainable pest control methods

- Achieving food security through prioritizing crop varieties adapted to agro-ecological zone

- Marketable crops and market access

Table 50 provides the training and capacity building plan

## 10.3 Objectives of the Capacity Building and Training

The main objective of the capacity building and training activities is to create, enhance and develop the necessary skills and abilities for successful implementation of the proposed project.

## 10.4 Methodology for the Capacity Building and Training

The methodology to implement the capacity building and training will include:

- workshops and site meetings;

- field demonstrations and trainings;

- Community sensitization.

Table 50: Capacity building and training plan

| Training | Target group | Training Phase |
| --- | --- | --- |
| Grievance Redress Mechanism, | -Construction Contractors, Beneficiaries, Responsible parties, and other stakeholders | Project design phase and prior to construction works |
| Sexual Exploitation, Abuse and Harassment | -Construction Contractor Responsible parties | Prior to construction works |
| ESMP implementation | -Responsible parties -Contractor | Construction phase |
| Irrigation Scheme Management | Farmers and IMC | Construction |
| Health and Safety Induction | Contractor workers Visitors | Construction Phase |
| HIV and AIDS awareness training | Farmers and contractors | Construction and Operational phase |
| Field demonstrations trainings on farming practices | Farmers | Operational stage |
| Pfumvudza / intwasa maize farming | Farmers | Operational Stage |
| Contractor Management | Contractors | Construction and operational stage |
| Use and management of the irrigation equipment | Farmers | Operational stage |
| Integrated Pest Management | Farmers | Operational Stage |
| Safe Use and handling of agrochemicals | Farmers | Operational stage |
| Water Management | Farmers | Operational stage |
| Road Safety | PMU & Responsible Parties | Construction and Operation |
| Waste management | Farmers | Construction, operational and decommissioning phase |
| Emergency Preparedness and Response | Contractors Farmers | Construction and Operational phase |
| Gender based violence | Farmers and contractors | Construction and operational stage |
| Sensitisation workshops on SEAH | Farmers | Preconstruction, Construction & Operational stage |

# 11.0 ESMP COST ESTIMATES

## 11.0 Estimated project cost

The estimated cost of revitalizing the scheme is approximately USD 154 150.00.

## 11.2 Estimated cost for implementing the environmental and social management plan

Table 51: Environment and social management plan implementation cost

| Management Plan | Estimated cost (USD) |
| --- | --- |
| Land management | $20 000 |
| Waste management | $5 000 |
| Water management | $17 000 |
| Hazardous substance | $2 400 |
| Biodiversity | $12 000 |
| Energy | $2 000 |
| Air quality | $ 5400 |
| Health and safety | $20 000 |
| Gender based violence | $12 000 |
| Sexual Exploitation, Abuse and Harassment | $10 000 |
| HIV and AIDS | $10 000 |
| COVID-19 | $2 000 |
| Total | $120 000 |

# 12.0 GRIEVANCE REDRESS MECHANISM

## 12.1 Introduction

An individual or group people may be negatively affected by the construction and implementation phases of the irrigation scheme revitalisation. Therefore, a comprehensive grievance redress mechanism is incorporated in this ESMP to ensure that conflicts that may arise due to social issues (Sexual Exploitation and Abuse, Sexual harassment, Gender based violence, land use conflicts, water allocation conflicts, crop damage conflicts, etc.) and/or environmental issues (decrease in water quality/quantity, air pollution, noise, etc.) are resolved in a free, fair, transparent, timely, efficient and economical manner. This grievance redress mechanism was documented to allow anyone that have a complaint or feel aggrieved by irrigation project social or environmental issues to raise their concerns in an appropriate and acceptable process.

The two main categories of grievances to be addressed are:

- Non-sensitive - Relate directly to the project as it is being implemented by UNDP and its partners. It usually involves grievances related to environmental pollution, health and safety issues, land disputes etc.

- Sensitive – Require more complex investigation and usually concern either Sexual Exploitation and Abuse (SEA) or Gender Based Violence (GBV). The investigation of suspected cases is regulated in the government’s policy and procedures, and UNDP’s related implementation guidelines.

Although most complaints or grievances can be quickly and amicably resolved, it is importance to log them in the complaints register and ensure mutually acceptable resolutions as per Grievance Redress Mechanism in this ESMP. The Grievance Redress Mechanisms is designed to:

- Encourage resolution of grievances or complaints by informal or formal means.

- Be easily accessible and well publicized

- Be simple to understand and use

- Be impartial and non-adversarial

- Allow swift handling within reasonable time limits and keep people informed of progress

- Ensure full, fair and transparent investigation by an independent committee where necessary

- Respect people’s desire for confidentiality

- Address all the points at issue and provide an effective response and appropriate response where necessary.

- Provide information to the MLAFWRD so that the Grievance Redress Mechanism can be improved

At every stage the Grievance Management Committee must make sure that they:

- Establish what has happened and who was involved and what remains

- Clarify the nature of complaint or grievance and what remains unresolved

- Meet with the complainant or aggrieved or contact them

- Clarify what the complainant feels would put things right

- Interview those involved in the matter and/or those complained of, allowing them to be accompanied if they wish

- Conduct the interview with a free mind and be prepared to persist in the questioning

- Keep notes on discussion and outcomes. Most response can be resolved satisfactorily by informal discussion either over the telephone or through a meeting involving the key people involved.

Exclusions

Not all complaints shall be handled through the project GRM. For instance, grievances that allege corruption, coercion, obtaining employment or major violations of rights and/or policies, are shall be referred to UNDP accountability mechanisms or judicial bodies for formal investigation, rather than to GRM, which is designed for collaborative problem solving

## 12.2 The Project GRM

The Project will set up a Grievance Redress Mechanism (GRM) on traditional conflict-resolution flows as well as administrative and project based steps to ensure community members or any stakeholders have the opportunity and means to raise their concerns, complaints and suggestions. A four tier redressal structure is proposed to address complaints that may emanate from the Project. It represents different stakeholders at the various levels of the conflict resolution process.

First tier of Redress: - Local/Community level GRC

Local/community level Grievance Redress Committees (GRCs) shall be established in each proximate cluster of beneficiary communities and shall comprise of nominated members representing a well spread out demography, including women, girls and youth. The members of the GRC shall be selected following a participatory process. Local interest groups such as relevant Civil Society Organisations (CSOs), community leaders and reputable community associations shall also participate in the selection of the GRC members, or be a part of such committee. A representative of the local traditional authority shall be included as a key member of the GRC.

The community-based GRCs through its secretary shall receive and register grievances and submit the same grievance(s) to the PMU through the Grievance Submission Form. Apart from registering grievance, the committee shall seek to resolve grievances at this level with the support of the PMU when required. However, grievances that are beyond the capacity of the Local GRC to resolve shall be escalated or referred to the District/Province GRC and subsequently the PMU or other appropriate organs such as the Police or the Judiciary.

Second tier of Redress: District level GRC

In a case that a grievance is not resolved at community level GRC to the satisfaction of the complainant, the grievance may be referred to the next tier of redressal, which is at the District level. The GRM focal person at district level and the Safeguards Officer will coordinate with the relevant government departments, in nominating the Committee members depending on the nature of the complaint. The necessary circulars will be issued so that the committee could convene whenever required. The complainant may decide to take the matter to the next level of grievance redress or a legal or any other recourse if s/he is not satisfied with the resolutions at lower levels of the GRM.

Third tier of Redress: Provincial level GRC

In a case that a grievance is not resolved at District level GRC to the satisfaction of the complainant, the grievance may be referred to the next tier of redressal, which is at the Provincial level. The GRM focal person at Provincial level and the Safeguards Officer will coordinate with the relevant government departments, in nominating the Committee members depending on the nature of the complaint. The necessary circulars will be issued so that the committee could convene whenever required. The complainant may decide to take the matter to the next level of grievance redress or a legal or any other recourse if s/he is not satisfied with the resolutions at lower levels of the GRM.

Fourth Tier of Redress: Project focal point

The Project focal point will act as the Apex  to hear and adjudicate on appeals against all other GRM Committee decisions. Pending cases shall be presented by the Project focal point of each of the Subcommittees to the PSC. The Project focal point will establish an adhoc committee that will review and resolve any appeals against the other GRM Committees. Unresolved grievances by the committee shall be referred to Arbitration or to the court of Law. The committee, while handling a complaint may request any staff for its assistance and/or may constitute a special committee if required.

Grievance Redress at Arbitration/Court of Law

In the event that there is dissatisfaction from affected parties that cannot be resolved within the project’s proposed grievance resolution process, the Project Steering Committee shall refer the dispute or difference to arbitration within 30 days. It should be noted that arbitration only works where the parties to a dispute agree to resolve a difference through arbitration. Where there is no consent, then a court of jurisdiction may be used to resolve a dispute. The Court is the final instrument for the resolution of all grievances that may not have been addressed by the Project GRM. Figure 19 illustrates the grievance resolution process.

Figure 19: Project Resolution Process

Sensitive grievances such as those of sexual nature may be addressed through various actors or institutions such as local leadership, respected elders, clinics, church leaders, civic organisations, UNDP PMU  (refer to Figure 20). There are civic organizations that work with communities to deal with gender-based violence. An example is collaborated work among Musasa project, the Ministry of Home Affairs and the Zimbabwe Women Lawyers Association (ZWLA) in dealing with issues to do with sexual exploitation, abuse, harassment and gender-based violence. grievances can sometimes be referred to the Zimbabwe Republic police or the national court system for redress.

Figure 20: Schematic procedure for handling sensitive grievances

PMU as the Secretariat

The PMU through the safeguards officer shall log all grievances using the grievance log register, and transfer such data to a centralized database. Apart from its role as a secretariat, the PMU shall perform some grievance resolution functions including holding meetings with the affected party / complainant through appropriate means and seeking to find a solution acceptable to all parties. The PMU shall ensure that all resolutions are communicated to the complainant formally and if she/he accepts the resolutions. The PMU safeguards officer shall ensure that all complaints are resolved within 30 working days.

## 12.3 Timelines for resolving grievances

The suggested timelines for resolving grievances are provided in Table 52.

Table 52: Suggested timeframes for grievance redress

| Receiving and Registering grievance | Within1 day |
| --- | --- |
| Acknowledge, Assess and Assign | Within 2 days |
| Develop Response | Within 14 days |
| Provide Feedback | Within 21 days |
| Implement Response | Within 21 days |
| Close Grievance | Within 1 month |

## 12.4 Other grievance redress channels

Social and Environmental Compliance Unit

UNDP established SECU to ensure accountability to individuals and communities. SECU responds to complaints that UNDP may not be meeting its social and environmental commitments. Any person or community who believes the environment or their wellbeing may be affected by a UNDP-supported project or programme may file a complaint. A representative, such as a civil society organization, may also file a complaint on behalf of affected communities. People who file complaints may request that SECU protect their names and identities.

Stakeholder Response Mechanism – Helping parties resolve disputes

The Stakeholder Response Mechanism (SRM) helps project-affected stakeholders, governments and others partners jointly resolve concerns and disputes. It is available when Implementing Partner and UNDP project-level stakeholder engagement processes have not successfully resolved issues of concern. UNDP Country Office management normally leads in Stakeholder Response; a headquarters function will also support the SRM.

Any person or community potentially affected by a UNDP-supported project may file a request for a response from the Stakeholder Response Mechanism, if they have raised their concerns with Implementing Partners and/or with UNDP through standard channels for stakeholder consultation and engagement and have not been satisfied with the response.

If a person or community has a concern about the ability of the UNDP Country Office to respond fairly and effectively to the request, they have the option to file the request directly with the Stakeholder Response Mechanism at UNDP Headquarters in New York. Requests can be sent to the SRM through the Internet or through the mail.

Where to File the Request

Aggrieved stakeholders can submit grievances to SECU or requests to SRM through the UNDP Country Office or directly to UNDP Headquarters in New York. Requests can be made through online, email, toll-free telephone hotline (in any language), mail, or an in-person meeting with the Country Office Designee,

- By phone Call (costs are incurred by caller) using 001 (917) 207 4285. Skype is an affordable way to place such a call.

- Submitting a Request by Post (in any language) to: Attn: SECU/SRM, OAI, UNDP, 1 U.N. Plaza, 4th Floor New York, NY USA 10017

- Submitting a Request by Email (in any language) to: secuhotline@undp.org / stakeholder.response@undp.org

- By Social Media Apps. Complaints or grievances can be send through WhatsApp, Viber and Signal using 001 (917) 207 4285, or through our WeChat account @SECUSRM

## 12.5 Without Prejudice

The existence and use of this Grievance Redress Mechanism is without prejudice to any existing rights under any other complaint mechanisms that an individual or group of individuals may otherwise have access to under national or international law or the rules and regulations of other institutions, agencies, or commissions.

## 12.6 Cost of Implementing the GRM

A budget estimate of USD 2100 is proposed for operationalizing the GRM presented in this report. A summary breakdown is provided in Table 53.

Table 53: Cost of implementing and operating the GRM

| TASK | AMOUNT | PERSON RESPONSIBLE |
| --- | --- | --- |
| Preliminary stakeholder engagements/awareness building | $333 | - GRM Coordinator - Environmental and Social Safeguards Committee |
| Orientation and training workshop, | $320 | - GRM Coordinator at PMU |
| Preparation of communication materials (awareness and instructive materials), including complaint boxes | $233 | - GRM Coordinator - Communication Consultant |
| Establish Telephone hotlines, Internet, Email and social platforms (Facebook, WhatsApp) | $233 | - GRM Coordinator - Communication Consultant |
| Set up of GR infrastructure at PMU, including meetings and logistics | $147 | - PMU - GRM Coordinator |
| Logistic support to key community- based GRC members | $600 | - GRM Coordinator, |
| Conduct GRM evaluation | $233 | - M&E Specialist |
| TOTAL | $2100 |  |

## 12.7 Conclusion

This GRM shall be applied throughout all project phases. The GRM is a live document that is subjected to necessary amendments to suit the specific needs of the project and to ensure fair and timeous resolving of grievances or complaints. The GRM shall always remain is appropriate and suits the social, cultural and legal situation in Matobo and Zimbabwe. All the arrangements for dealing with complaints and grievances under the project as set out in this GRM shall be implemented.

# 13.0 CONCLUSION AND RECOMMENDATIONS

## 13.1 Conclusion

The anticipated impacts on air, water, land, biodiversity, safety and health and socio economics for the proposed project area are expected to be minimal; the mitigation measures will ensure that impacts remain minimal.

Overall, the ESMP study has concluded that the project would be environmentally and socially acceptable in compliance with relevant environmental legislation and standards and that it will significantly contribute towards the economic development. With the implementation of the environmental and social control measures during construction, operational and decommissioning phases, there would be no adverse residual impacts from the project that are not manageable in order to achieve sustainable development. The involvement of stakeholders in the planning, design and implementation of a project is crucial in minimizing probable conflicts. A detailed ESMP has been developed with the aim of addressing all probable adverse impacts anticipated to arise from the project.

## 13.2 Recommendations

The following recommendations should be taken into consideration:

- All permits and licenses should be obtained as required

- Excavation/ trenching should be done according to the ESMP

- All the Environmental and Social Management and Monitoring Plans (ESMPs) must be adhered to and if there are any changes the Director General must be notified.

# APPENDIX 1: LETTER OF CONSENT

# Appendix 1: Focus Group Discussion Tool

Socio-economic Impact Assessment of the Climate Proofing of Smallholder Irrigation Scheme in Matobo and Gwanda District

Name of Interviewer: …………………………….………………..…………………………….

Date: ………………………………… Location: ….………………….………………..……….

Date of FGD…………………………… / # of participants ………………………………….…

Notes: A register is affixed at the end of this document to capture additional respondents’ information to disaggregate the FGD participants.

1, a) Who owns the land on which the irrigation schemes have been established?

.………………………………………………………………………………………………………..

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

2), a. How was the Irrigation Management Committee formulated?

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

c., Does the committee have any rules or guidelines for its function(s)? (Yes / No)

Explain……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

d., What are the roles of the IMC?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

e., What is the selection criterion or qualifying requirements to become a member of the IMC?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

f., Does this irrigation scheme make a specific effort to include, women, youth, the elderly, child headed households or people living with disability on identifying beneficiaries to participate in the project?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

g., Have irrigation scheme members received any training to assist their purpose of crop production and sales? (Yes / No)

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Year trained	Trained in	Trained by	# trained	#women

Paid trainings ………………………………………………………………………………………

………………………………………………………………………………………………………

………………………………………………………………………………………………………

Free trainings

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

h.,What criteria are used to select participants for training?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

i.,When irrigation scheme members or IMC members are trained is this new knowledge shared with other irrigation scheme members or other IMC members as is relevant? (Yes / No)

Explain ……………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

3), What is the community leadership set-up in your ward (circle appropriate) and do they play a role in projects such as irrigation schemes for example by determining membership criteria, etc?

………………………….………………………………………………………………………..…………………………………………………………………………………………………………

…………………………………………………………………………………………………….

………………………….………………………………………………………………………..…………………………………………………………………………………………………………

…………………………………………………………………………………………………….

4), a. While the irrigation scheme was operational, were there any occurrences of conflict between irrigation scheme beneficiaries and the rest of the community members? (Yes / No) Explain

…………………….………………………………………………………………………………

………………………………………………………………………………………………….....

…………………………………………………………………………………………………….

b. How was this conflict resolved?

………………………….………………………………………………………………………………

………………………………………………………………………………………………………...........................................................................................................................................................

5), What grievance mechanism or channel is in place for irrigation scheme members including IMC to have their concerns known, documented and addressed? ……………………………….…………………………………………………………………….

………………………….…………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………...........................................................................................................................................................

………………………………………………………………………………………………………………………………………………………………………………………………………………

6), Do you see any linkage between irrigation of small holder plots and improving food security against climate change shocks such as drought, reduced rainfall and changing rainfall seasons and patterns? (Yes / No)

Explain……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………… …………………………………………………………………………………………………..

7), Are there any national policies, statutory instruments, acts / communal laws or related guidelines that give your irrigation scheme instruction on the processes / procedures / permits required for the establishment and running of smallholder irrigation schemes? (Yes / No)

Processes ……………………………….………………………….…………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Procedures ………………………………………………………………..…………………………………….

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Permits …………………………………………….……………………..…………………………………

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

8), Does this irrigation scheme have a community based or other funding model for its operation and maintenance? (Yes / No)

a. What typical maintenance and repair does the irrigation scheme infrastructure require?

b. How do you finance repairs for ;

i) Minor issues ……………………………………………………………………………….

ii) Major issues ……………………………………………………………………………….

c. Where do irrigation scheme members’ source their markets and for which produce?

Produce type			Intended market (& distance from scheme)

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

____________________________ / _________________________________/_________km

d. How do you transport produce from the irrigation scheme to the intended market?

Individual    		_______________________________________________________

Collective group	_______________________________________________________

Buyers come		_______________________________________________________

Other			_______________________________________________________

e. How do you determine market price for irrigation scheme produce

i) Sold on site ____________________________________________________________

ii) Sold off site by individuals ________________________________________________

iii) Sold off site by the collective ______________________________________________

f. Does the irrigation scheme have any means of savings and what are they used for? (Yes / No)

i) Collective savings _______________________________________________________

ii) Individual savings _______________________________________________________

iii) Small group savings ______________________________________________________

iv) Other __________________________________________________________________

g. Has the IMC or other members of the Irrigation Scheme been taught in financial responsibility, handling money, savings, etc? (Yes / No)

Explain

…………………………………………………………………………………………………

………………………………………………………………………………………………… …………………………………………………………………………………………………

9) Who determines the cropping plan at the irrigation scheme?

a. Cropping plan (what to plant)

…………………………………………………………………………………………………

………………………………………………………………………………………………… …………………………………………………………………………………………………

b. Cropping calendar (when to plant)

…………………………………………………………………………………………………

………………………………………………………………………………………………… …………………………………………………………………………………………………

c. Where do community members get their seed / seedlings from?

…………………………………………………………………………………………………

………………………………………………………………………………………………… …………………………………………………………………………………………………

d. Does the irrigation scheme make use of government extension services? (Yes / No) Explain

…………………………………………………………………………………………………

………………………………………………………………………………………………… …………………………………………………………………………………………………

10) Are there any socio-economic issues you can identify that currently positively or negatively affect or influence the uptake, success and sustainability of your small holder irrigation scheme? (yes / no)

a. Issues related to land tenure (ownership, tenancy etc)

…………………………………………………………………………………………………………

………………………………………………………………………………………………………… ……………………………………………………………………………………………………………………………………………………………………………………………………………………

b. Issues related to equity (including gender disparities, vulnerable groups) …….…………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

……………………………………………………………………………………………………………………………………………………………………………………………………………………

…………………………………………………………………………………………………………

c. Issues related to community knowledge management (use of agriculture extension services, cropping calendars, training in sustainable agriculture practices) …..……………………………

…………………………………………………………………………………………………………………………………………………………………………………………………………………….

………………………………………………………………………………………………………….

……….………………………………………………………………………………………………...

d. Issues related to financial constraints or incentives (community ability to mobilize funds for repairs, collective funds, savings, willingness of irrigation scheme members to pay towards seed / seedlings, training, or incentives such as government or other funds or policies that prioritize small holder farming e.g loan access etc)  ….…………………………………………………….

…………………………………………………………………………………………………………………………………………………………………………………………………………………….

………………………………………………………………………………………………………....

………………………………………………………………………………………………………….

e. Issues related to water (availability, quantities, conveyance, storage, access, timing e.g watering schedules …………………………………………………………………………………………..

…………………………………………………………………………………………………………………………………………………………………………………………………………………….

………………………………………………………………………………………………………....

………………………………………………………………………………………………………….

Other (explain) …………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………………………………………………………………………………………………………………….

- Are there any NGOs active in this areas? (Yes / No)

Explain yes (NGO name and intervention areas / types of intervention provided) ……………………

Farming………………………………………………………………………………………………..

Livelihoods……………………………………………………………………………………………..

Water.....………………………………………………………………………………………………..

Food security…………………………………………………………………………………………..

Other …………………………………………………………………………………………………..

………………………………………………………………………………………………………….

- What adjustments (if any) do you think are needed to ensure the revitalized irrigation scheme is successful?

Community ………………………………………………………………………………………...

………………………………………………………………………………………………………………………………………………………………………………………………………………

Beneficiaries ………..……………………………………………………………………………...

………………………………………………………………………………………………………………………………………………………………………………………………………………

Donor ………………………………….…………………………………………………………...

………………………………………………………………………………………………………………………………………………………………………………………………………………

Government ………………………………………………………………………………………...

………………………………………………………………………………………………………………………………………………………………………………………………………………

Other …………………………………………..…………………………………………………...

………………………………………………………………………………………………………………………………………………………………………………………………………………

- Are there any other comments or contributions you would like to make about the project?

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

# Appendix 2: Key informant Questionnaire

IRRIGATION REVITALISATION PROJECT QUESTIONNAIRE

BACKGROUND INFORMATION

The Government of Zimbabwe, Green Climate fund (GCF) and United Nations Development Programme has allocated funds through the Ministry of Lands, Agriculture, Water and Rural Resettlement (MLAWRR) to provide climate proofing irrigation infrastructure for enhanced water security under climate change impacts through revitalization. The irrigation revitalisation aims to fortify the adaptive capacities of vulnerable smallholder famers, especially women. The project has potential to increase and stabilise crop yields by allowing multiple cropping year round. Irrigated crop production enhances farm incomes from sale of surplus yields, off season premiums and also provides employment. The project will also allow farmers to plan agricultural tasks, manage crops, soil and water to reduce water stress

This project will be climate proofed through installation of efficient irrigation systems (Centre Pivot (Masholomoshe Irrigation Scheme) and Sprinkler or Drip irrigation system for Masiyepambili Irrigation Scheme.

HOW CAN YOU PARTICIPATE IN THE PROCESS?

As an interested stakeholder you are requested to submit your environmental and social concerns pertaining the irrigation revitalisation project. You can do so by completing your full details, comments and stamp on the provided form.

Full name……………………….................................................................................................

ID No…………………………………………...........................................................................

Organisation…………………………….Designation…………………………………………

Address…………………………………………………………………………………………

Phone Number…………………………………………………………………………………

Date…. …………..

As stakeholders what are your concerns on the following:

1) Environmental components

a) Air

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..…………………………………………………..…………………………………………………………………

b) Land

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….……………………………………………………………………………..

c) Water

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

d) Vegetation

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..…………………………………………………………………….…………………………………………………

e) Animals

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………..………………………………………………..

2) Social issues that can arise through the use of the following:

a)Land use ……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….………………………………..………………………………………………

b)Water use …………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

c) Livestock

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

d)Gender based violence issues ……………………………………………………………………………………………………………………………………………………………………………………………………………………………….…………………………………………………………………………………………..………………………………………………………………………………

i) Which gender group is mainly affected mostly?  [   ] males [   ] females

ii) Which age group is mostly affected?  [   ] 11-20yrs [   ] 21-30yrs [   ] 31-40yrs [   ] 41-50yrs [   ] 51-60yrs [   ] 61yrs and above

Recommendations………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Official Stamp………………….

# Appendix 3: Some Stakeholder Responses

# Appendix 4: Focus Group Discussion Summary Notes

# Appendix 5: Grievance log sheet at PMU

|  |  | GRIEVANCE LOG SHEET - PMU |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PROJECT TITLE: GCF BUILDING CLIMATE RESILIENCE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| S/N | NAME OF COMPLAINANT | CONTACT DETAILS | DESCRIPTION OF COMPLAINT | DATE RECEIVED | CHANNEL GRIEVANCE RECEIVED | UNIT/ CONTRACTOR COMPLAINED OF | RISK PRIORITY INDEX | LOCATION DISTRICT, PROVINCE) | GENDER | ACTION TAKEN/AGREED RESOLUTION | FEEDBACK GIVEN Y/N? | OUTCOME | DATE GRIEVANCE CLOSED |
|  |  |  |  |  | (Suggestion Box; Toll-free Line, email, letter etc.) |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |

# APPENDIX 6: Chance Find Procedure

Purpose of the chance find procedure

The chance find procedure is a project-specific procedure developed specifically for the Project “Building the climate resilience for vulnerable agricultural livelihoods in Southern Zimbabwe”. The procedure outlines actions required if previously unknown tangible forms of cultural or archaeological importance are encountered during project construction phase. A Chance Find Procedure is a process that prevents chance finds from being disturbed until an assessment by a competent specialist is made and actions consistent with the requirements are implemented.

Scope of the chance find procedure

This procedure is applicable to all activities conducted by the personnel, including contractors that have the potential to uncover a heritage item/site. The procedure details the actions to be taken when a previously unidentified and potential heritage item/site is found during construction activities. The procedure outlines the roles and responsibilities and the response times required from both project staff, and the National Museums and Monuments of Zimbabwe (NMMZ).

Induction/Training

All personnel, especially those working on excavations and or trenching are to be inducted on the identification of potential heritage items/sites and the relevant actions for them with regards to this procedure during the Project induction and regular toolbox talks.

Chance find procedure

If any person discovers a physical cultural resource, such as (but not limited to) archaeological sites, historical sites, remains and objects, or a cemetery and/or individual graves during excavation or construction, the following steps shall be taken:

- Stop all works in the vicinity of the find, until a solution is found for the preservation of these artefacts, or advice from the relevant authorities is obtained;

- Immediately notify the site foreman who will then notify the site Manager and the Safety, Health and Environment (SHE) Officer;

- The site SHE officer shall record details in Incident Report Form and take photos of the find;

- The site SHE officer shall ensure the discovered site or area is delineated and secured to prevent any damage or loss of removable objects. In cases of removable relics or sensitive remains, a night guard shall be arranged until the responsible local authorities take over;

- The SHE officer shall inform the relevant local authority and the NMMZ immediately and in writing within 7 days from the find;

- The site SHE officer shall provide the NMMZ team with photos, other information as relevant for identification and assessment of the significance of heritage items;

- The Department of National Museums and Monument shall investigate the find and provide response in writing.

- Decisions on how to handle the find shall be taken by the NMMZ. This could include changes in the layout (such as when finding an irremovable remain of cultural or archaeological importance) conservation, preservation, restoration and salvage;

- Construction works shall resume only after permission is granted from the responsible authorities.

- All records of communication with decision making authorities including conclusions and recommendations/guidance, implementation reports shall be kept.

Additional information

Management options for archaeological site

Site avoidance. If the boundaries of the site have been delineated, attempt must be made to redesign the proposed development to avoid the site. (The fastest and most cost-effective management option)

Mitigation. If it is not feasible to avoid the site through redesign, it will be necessary to sample it using data collection program prior to its loss. This could include surface collection and/or excavation. (The most expensive and time-consuming management option.)

Site Protection. It may be possible to protect the site through the installation of barriers during the time of the development and/or possibly for a longer term. This could include the erection of visibility fencing around the site or covering the site area. The exact prescription would be site- specific.

Management of replicable and non-replicable heritage

Different approaches for the finds apply to replicable and non-replicable heritage.

Replicable heritage

Where tangible cultural heritage that is replicable and not critical is encountered, mitigation measures will be applied. The mitigation hierarchy is as follows:

Avoidance;

Minimization of adverse impacts and implementation of restoration measures, in situ;

Restoration of the functionality of the cultural heritage, in a different location;

Permanent removal of historical and archaeological artefacts and structures;

Compensation of loss where minimization of adverse impacts and restoration not feasible.

Non-replicable heritage

Most cultural heritage is best protected by in situ preservation, since removal is likely to result in irreparable damage or even destruction of the cultural heritage.

Nonreplicable cultural heritage must not be removed unless all of the following conditions are met:

  - There are no technically or financially feasible alternatives to removal;

  - The overall benefits of the project conclusively outweigh the anticipated cultural heritage loss from removal; and

Any removal of cultural heritage must be conducted using the best available technique advised by the Department of National Museums and Monuments

##### Human Remains Management Options

The handling of human remains believed to be archaeological in nature requires communication according to the same procedure described above.

##### EMERGENCY CONTACTS

National Museums and Monuments of Zimbabwe

Address: 107 Rotten Row, Penrose Hill Building Causeway, Harare, Zimbabwe

Tel: +263 24 2774208.

E-Mail: natmus@nmmz.co.zw

Website: http://www.nmmz.co.zw/

# Appendix 7: Voluntary land use agreement forms

Replicable cultural heritage is defined as tangible forms of cultural heritage that can themselves be moved to another location or that can be replaced by a similar structure or natural features to which the cultural values can be  transferred by appropriate measures. Archaeological or historical sites may be considered replicable where the particular eras and cultural values they represent are well represented by other sites and/or structures.

Nonreplicable cultural heritage may relate to the social, economic, cultural, environmental, and climatic conditions of past peoples, their evolving ecologies, adaptive strategies, and early forms of environmental management, where the (i) cultural heritage is unique or relatively unique for the period it represents, or (ii) cultural heritage is unique or relatively unique in linking several periods in the same site. Examples of non-replicable cultural heritage may include an ancient city or temple, or a site unique in the period that it represents